3GPP TSG-RAN WG2 #131	R2-2505620

Bengaluru, India, 25th – 29th August 2025

Source:		NEC

Agenda item:		8.6.4	Conditional intra-CU LTM

Title:	Remaining Open Issues for CLTM

Document for:		Discussion and decision

- Introduction

During the last RAN2 meeting, the following agreements were made for the support of CLTM [1]:

Agreements on C-LTM

- (MAC-9) If there is at least one beam associated with the configured uplink grant with RSRP above configured RSRP threshold, the UE selects a beam among the beams and CG-based RACH-less CLTM is performed. If the UE finds out multiple beams above threshold, it is up to UE implementation to select which one. If there is no beam associated with the configured uplink grant with RSRP above configured RSRP threshold, RACH-based CLTM is performed.

- For L3 based CLTM, the UE performs RACH-based CLTM if there is no configured grant.

- Upon CLTM execution, if TA is valid (i.e. TAT timer is running) at the first available CG occasion, and there is CG resource configured, RACH-less CLTM will be performed.

- During CLTM is ongoing, after the first transmission, if TAT timer expires while RACH-less LTM is ongoing, fallback to RACH-based CLTM.

- In case CG resource is only associated with SSB, if the beam meeting the execution condition is CSI-RS, the UE determines whether a CG resource is valid based on the SSB associated with the CSI-RS.

- RAN2 will not do anything for the separate TCI mode optimization.

In this paper, we provide our consideration on the remaining issues of conditional intra-CU LTM.

- Discussion

2.1 MAC-14

MAC-14 is about the issue that the TAT timer associated with one candidate cell is running at the UE side, however, the UE receives from the network a LTM cell switch command indicating that there is no valid TA for the PTAG, how UE behave. From our perspective, if the network indicates that there is no valid TA for the PTAG, it means that the network consider the TA provided to the UE previously is un-trustable, therefore the UE should not perform RACH-less LTM based on the TA of the candidate cell maintained at the UE side. Instead, the UE should consider the TAT timer associated with the candidate cell as expired.

Proposal 1. (MAC-14) Upon reception of (Enhanced) LTM Cell switch Command MAC CE indicating that there is no valid TA for PTAG, the UE considers the time alignment timer associated with the candidate cell as expired.

2.2 MAC-30

During previous meetings, it was agreed that the UE starts the time alignment timer for PTAG using the remaining time from the C-LTM timer. And if TAT timer (we understand that it is the PATG) expires while RACH-less LTM is ongoing, fallback to RACH-based CLTM. For this fallback case, one remaining issue to be addressed is how to handle the Msg 3/A buffer. Since the initial transmission content including the LTM complete message (RRCReconfigurationComplete) is included in the HARQ buffer of the HARQ process used for the first PUSCH transmission of LTM, we need to transfer it from the HARQ buffer to the Msg3 or Msg A buffer. Otherwise, there may be no content for the Msg 3/A buffer, which is abnormal, and the RRCRconfigurationComplete message is not to be transmitted to the target cell in time, which delays the completion of the conditional LTM cell switch procedure. Therefore, the best way is to obtains MAC PDU from the HARQ buffer of the HARQ process used for the first PUSCH transmission of LTM, and stores the obtained MAC PDU in the Msg 3 or Msg A buffer.

Proposal 2. (MAC-30) When fallback from RACH-less LTM to RACH-based LTM, the UE obtains MAC PDU from the HARQ buffer of the HARQ process used for the first PUSCH transmission of LTM, and stores the obtained MAC PDU in the Msg 3 buffer or Msg A buffer of the RA procedure.

2.3 PDCP SN gap due to CLTM fast recovery

At last, we would also like to discuss about the issue raised by Fujitsu in [POST130][110][MOB] RRC CR . During Rel-18 LTM discussion, the issue of PDCP SN gap caused by LTM fast recovery was discussed, and it was agreed that the network can handle it by proper implementation because the failed LTM cell switch is triggered by the LTM cell switch command sent by the network. For background information, the PDCP SN gap is caused by the unsuccessful transmission of the RRCReconfigurationComplete message during a failed LTM cell switch, and to avoid the security key stream reuse issue, UE continues the PDCP COUNT value to transmit another RRCReconfigurationComplete message during the LTM fast recovery procedure.

R2-2404781 Discussion on PDCP SN gap at LTM fast recovery NTT DOCOMO, INC. discussion Rel-18 Moved from 7.4.3.1 - Intel think the network knows this and can then handle this gap without impacts. Ericsson agrees. MTK too. OPPO too. - Docomo think we should have some note, e.g. in 331 T304 expiry. - HW and Nokia think that network usually handles cases .. - Nokia think that if we have a note it could be in stage-2. R2 assumes that network implementation can handle this. Will have a note, where and what discussed in CR discussions after meeting.

And following Note is added in TS38.300:

NOTE: PDCP SN gap for SRB may exist upon LTM attempt toward the selected cell after LTM fails. It is up to network implementation to avoid the latency caused by the PDCP SN gap.

For conditional LTM, it was agreed that CLTM fast failure recovery is supported. This means the case that upon a failure of a CLTM, the UE selects a cell which is a conditional LTM candidate cell, and performs LTM cell switch execution to the selected candidate cell. For this case, the network is not able to determine whether one LTM cell switch is triggered due to execution conditional fulfillment or due to fast recovery, and therefore may wait for a missing PDCP SRB1 packet for a long time, which delays the completion of the fast recovery.

Observation. The PDCP SN gap caused by CLTM fast recovery cannot be handled by network implementation.

In R2-2404781, PDCP based solutions are provided, e.g., new PDCP header or new PDCP control PDU. However, from our perspective, PDCP impact should be avoided, if possible, for this lower layer mobility issue. Therefore, we prefer to have a lower layer based solution, e.g., a MAC layer based LTM fast recovery indication from the UE to the target cell. The MAC layer based indication, for example, may be a new MAC CE transmitted from the UE to the network, or a dedicated (e)LCID used by the UE to transmit the RRCReconfigurationComplete message during the LTM fast recovery procedure.

Proposal 3. RAN2 to work on lower layer (e.g., MAC layer) based solution for the PDCP SN gap issue caused by CLTM fast failure recovery.

# 3. Conclusion

In this paper, we discussion on the remaining issues of CLTM, and have the following proposals:

Proposal 1. (MAC-14) Upon reception of (Enhanced) LTM Cell switch Command MAC CE indicating that there is no valid TA for PTAG, the UE considers the time alignment timer associated with the candidate cell as expired.

Proposal 2. (MAC-30) When fallback from RACH-less LTM to RACH-based LTM, the UE obtains MAC PDU from the HARQ buffer of the HARQ process used for the first PUSCH transmission of LTM, and stores the obtained MAC PDU in the Msg 3 buffer or Msg A buffer of the RA procedure.

Observation. The PDCP SN gap caused by CLTM fast recovery cannot be handled by network implementation.

Proposal 3. RAN2 to work on lower layer (e.g., MAC layer) based solution for the PDCP SN gap issue caused by CLTM fast failure recovery.

# Reference

[1] meeting report of RAN2 #130.