3GPP TSG-RAN-WG2 Meeting #131	                                       R2-250XXXX

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	8.3.3

Source: 	Sharp

Title:  	Discussion on NW-side model input

Document for: 	Discussion

Introduction

In RAN2#130 [1], the following agreements were made:

Agreements - For network-side RRM measurement prediction, the legacy RRM measurement configuration and RRM measurement reporting framework can be used. For L1-filtered beam-level RSRP reporting can be configured by setting co-efficient to zero. FFS if there are specification impacts to support subcase 1 and 3 (if supported) and interference for cell level prediction. - To support cell/beam level prediction, one enhancements is to report RRM measurement results per cell at multiple time instances in one measurement report for NW-sided model.

In the following section, we discuss the enhancement from the legacy L3-cell level RRM measurement provided from the UE, and the measurement configuration from the gNB/NW to enhance the inference results.

Discussion

In TR 38.744 [2], a consensus will be formed that the RRM measurement prediction can be performed using either the UE-sided model or the network-sided model. Following this concept, we can further consider whether the cell-level prediction RRM sub-use cases 1 and 3 are supported at the NW-side model inference. The definitions of the RRM sub-use cases 1 and 3 are as follows:

- RRM sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated;

- RRM sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s).

For the RRM sub-use case 1, there might be a case where, in a specific area, such as a specific geographic location, cell group, or frequency band, the accessed UEs can only provide the L1 beam-level measurement results for some particular reasons, e.g., power saving. Another case might be that there are no UEs in the specific area as mentioned, but the gNB/NW has obtained the L1 beam-level measurement results in the near past. In these cases, the gNB/NW can only predict the L1 beam-level measurement results and generate the L3 cell-level measurement results based on the received L1 beam-level measurement results. The cases may be extreme, but they could still warrant further study. Whereas RRM sub-use case 3 can be considered in the same cases above.

Proposal 1: For the support of cell-level prediction RRM sub-use cases 1 and 3, RAN2 to consider the possibility of the use cases that the UE can only provide the L1 beam-level measurement results on a specific area, such as a specific geographic location, cell group, or frequency band.

To obtain a comprehensive inference result, the gNB/NW should be able to acquire the RRM measurement results from the UEs in some specific geographic locations, cell groups, or frequency bands. While the measurement configuration is from the RRC message RRCReconfiguration, the measurement report can also be triggered via low layers (e.g., MAC CE/DCI), as in the existing CSI measurement report. Regarding the RRM measurement (of L1 and L3), the UE can provide the results through the existing measurement mechanism, and/or the “inference-assisted measurement results”. The “inference-assisted measurement result” is used for gNB/NW inference, which can be the measurement results other than performing the existing measurement mechanism, e.g., to measure all of the whitelisted cells in the measurement configuration.

Proposal 2: RAN2 to discuss the following options to provide L1/L3 measurement results to the network for inference in the work item phase.

- Option 1: The existing L1/L3 measurement results.

- Option 2: New inference-assisted measurement results.

- Option 3: The existing L1/L3 measurement results and the new inference-assisted measurement results.

Some companies consider that the NW-side model event prediction should not be treated in this study item. However, we think that the NW-side model event prediction still has benefits, such as having a system-wide prediction view to make the cell switching decision of the handover UEs. In addition, the NW-side model event prediction can reduce the inference burden of the UE-side model and the signaling exchange for LCM in NW-side monitoring and NW-side decision, such as performance monitoring. In this sense, we suggest RAN2 consider the NW-side model event prediction in the work item phase.

Proposal 3: RAN2 to consider the NW-side model event prediction in the work item phase.

From a performance enhancement perspective, to obtain a more geographically wide, frequency band wide, and cell wide measurement input in a specific time instance in the gNB/NW aspect, we should further consider not only the UEs in RRC CONNECTED, but also the UEs in RRC INACTIVE and maybe RRC IDLE, to provide the “inference-assisted measurement results” by receiving some specific signaling, such as a dedicated paging message. Regarding AIML for mobility, it should also be worth considering that the UEs can provide additional information other than the existing UAI on inference assistance, such as moving speed while measuring. The existing seed evaluation mechanism (e.g., the normal-mobility state, medium-mobility state, and high-mobility state of the speed evaluation mechanism in TR 38.304 [3]) may be insufficient to support the NW-side model measurement predictions. Therefore, we should consider a finer speed evaluation mechanism for measurement predictions. FFS to introduce any other inference-assisted parameters.

Proposal 4: If option 2 is supported, RAN2 to consider that the gNB/NW can acquire the UEs, providing the inference-assisted measurement results, whether it is in RRC CONNECTED.

Proposal 5: RAN2 to consider introducing a finer speed evaluation mechanism for mobility measurement predictions. FFS to introduce any other inference-assisted parameters.

Conclusions

Based on the discussion above, we have the following proposals:

Proposal 1: For the support of cell-level prediction RRM sub-use cases 1 and 3, RAN2 to consider the possibility of the use cases that the UE can only provide the L1 beam-level measurement results on a specific area, such as a specific geographic location, cell group, or frequency band.

Proposal 2: RAN2 to discuss the following options to provide L1/L3 measurement results to the network for inference in the work item phase.

- Option 1: The existing L1/L3 measurement results.

- Option 2: New inference-assisted measurement results.

- Option 3: The existing L1/L3 measurement results and the new inference-assisted measurement results.

Proposal 3: RAN2 to consider the NW-side model event prediction in the work item phase.

Proposal 4: If option 2 is supported, RAN2 to consider that the gNB/NW can acquire the UEs, providing the inference-assisted measurement results, whether it is in RRC CONNECTED.

Proposal 5: RAN2 to consider introducing a finer speed evaluation mechanism for mobility measurement predictions. FFS to introduce any other inference-assisted parameters.

References

[1] Chair notes, RAN2 130 3GPP TSG-RAN WG2 Meeting #130, St.Julians, Malta, 2025

[2] TR 38.744 V1.1.0 (2025-08)

[3] TS 38.304 V18.4.0 (2025-01)