3GPP TSG-RAN WG2 Meeting #131	R2-2505643

Bengaluru, India, 25th – 29rd August 2025

Agenda Item:	8.7.5

Source:	ITRI

Title:	Discussion on RLC Enhancements for Unnecessary Retransmissions Avoidance

Document for:	Discussion and Decision

1 Introduction

In RAN2 #130, the following agreements of enhanced RLC had been reached.

Agreements for unnecessary retransmission avoidance in RLC - (RLC-6) There is no specification impact foreseen for RLF triggering due to RLC maximum retransmission. - Working assumption: (RLC-11) No need to address window stalling issue with polling retransmission (TBC next meeting) - Assumption from companies is that this should not happen due to SR triggering from Rx side, but it needs to be checked whether this will always work - (UE capability-2) Define an (optional) per-UE capability with signalling for the Rx-side aspect, where an outdated SDU is abandoned based on a new RLC timer and the abandoned SDUs are positively acknowledged in an RLC status report. - (UE capability-2) Define an (optional) per-UE capability with signalling for the Tx-side aspect, where the Tx side stops transmissions for an outdated SDU based on an indication from the PDCP. FFS A UE supporting this feature shall also indicate the support of Rx-side aspect.

In this document, for RLC-11, we confirm that the stalled transmitting window is not needed to be addressed with polling retransmission beacuse we had agreed that at the receiving side of an AM RLC entity, when the t-RxDiscard expires, the expiration of t-RxDiscard triggers a STATUS report.

2 Discussion

During the on-line discussion of RLC running CR at RAN2 #130, companies have different views about the procedure of expiry of t-PollRetransmit. Some companies thought that window stalling should be addressed in case the transmitting window is stalled when t-PollRetransmit expires. That means the poll needs to be re-sent in a retransmitted SDU, even if only SDUs discarded by PDCP are buffered.

However, an agreement at RAN2 #129bis had been reached, “When the t-RxDiscard expires, the expiration of t-RxDiscard triggers an SR.” While the transmitting window is stalled when t-PollRetransmit expires and only SDUs discarded by PDCP are buffered, after a short time interval, the receiving side of an AM RLC entity shall trigger a STATUS report when the t-RxDiscard expires. At the transmitting side, stopReTxObsoleteSDU is configured to stop the unnecessary retransmissions as soon as possible, and at the receiving side, t-RxDiscard is configured to decide an obsolete SDU for simplicity. We believe that the time interval between discard indication for SDUs from upper layer at the transmitting side and the corresponding t-RxDiscard expiration at the receiving side is very short. So a stalled transmitting window when t-PollRetransmit expires is a corner case.

Therefore we think that the current running CR works and confirm that no need to address window stalling issue with polling retransmission. Regarding the issue, “Whether/If the UE needs to continue to run t-PollRetransmit when it already knows the status of all RLC SDUs (either positively/negatively acknowledged, or discarded) up to POLL_SN”, we do not have strong opinion and follow the majority’s view.

Proposal (RLC-11): No need to address window stalling issue with polling retransmission is confirmed.

3 Conclusion

Based on the discussion in the previous section we propose the following:

Proposal (RLC-11): No need to address window stalling issue with polling retransmission is confirmed.

4 References

- R2_130 R19 XR session notes (Dawid) 2025-05-23_EOM