3GPP TSG-RAN WG2 Meeting #131	R2-2505645

Bengaluru, India, 25 – 29 August, 2025

Agenda item:	8.4.4

Source:	InterDigital

Title:	LP-WUS in CONNECTED mode

WID/SID:	NR_LPWUS-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

This TDoc discusses the issues with LP-WUS in RRC_CONNECTED mode.

# 2	Discussion

## 2.1	LP-WUS capability

As it has been agreed that the LP-WUS monitoring is configured for the UE through dedicated RRC signaling, it is evident that UE capability is required for gNB to be able to trigger the configuration for a UE.

RAN1#118-Bis meeting agreed the following:

Agreement For RRC CONNECTED mode, UE reports one value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. - FFS: X - FFS: definition of reported candidate value - UE can indicate its preference for configured time offset between LP-WUS and PDCCH reception using UAI mechanism. FFS details. UAI is optional. - FFS: A single time offset is configured for a UE at a given time

Such capability on the minimum gap between LP-WUS reception and MR to start PDCCH monitoring can be used as LP-WUS support capability in RRC_CONNECTED mode.

Proposal 1: The UE capability signaling on minimum time gap between LP-WUS reception and MR to start PDCCH monitoring per SCS indicates support for LP-WUS in RRC_CONNECTED.

## 2.2	Entry/exit conditions

RAN2 agreed the following baseline for entry/exit conditions for RRC_IDLE/INACTIVE mode in RAN2#126 and RAN2#127 meetings, respectively:

RAN2#126:

Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed. Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS.

RAN2#127:

Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS. Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds. The metrics for serving cell quality measured by MR/LR for entry condition includes (LP-)RSRP and optional (LP-)RSRQ. The metrics for serving cell quality measured by LR for exit condition includes (LP-)RSRP and optional (LP-)RSRQ.

This baseline seems to work as a reasonable foundation for entry/exit conditions also for RRC_CONNECTED mode where the UE monitors PDCCH solely based on C-DRX when the serving cell quality does not satisfy the condition to monitor LP-WUS (entry condition not satisfied); and where the UE monitors PDCCH based on both the LP-WUS and C-DRX when the serving cell quality satisfies the entry condition. Given the dedicated configuration of the LP-WUS monitoring for RRC_CONNECTED mode UE agreed, it seems reasonable to allow the configuration of these thresholds separately for RRC_CONNECTED UE from RRC_IDLE/INACTIVE conditions, or the same can be applied when no dedicated configuration is provided. To avoid ping-pong effect at the UE (especially, if there would be a requirement to report out-of-coverage events to the NW), UE could be required to have a threshold number of measurement events (in or out-of-coverage) before entering/leaving the LP-WUS monitoring.

Proposal 2: As a baseline, support the same entry/exit condition criteria based on serving cell quality to activate/deactivate LP-WUS monitoring in RRC_CONNECTED mode as agreed for RRC_IDLE/INACTIVE mode.

Proposal 3: UE can be configured with a threshold number of LP-WUS receptions for determining LP-WUS in coverage (e.g., successful reception) and out-of-coverage (e.g., reception failure).

It is worth discussing whether only the serving cell quality based entry/exit condition is enough to maintain synchronization between UE and the gNB and avoid any service interruptions if the UE cannot receive the LP-WUS signal regardless of entry/exit condition being satisfied. Similarly to data inactivity timer (which protects the system from RRC mismatch), a timer could be exploited to avoid an issue where the entry/exit condition is satisfied but the UE cannot receive the LP-WUS signal while gNB tries to wake up the UE. While the timer is running (and entry/exit condition satisfied), the UE can activate LP-WUS monitoring whereas when the timer expires, the UE deactivates LP-WUS monitoring and monitors PDCCH based on the other rules in the specification (i.e., C-DRX).

Proposal 4: Support a timer as additional condition to activate/deactivate LP-WUS monitoring.

While the UE activating the LP-WUS monitoring based on the entry/exit condition being satisfied doesn’t need to be known to the gNB, the UE deactivating the LP-WUS monitoring based on the events as above would seem reasonable to be made known to the gNB, especially, if it happens through the timer while the entry/exit condition was still satisfied.

Proposal 5: UE reports the events of autonomous deactivation of LP-WUS monitoring to the gNB.

## 2.3	UE behaviour for potential collision with Option 1-2 LP-WUS

RAN1 has made the following agreement and conclusion in their RAN1#121 meeting reflected in the incoming RAN2 LS in R1-2504888:

Agreement: As the initial reply to RAN2 LS in R1-2503616, RAN1 confirms that at least the collision with Active Time, measurement gap, and RAR window monitoring for BFR can be considered for the cases/scenarios on when the UE is not able to monitor LP-WUS. Conclusion From RAN1 perspective, for the case of potential collision (if any) in Option 1-2, when the UE is not able to monitor all the LP-WUS MO(s) in a LP-WUS periodicity, - It is up to RAN2 to further discuss and finalize the specification support, if any.

In the post meeting email discussion “[Post130][213][LPWUS] Running CR for 38.321 (Apple)” many companies argue that with Option 1-2 LP-WUS the periodicity for monitoring LP-WUS is much smaller than with Option 1-1 and, hence, the UE should not start the lpwus-PDCCHMonitoringTimer upon not being able to monitor for LP-WUS in an LO (LP-WUS Occasion). However, such an assumption seems bold as it was agreed previously that the LP-WUS monitoring periodicity with Option 1-2 could be as long as Long DRX cycle. On the other hand, when the LP-WUS monitoring periodicity is indeed frequent, it would not make sense to always start the lpwus-PDCCHMonitoringTimer when the collision happens as in such a case the energy efficiency will suffer from potentially frequent PDCCH monitoring. Hence, it would seem simplest allowing NW to configure whether the UE should start the lpwus-PDCCHMonitoringTimer upon collision or not, since the NW can take into account the LP-WUS periodicity as well as most of the possible collision events upon configuration.

Proposal 6: NW configures whether the UE starts lpwus-PDCCHMonitoringTimer upon not being able to monitor for LP-WUS when there is a collision with Active Time, measurement gap, or RAR window monitoring for BFR.

# 3	Conclusion

This document has made the following observations and proposals:

Proposal 1: The UE capability signaling on minimum time gap between LP-WUS reception and MR to start PDCCH monitoring per SCS indicates support for LP-WUS in RRC_CONNECTED.

Proposal 2: As a baseline, support the same entry/exit condition criteria based on serving cell quality to activate/deactivate LP-WUS monitoring in RRC_CONNECTED mode as agreed for RRC_IDLE/INACTIVE mode.

Proposal 3: UE can be configured with a threshold number of LP-WUS receptions for determining LP-WUS in coverage (e.g., successful reception) and out-of-coverage (e.g., reception failure).

Proposal 4: Support a timer as additional condition to activate/deactivate LP-WUS monitoring.

Proposal 5: UE reports the events of autonomous deactivation of LP-WUS monitoring to the gNB.

Proposal 6: NW configures whether the UE starts lpwus-PDCCHMonitoringTimer upon not being able to monitor for LP-WUS when there is a collision with Active Time, measurement gap, or RAR window monitoring for BFR.