3GPP TSG-RAN WG2 Meeting #131	R2-2505647

Bangalore, India Aug 25th – 29th, 2025

Agenda Item:	8.7.5

Source:	InterDigital

Title:	Discussion on RLC enhancements

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion

# 1. Introduction

In RAN2#130 meeting, several agreements for RLC enhancement [1] were made. Afterwards, an email discussion was conducted to further discuss the RLC CR and open issues. The list of agreements in the last RAN2#130 meeting and open RLC issues in the email discussion are provided in the Appendix. In this contribution, we discuss the remaining issues related to timely RLC retransmissions, which are identified in the email discussion, or have not been addressed in the last RAN2 meetings.

# Discussion

  - 2.1 Autonomous retransmission:

According to legacy [3], retransmission of an RLC SDU (segment) due to NACK status report is always prioritized over initial transmission of another RLC SDU (segment). Another issue is how to prioritize between autonomous retransmission of an RLC SDU and retransmission of another RLC SDU (segment) due to NACK status report. It is possible that the RLC SDU (segment) considered for autonomous retransmission has been successfully received from the receiver RLC entity. On the other hand, the RLC SDU (segment) subject for retransmission due to NACK report from the receiver absolutely requires retransmission to be successfully received. Therefore, this RLC SDU (segment) should be prioritized over any other autonomous retransmission RLC SDU (segment).

Proposal 1:	Status Report based retransmission of an RLC SDU (segment) is prioritized over autonomous retransmission of any other RLC SDU (segment).

  - 2.2 Polling enhancement:

As RAN2 agreed in the last meeting that polling can be triggered when the remaining time of a RLC SDU (segment) falls below a threshold. Like autonomous retransmission case, the remaining time of a RLC SDU (segment) may fall below a threshold before the original RLC SDU is completely transmitted. In this case, enabling polling may not be beneficial as no status for such RLC SDU is expected in the status report. It is necessary to enable polling bit when the RLC SDU is completely transmitted. In another words, polling bit can be enabled at earliest when the RLC SDU or the last segment of the RLC SDU is being transmitted.

Proposal 2: 	When the remaining time of the RLC SDU falls below a specified threshold, polling is triggered for an RLC SDU at the earliest when the last segment of the RLC SDU is being transmitted.

One remaining issue related to polling enhancement is whether/how to handle the case where there are only SDUs discarded by PDCP due to the expiry of the discard timers in the buffer. In our view, even window stalling can happen, there is no problem as there are no more SDUs in the buffer. In addition, when SDUs are discarded, eventually, the Rx side will trigger status report and the transmission window at the Tx can be updated accordingly. Therefore, we propose to confirm the WA made in the last meeting that there is no need to address window stalling issue with polling retransmission.

Proposal 3: 	[RLC-11] Confirm the following WA regarding window stalling issues with polling retransmission:

- No need to address window stalling issue with polling retransmission.

  - 2.3 Status report enhancement:

It was discussed in the email discussion that time-critical status (SR) at RX side should be speed up to enable the Tx side to be aware of the transmission status of the delay-critical SDUs. Specifically, it was proposed to enable the Rx side to bypass/ignore the t-Reassembly, t-StatusProhibit timers. These timers are configured to balance between reporting overhead and delay in the SR as well as account the HARQ reordering delay. It is should be noted that it is not possible for the Rx side to differentiate polling triggered by the remaining time of an RLC SDU or by other means. Therefore, bypassing t-Reassembly and t-StatusProhibit timers to speed up the SR creates issues and potentially unnecessary re-transmissions. In our view, it can be left to the gNB’s implementation to speed up SR reporting.

Proposal 4: 	No enhancement is needed to accelerate SR transmission at RX side.

# Conclusion

In this contribution, the following proposals are made for timely RLC retransmission of RLC enhancements:

Proposal 1:	Status Report based retransmission of an RLC SDU (segment) is prioritized over autonomous retransmission of any other RLC SDU (segment).

Proposal 2: 	When the remaining time of the RLC SDU falls below a specified threshold, polling is triggered for an RLC SDU at the earliest when the last segment of the RLC SDU is being transmitted.

Proposal 3: 	[RLC-11] Confirm the following WA regarding window stalling issues with polling retransmission:

- No need to address window stalling issue with polling retransmission.

Proposal 4: 	No enhancement is needed to accelerate SR transmission at RX side.

# References

- RAN2#129-Bis chair notes.

- [POST130][508][XR] RLC running CR and open issues (vivo)

- 3GPP TS 38.322, Radio Link Control (RLC) protocol specification, v.18.0.0, 2023-12.

# Appendix:

  - 5.1 RAN2 #130 agreements

(RLC-8) We just keep the current specifications for polling triggering, i.e. no need to specify that polling shall only be triggered once per RLC SDU when its remaining time falls below a specified threshold, unless an issue is identified with this (RLC-9) No additional conditions are needed for the polling enhancement. RAN2 agrees that autonomous retransmissions should trigger an increment of the RETX_COUNT. (PDCP-2) The UE shall trigger remaining-time-based RLC retransmission and polling in PDCP SDU level, no matter pdu-SetDiscard is configured or not. No specification change is needed. Autonomous retransmission and polling is triggered only based on discardTimer, i.e. not based on discardTimerForLowImprotance The remaining time thresholds (for both autonomous retransmission and polling) are configured per PDCP entity. (RLC-6) There is no specification impact foreseen for RLF triggering due to RLC maximum retransmission. Working assumption: (RLC-11) No need to address window stalling issue with polling retransmission (TBC next meeting) Assumption from companies is that this should not happen due to SR triggering from Rx side, but it needs to be checked whether this will always work Define an (optional) per-UE capability with signalling for the Rx-side aspect, where an outdated SDU is abandoned based on a new RLC timer and the abandoned SDUs are positively acknowledged in an RLC status report. Define an (optional) per-UE capability with signalling for the Tx-side aspect, where the Tx side stops transmissions for an outdated SDU based on an indication from the PDCP. FFS A UE supporting this feature shall also indicate the support of Rx-side aspect.

  - 5.2 RLC open issues

| RLC-10 | Whether/How to introduce any limitations or associations for the configuration of stopReTxObsoleteSDU and DL t-RxDiscard. |
| --- | --- |
| RLC-11 | FFS how to handle the case “if there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, there is no SDU to retransmit the poll with”. |
| RLC-12 | FFS how to handle PDCP SN gap report handling during UE mobility. |
| RLC-13 | Whether the delay-reporting or non-delay-reporting information is visible or not to RLC? |