3GPP TSG-RAN WG2 Meeting #131		R2-2505651

Bengaluru, India, August 25-29th 2025

Agenda Item: 		8.2.2

Source:			Sony

Title:	Discussion on MAC Open issues on Paging

Document for:		Discussion & Decision

# Introduction

In this contribution we provide our views on Paging issues leftover from AIOT MAC spec open issues. These issues were discussed during the email discussion [1]. We try to provide further justifications.

# Discussions

Issue 1-2: transaction ID

Issue 1-2: transaction ID; Whether/how to specify how the reader generate Transaction ID, and the size - Relevant agreements: - The “transaction ID” can be generated by reader based on CN corelation ID. FFS how reader will generate “transaction ID”. FFS the size of transaction ID - 1 bit solution is excluded. FFS the size. Aim to have a reasonable size - Status in running CR: captured as Editor’s Note in 6.2.1.1.; Companies are invited to input views for Q#2. According to previous discussion, the main motivation to consider long transaction ID/specify the generation of transaction ID seems for multi-reader scenario. However, in last meeting, it has been agreed that it should be network implementation to avoid devices receiving parallel service requests. In this case, network implementation can also avoid transaction ID collision, if needed. Therefore, there is no need to further discuss how to specify the transaction ID generation. Then for the length of transaction ID, proposals from companies seem to focus between 2 and 6. So the rapp would like to ask for company’s inputs on the suggested transaction ID length, based on that we can follow majority view. Q#2: Companies are invited to provide feedback regarding: - Q2.1: do you agree how to generate transaction ID is also left to implementation? Q2.2: how many bits between 2 and 6 of transaction ID do you prefer?

All companies during the email discussion seem to agree that the generation of transaction ID can be left to implementation. There is a correlation between Core network generated CN correlation ID and the transaction ID. Correlation ID (with the size as FFS in RAN3) identifies an AIOT session in a gNB and a cell and eventually a reader. Transaction ID or the part of correlation ID, referring to AIOT session within a reader, needs to identify a session between a reader and a device or set of devices. We understand that no parallel service sessions are planned in rel-19. In this case, transaction ID is needed to differentiate between different service sessions. For identifying if the service is old or new, a single bit representing a new or old service should be sufficient.

During rel-19, a single reader scenario is supported so there is no need to distinguish different readers. But there may be multi reader scenario being supported in rel-20, so, 2 bits identifying different readers may be sufficient. Hence, we think that in total 3 bits are sufficient for transaction ID.

Proposal 1: 3 bits transaction ID is supported with 2 bits corresponding to reader ID and one bit indicating service ID.

Further, there could be a scenario where a device should be able to respond to any of the readers e.g. for the purpose of inventory. In other words, all readers are involved in inventory procedure and any of the readers is able to pick up the response from the device. There could be a single service, and no service ID being used. In this case, presence of transaction ID will be same from all readers. We think that depending on network configuration, the presence of transaction ID should be optional in different messages.

Proposal 2: Presence of transaction ID is optional or value of “0” should be permitted.

Issue 3-3: AS ID release

Issue 3-3: AS ID release; Whether a release message is needed for AS ID release - Relevant agreements: - FFS other cases for release ASID to avoid keeping it indefinitely. - - For CBRA, to avoid AS ID being occupied for unnecessary time and to keep alignment between reader and device on AS ID release, device can release AS ID upon receiving paging message with different transaction ID, no matter the paging message is for it or not. FFS for CFRA - - FFS for need for release message - Status in running CR: captured as Editor’s Note in 5.2.; Companies are invited to input views for Q#6. According to the current agreements/running CR, devices should release AS ID upon a CFRA paging message or upon a CBRA paging message with new transaction ID/resulting re-access which already address the issue “avoid keeping it indefinitely”. In this sense, whether to have a release message to enable early AS ID release is not a critical issue. But the rapp would like to check if there is a majority view on the need of the release message and the detailed solution. Q#6: Do companies agree that release message is needed for AS ID release? If so, which option is preferred? Opt1. Unicast message (only one AS ID) Opt2. Multiplexing with a list of AS ID (NACK message-like) Opt3. Broadcast message (without device AS ID)?

There was a concern about the risk of reusing the same ID if there are hanging AS IDs. But as companies suggested during the email discussion that AS ID will be used during command procedure only.  Also, the FFS exists for CFRA case only. Subsequent Paging may be far away in time, but Paging can implicitly act as a release indication. For most cases, AS ID length should be sufficient to handle the hanging AS IDs and not cause AS ID collision. If network is concerned about hanging AS ID, then it can simply not configure CFRA.

Proposal 3: AS ID release can be handled by network implementation.

Q#12: Which R2D message(s) other than paging message need to consider forward compatibility using similar handling as paging, with the corresponding the use case clearly clarified.

Rel-19 and Rel-20 Paging message structures might be different, as discussed during past a few RAN2 meetings. Companies have expressed strong desire to allow a Rel-19 UE to receive at least Paging message in a Rel-20 network. So, a reserve bit or a bit allowing future extensibility is required. There were proposals to introduce more than single bit. But since the intention of introducing a new bit is to allow forward compatibility rather than identifying different readers, a single R bit is sufficient. The running CR already captures R bit for Paging. We think R bit should be introduced for other messages as well, in line with companies’ views during the email discussion.

Proposal 4: Single R bit is introduced to all messages.

Issue 3-7: more data indication

(New)Issue 3-7: more data indication handling; How to set “more data indication” value in case of no NAS response available (i.e., zero SDU) - During CR implementation, the rapp identified there is no clear conclusion how to set the “more data indication” in case of no data available, i.e., zero SDU. - Status in running CR: captured as value FFS.; Companies are invited to input views for Q#9. Given that the “more data indication” is a 1-bit field, and value 0 means there is no more data, while value 1 means there is more data, in this “no NAS response available” case, this indication should be set to “1”, as there will be more data once the NAS response is available.

RAN2 has sent an LS to CT1/SA2 describing the issue with the delay being taken to perform a write operation in response to a command message. This may delay the response to a NAS message from AIOT device. LS listed one possible solution as RAN2 considered was that the device NAS layer might generate and deliver to the device MAC layer an immediate NAS response upon successful reception of a command (e.g., a write command), without waiting for the write command to execute. Alternatively, CT1 or SA2 may configure a timer in the gNB which should take into account the time taken for AIOT device to generate the response. But this solution is in SA2 or CT1 remit.

In addition, indication of ‘0 SDU’ is already clear to indicate that the NAS response is not ready. So, we think there are enough mechanisms available and no “more data indication” is required.

Proposal 5: “more data indication” is not required.

# Summary

In this contribution, we have the following proposal for RAN2 to agree:

Proposal 1: 3 bits transaction ID is supported with 2 bits corresponding to reader ID and one bit indicating service ID.

Proposal 2: Presence of transaction ID is optional or value of “0” should be permitted.

Proposal 3: AS ID release can be handled by network implementation.

Proposal 4: Single R bit is introduced to all messages.

Proposal 5: “more data indication” is not required.

# References

- Email discussion POST130][027][AIoT] MAC Running CR (Huawei)