3GPP TSG-RAN WG2 Meeting #131												R2-2505663

Bangalore, India, Aug 25th – 29th, 2025

Agenda Item:	8.19.1

Source: 	Sony

Title:	LTM CG Resource consumption for the target cells [LTM_enh]

Document for:	Discussion and Decision

# Introduction

In Rel-18, LTM enhancements for intra-CU was specified which provided some mobility benefits of reducing HO latency and interruption time. However, there is a restriction that only mobility between cells of the same gNB (intra-CU) were supported in Rel-18 as shown on Figure 1.

In Rel-19, the benefits of Rel-18 LTM have been extended to support inter-CU scenario [2], i.e., cells of different gNBs as shown on Figure 2.

In this contribution, we discuss how to reduce the configured grant (CG) resource consumption for the candidate target cells for both intra-CU and inter-CU scenarios.

## 2. LTM CG Resource consumption for the target cells

In Rel-19 LTM, configured grant resources for the target candidate cells are allocated in the RRC reconfiguration message. As the resources are pre-allocated for multiple candidate cells even if they are not used, there is an issue of resource consumption (i.e., allocated but not used at the end). Hence, how to minimize the resource consumption is an issue to be solved for both intra-CU and inter-CU deployment scenarios.

In the current LTM procedure as shown on Figure 1 below, the CG resources for the target cell are allocated in the RRC reconfiguration message (in Step 8). In step 14, a UE receives the LTM Cell Switch Command to the target cell, and in step 16, the UE applies CG resources to transmit the first message (i.e., RRCReconfigurationComplete message) to indicate its arrival at the target cell.

Figure 1. Rel-19 LTM procedure (Figure 9.2.3.5.2-1. Signalling procedure for LTM) [2]

In order to minimize the CG resource consumption, the CG resources that are allocated in step 8 can be initially deactivated by default, and then when the serving cell receives L1 measurement report from the UE and the serving cell makes the decision to issue the LTM Cell Switch Command to the UE for a target cell (for example target cell index 1), the CG resources are activated only for that target cell (index 1). This means that activation message should be sent from the serving cell to the target cell only.

Proposal 1: RAN2 to discuss how minimize the configured grant resource consumption for the candidate target cells.

Proposal 2: CG resources of the target cell should be activated only after the serving cell receives L1 measurement report from the UE and the serving cell makes the decision to issue the LTM command to the UE for the target cell. The activation message should be sent from the serving cell to the target cell only.

As the activation message is sent from the serving cell to the target cell, some signalling impact is expected in RAN3, but we think implicit signalling would be enough by using existing message of CELL SWITCH NOTIFICATION in step 15 of Figure 1, that is: when target cell receives this message, it should activate the CG resources of that target cell only.

Proposal 3: Target cell should activate CG resources after receiving CELL SWITCH NOTIFICATION message from the source cell.

Based on proposal 3, it should be noted that in case of Inter-CU scenario, there could be noticeable delay for the target cell to activate the CG resources, in that case, an implementation can handle this issue whereby the serving cell slightly delays sending LTM CSC MAC CE to the UE as latency is better than interruption [3]. This may need slight changes to the current order of step 14 and step 15. We think step 15 should come earlier than step 14 by swapping them, or the order could be up to network implementation and a note can be added in the spec.

Proposal 4: Add a note in the spec that it is up to network implementation the order of step 14 and 15 where step 15 can be earlier than step 14.

## Conclusions

In this contribution, we have discussed how to reduce the configured grant resource consumption for the candidate target cells for both intra-CU and inter-CU scenarios. We have the following proposals:

Proposal 1: RAN2 to discuss how minimize the configured grant resource consumption for the candidate target cells.

Proposal 2: CG resources of the target cell should be activated only after the serving cell receives L1 measurement report from the UE and the serving cell makes the decision to issue the LTM command to the UE for the target cell. The activation message should be sent from the serving cell to the target cell only.

Proposal 3: Target cell should activate CG resources after receiving CELL SWITCH NOTIFICATION message from the source cell.

Proposal 4: Add a note in the spec that it is up to network implementation the order of step 14 and 15 where step 15 can be earlier than step 14.

## References

[1] RP-234036, “New WID: NR mobility enhancements Phase 4,” RAN#102.

[2] R2-2503732, “Introduction of NR mobility enhancements Phase 4 in TS 38.300”, draftCR	Rel-19	38.300.

[3] R2- 2408259, “Further discussion on Inter-CU LTM”, 3GPP TSG-RAN WG2 Meeting #127bis, MediaTek.