3GPP TSG-RAN WG2 Meeting #131					           R2-2505671

Bangalore, India, 25 - 29 August, 2025

Agenda Item:		8.3.2.1

Source:			Huawei, HiSilicon

Title:			Functionality management for RRM measurement prediction

Document for:		Discussion and Decision

# 1   Introduction

In RAN2#130 meeting, agreements related to functionality management were reached as follows [1]:

Agreements - Use the beam management agreements as the baseline, including the following aspects (details can be updated after further progress is made in AI/ML PHY BM use case) • Consider Option A and Option B like scheme (if/when AI/ML PHY makes further progress) • Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability. • Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly. • Support the explicit reporting of applicability/inapplicability in the initial report and subsequent reporting when the applicability changes. • Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability (FFS pending AI/ML PHY progress). • No prohibit timer is introduced. - Associated ID should be optionally configurable for training and inference (i.e. it may not mandatorily required for all training/inference configurations). FFS for WI phase further details (what absence means, which use case, whether it is per cell or multiple cells/frequency, terminology). Agreements - The following potential “inference parameters” can be considered for inference/measurement configuration, which provides the necessary information for UE to examine the applicability, it may contain • PW length (for temporal case A) • Skipping pattern (e.g. SSB config that indicates SSBs transmitted) (optional). MRRT (for temporal case B ) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted beam pattern (optional). MRRS (for spatial domain prediction) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted frequency (For inter-freq domain prediction) Model related choices (i.e., cluster-based vs. cell-based, L1 filtering, RRM sub-use cases, OW length, direct vs. indirect) can be up to UE implementation unless a requirement is identified to specify them. Agreements 1 For UE-sided model, NW-sided monitoring, the performance monitor procedure contains - NW sends monitoring configuration and inference configuration to UE. - UE reports measurements and inference output based on the corresponding configurations to NW. - NW performs monitoring and makes decisions, which can be fully NW-implemented. 2 For UE-sided model, UE-sided monitoring, the performance monitor procedure as baselined contains - NW sends monitoring configuration to UE - UE measures monitoring data and generates monitoring results by comparing inference output and monitoring data. - (NW-decision): UE reports the monitoring result to NW, and NW makes the management decision. - Can further consider UE sided monitoring and (UE-decision) for some use cases: UE makes the management decision based on network configuration and sends the decision to NW. FFS which use cases. - For RRM measurement prediction - RSRP differences can be used as the performance metric for monitoring.

In this Tdoc, we will discuss some remaining issues of functionality management for RRM measurement prediction. The document is divided into two parts which focus on applicability reporting and performance monitoring issues (section 2.1), and reporting related issues (section 2.2).

# 2   Discussion

## 2.1 Applicability reporting and performance monitoring

Figure 1 presents overall LCM procedure which is used for AIML for air interface developed in Rel-19.

Figure 1: LCM procedure agreed in AIML interface.

This framework is supposed to be reused for the AIML for mobility use case, including aspects such as applicability reporting and performance monitoring, as agreed by RAN2 previously. However, there are still some open issues which are supposed to be decided by RAN2.

Applicability reporting

Draft version of TR 38.744 which is an outcome of the post RAN2#130 meeting e-mail discussion captures the following editor’s notes:

Editor Note 1: It is FFS when UE can perform inference. Editor Note 2: The detail of inapplicability cause value is FFS Editor Note 3: The detail description of partial inference configuration can be further checked with progress of NR_AIML_air

In general, the intention of RAN2 was to reuse the applicability reporting procedure developed for AIML for air interface and specified in Rel-19 for AIML for mobility use case as much as possible. Therefore, there are some parts of the description which need to be updated based on the latest agreements RAN2 made within AIML for air interface work, in particular:

- Introduce “release configuration” flag instead of inapplicability cause to indicate UEs preference to release a configuration (e.g. due to model in availability in the local device FFS reason to be specified) - (RRC 7) RAN2 assumes applicability report for Option B (sets of inference related parameters) can be included in both RRCReconfigurationComplete and UAI (i.e., same as Option A). This can be revisited based on RAN1 conclusions/final signaling design. - (RRC8) RAN2 confirm that option A and option B can be configured in the same RRCReconfiguration message with the unified applicability report procedure.

Firstly, RAN2 made an agreement to introduce a release indication instead of inapplicability cause, so this needs to be corrected in TR 38.744.

Proposal 1: Correct a description in the TR to capture that the UE may include the release indication upon reporting inapplicability of the configuration (instead of “inapplicability cause”).

Furthermore, Option B description in the TR needs to be updated as per the agreements reached during RAN2#130 meeting as quoted above.

Proposal 2: Update the description of applicability reporting in the TR to clarify that:

- applicability report for Option B (sets of inference related parameters) can be included in both RRCReconfigurationComplete and UAI (i.e., same as Option A)

- option A and option B can be configured in the same RRCReconfiguration message with the unified applicability report procedure

UE sided monitoring and UE functionality management decisions

Another question related to functionality applicability is the UE behavior once a certain functionality becomes inapplicable. In RAN2#130 meeting, it was agreed that UE sided monitoring and UE autonomous functionality management decisions for some use cases can be considered, whereas the UE makes the management decision based on network configuration and sends the decision to NW.

Regarding temporal domain case B prediction, its purpose is to obtain measurement reduction. When some RS(es) are turned off at the NW side for saving RS resources, the UE cannot simply switch to non-AIML based measurements as the network needs to turn on the required signals first. The same problem exists for spatial domain prediction where some RS(es) may be turned off. In case of inter-frequency measurements on the other hand, the UE needs to be configured with measurement gaps which need to be provided by the network, as they are not required for frequency domain prediction. Hence, in these cases it is not possible to allow for UE autonomous decisions based on performance monitoring.

Observation 1: In case of temporal domain case B or spatial domain prediction, if NW want to reduce the RS overhead, some RS(es) may be turned off at NW side in case the UE performs prediction for certain RS(es) or time instances.

Observation 2: In case of frequency domain prediction, the UE may not use measurement gaps to predict another frequency RSRP while gaps are needed to perform traditional inter-frequency measurements.

Based on the above, in case of spatial domain prediction, frequency domain prediction and if the network reduces the RS overhead for temporal domain case B prediction, it is infeasible to allow UE to make functionality management decisions by itself. We then propose the following:

Observation 3: For frequency domain prediction and in case the network reduces RS overhead for temporal domain case B prediction or spatial domain prediction, UE sided monitoring with UE-decision cannot be allowed.

On the other hand, in case AIML based prediction is applied without the NW reducing RS overhead, UE autonomous switching to non-AIML based measurements is still possible for Case B or spatial domain prediction. Similarly, for temporal domain case A prediction, the RS(es) will be transmitted continuously, so the UE autonomous decision about whether to apply predicted results or actual measurements is feasible as well. Furthermore, the HO is a delay sensitive procedure, so it may be actually better if the UE does not wait for the network to switch to the regular measurements in case prediction accuracy deteriorates.

Observation 4: In case AIML based prediction is applied without the NW reducing RS overhead, UE autonomous switching to non-AIML based measurements is still possible for Case B or spatial domain prediction.

Observation 5: For temporal domain Case A prediction, it is feasible for the UE to autonomously switch from predicted measurements to regular measurements as all required RS(es) are continuously provided by the network.

Observation 6: The HO performance may deteriorate if the UE waits for the NW to switch from predicted measurements to regular measurements.

Based on these observations, we propose the following:

Proposal 3: UE sided monitoring with UE-decision can be supported for temporal domain Case A prediction and for temporal domain Case B and spatial domain prediction in case the NW does not reduce RS overhead.

## 2.2 Reporting aspects for different scenarios and sub-use cases

Prediction indication

Regarding the predicted RRM measurement reporting, one thing worth discussing is whether the UE needs to indicate whether the reported result is predicted or not. For RRM measurement prediction, one may argue that the gNB already knows implicitly whether reported results are predicted since it configures the measurement configurations (e.g., measurement ID, measurement object, reporting configuration). However, this also depends on whether a UE can be configured with AI and non-AI RRM measurement configuration simultaneously and whether a UE can autonomously fallback to non-AI configuration in some cases as discussed in section 2.1 (e.g. due to inapplicability or performance degradation).

Observation 7: For RRM measurement prediction, whether an indication about the reported result being either an actual measurement or predicted measurement depends on whether UE can be configured with AI and non-AI RRM measurement configurations at the same time and whether UE autonomous fallback is allowed.

Proposal 4: In case the UE is allowed by the network to autonomously switch from AIML to non-AIML based RRM measurements, an indication for whether the reported RRM measurement is a predicted or an actual result should be supported.

When to perform inference

In RAN2#129bis meeting, an FFS was captured on when the UE can perform inference once configured with predictions. During online discussion, some companies proposed that the UE can perform inference when the event entry condition is met based on actual measurement result, and some other companies proposed that the UE can perform inference when signal level drops below a certain threshold. We believe that when exactly the UE performs inference depends, e.g. on the use case which is configured:

- In case of RRM measurement prediction with periodic reporting over the serving cell, it seems obvious the UE needs to perform inference continuously.

- In case of RRM measurement prediction with periodic reporting over the neighbouring cells, also some additional conditions may apply, e.g. s-Measure. But once these conditions are met, the UE needs to perform inference continuously.

- In case of event triggered predicted RRM measurement reporting, the UE only needs to perform inference once an event that triggers prediction reporting is met.

- In case of measurement event prediction, when to perform inference depends on the PW length and TTT, e.g. in case the PW length is longer than TTT, it seems the UE should perform inference continuously. On the other hand, in case PW length is shorter or equal to TTT, then the UE may only start to perform inference once the entering condition of the event is met for a certain time (e.g. TTT-PW length) to verify that the event needs to be reported.

Observation 8: The exact time when the UE needs to perform inference depends on certain use case, reporting types, whether the measurements are for the serving cell or neighbouring cells etc.

Proposal 5:  The exact time of triggering inference follows the inference configuration provided from the network, e.g. PW length, time instances to be reported etc.

In general, all above discussions and proposals can be applicable for RRM measurement event prediction, since they share some common spec impacts.

Proposal 6:  The above proposal 1 to proposal 5 can be applied for RRM measurement event prediction.

# 3   Summary

In the paper, the functionality management for RRM measurement prediction is discussed, and the following observations and proposals are provided:

Applicability reporting and performance monitoring

Observation 1: In case of temporal domain case B or spatial domain prediction, if NW want to reduce the RS overhead, some RS(es) may be turned off at NW side in case the UE performs prediction for certain RS(es) or time instances.

Observation 2: In case of frequency domain prediction, the UE may not use measurement gaps to predict another frequency RSRP while gaps are needed to perform traditional inter-frequency measurements.

Observation 3: For frequency domain prediction and in case the network reduces RS overhead for temporal domain case B prediction or spatial domain prediction, UE sided monitoring with UE-decision cannot be allowed.

Observation 4: In case AIML based prediction is applied without the NW reducing RS overhead, UE autonomous switching to non-AIML based measurements is still possible for Case B or spatial domain prediction.

Observation 5: For temporal domain Case A prediction, it is feasible for the UE to autonomously switch from predicted measurements to regular measurements as all required RS(es) are continuously provided by the network.

Observation 6: The HO performance may deteriorate if the UE waits for the NW to switch from predicted measurements to regular measurements.

Proposal 1: Correct a description in the TR to capture that the UE may include the release indication upon reporting inapplicability of the configuration (instead of “inapplicability cause”).

Proposal 2: Update the description of applicability reporting in the TR to clarify that:

- applicability report for Option B (sets of inference related parameters) can be included in both RRCReconfigurationComplete and UAI (i.e., same as Option A)

- option A and option B can be configured in the same RRCReconfiguration message with the unified applicability report procedure

Proposal 3: UE sided monitoring with UE-decision can be supported for temporal domain Case A prediction and for temporal domain Case B and spatial domain prediction in case the NW does not reduce RS overhead.

Reporting aspects for different scenarios and sub-use cases

Observation 7: For RRM measurement prediction, whether an indication about the reported result being either an actual measurement or predicted measurement depends on whether UE can be configured with AI and non-AI RRM measurement configurations at the same time and whether UE autonomous fallback is allowed.

Observation 8: The exact time when the UE needs to perform inference depends on certain use case, reporting types, whether the measurements are for the serving cell or neighbouring cells etc.

Proposal 4: In case the UE is allowed by the network to autonomously switch from AIML to non-AIML based RRM measurements, an indication for whether the reported RRM measurement is a predicted or an actual result should be supported.

Proposal 5:  The exact time of triggering inference follows the inference configuration provided from the network, e.g. PW length, time instances to be reported etc.

Proposal 6:  The above proposal 1 to proposal 5 can be applied for RRM measurement event prediction.

# 4   References

- Report of 3GPP TSG RAN WG2 meeting #130, St Julian's, Malta