3GPP TSG-RAN WG2 Meeting #131	R2-2505678

Bengaluru, India, Aug 25th - 29th, 2025

Agenda item:		8.7.4

Source:		Samsung

Title:			Discussion on open issues for scheduling enhancements

Document for:	Discussion & Decision

# 1 Introduction

In this paper, we provide our view for the following open issues.

[PDCP-1] (Editorial) Whether the text “and are not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i” needs to be moved to the definition section for both delay-reporting PDCP SDU and non-delay-reporting PDCP SDU. [PDCP-2] Whether the PDCP triggers a PDCP SN gap report when at least one byte for the discarded PDCP SDU(s) have been submitted by any RLC entity to lower layers. [UE capability-01] Whether a UE supporting multipleEntryDelayStatusReport-r19 shall also indicate support of delayStatusReport-r18. [MAC-1] Discuss whether a pending DSR should be canceled if the UE has delay reporting SDUs but no delay critical SDUs or all delay critical SDUs have been reported. [MAC-2] Discuss whether all pending BSRs may be cancelled if all the data eligible for inclusion in a BSR MAC CE is reported in a DSR MAC CE. [MAC-6] Whether any spec change is needed to capture the agreement on DSR cancellation in the DC case.

# 2 Discussion

- [PDCP-1] (Editorial) Whether the text “and are not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i” needs to be moved to the definition section for both delay-reporting PDCP SDU and non-delay-reporting PDCP SDU.

The present definitions of delay-reporting PDCP SDU and non-delay-reporting PDCP SDU are not uniquely associated with i:th dsr-ReportingThreshold and therefore, it is suggested to amend the definitions (see textbox) to make them distinct and definite.

Delay-reporting PDCP SDU: if pdu-SetDiscard is not configured, a delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold is a PDCP SDU for which the remaining time till discardTimer expiry is less than the i:th dsr-ReportingThreshold and larger than or equal to the i-1:th dsr-ReportingThreshold (if i>1) or larger than zero (if i=1). If pdu-SetDiscard is configured, a delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold is a PDCP SDU belonging to a PDU Set of which the PDU Set remaining time is less than the i:th dsr-ReportingThreshold and larger than or equal to the i-1:th dsr-ReportingThreshold (if i>1) or larger than zero (if i=1), and is not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i. Non-delay-reporting PDCP SDU: a non-delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold is a PDCP SDU that will be transmitted prior to any of the delay-reporting PDCP SDUs associated with the i:th dsr-ReportingThreshold and that is not a delay-reporting PDCP SDU, and is not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i.

With the above definition changes in place, the procedural part can be greatly simplified (i.e., the yellow highlighted text can be completely omitted)

For the purpose of multiple entry MAC delay status reporting, the transmitting PDCP entity shall evaluate the delay-reporting PDCP data volume in ascending order of dsr-ReportingThreshold, and consider the following as delay-reporting PDCP data volume associated with the i:th dsr-ReportingThreshold: - the delay-reporting PDCP SDUs associated with the i:th dsr-ReportingThreshold for which no PDCP Data PDUs have been constructed, and are not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i; - the PDCP Data PDUs that contain the delay-reporting PDCP SDUs associated with the i:th dsr-ReportingThreshold and have not been submitted to lower layers, and are not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i; - if i = 1, the PDCP Control PDUs; - if i = 1, for AM DRBs, the PDCP SDUs to be retransmitted according to clause 5.1.2 and clause 5.13; - if i = 1, for AM DRBs, the PDCP Data PDUs to be retransmitted according to clause 5.5. If dsr-ReportNonDelayCriticalData is configured, the transmitting PDCP entity shall further consider the following as delay-reporting PDCP data volume associated with the i:th dsr-ReportingThreshold: - the non-delay-reporting PDCP SDUs associated with the i:th dsr-ReportingThreshold for which no PDCP Data PDUs have been constructed, and are not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i; - the PDCP Data PDUs that contain the non-delay-reporting PDCP SDUs associated with the i:th dsr-ReportingThreshold and have not been submitted to lower layers, and are not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i.

[PDCP-1] Proposal 1: Move the text “and are not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i” to the definition section for both delay-reporting PDCP SDU and non-delay-reporting PDCP SDU. (Adopt TP1)

- [PDCP-2] Whether the PDCP triggers a PDCP SN gap report when at least one byte for the discarded PDCP SDU(s) have been submitted by any RLC entity to lower layers.

### 5.16.1 Transmit operation For UM DRBs and AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink (sn-GapReport in TS 38.331 [3]), the transmitting PDCP entity shall trigger a PDCP SN gap report when: - PDCP SDU(s) are discarded as specified in clause 5.3; and - there is at least one stored PDCP SDU(s) which is associated with a COUNT value larger than the COUNT value associated to the discarded PDCP SDU(s); and - the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers.

In legacy R18, upon receiving a discard indication for a RLC SDU from PDCP, a RLC SDU is either fully submitted to lower layer, or fully discarded at RLC (when no segment is submitted to lower layer).

With R19 AM RLC enhancements (i.e. stopReTxDiscardedSDU), we can have a new scenario where an RLC SDU has been incompletely submitted (in the form of SDU segment) to the lower layer, when becomes obsolete upon receiving the discard indication from PDCP. Such SDUs will never be delivered to the receiving PDCP entity, and end up with being discarded at the receiving side of AM RLC entity after t-RxDiscard expiry. However, based on the legacy triggering conditions of PDCP SN gap report, there will be no report triggered for such SDUs, and hence, the receiving PDCP entity has no chance to know about the discard status for such incompletely submitted SDUs. This causes the reoderering delays to creep in.

Therefore, with R19 AM RLC enhancements in place, we have an issue about the highlighted legacy phrase “the discarded PDCP SDU(s)” in the third condition as it does not address the new scenario. More precisely, for R19 specifications, the third condition should be enhanced to “at least one byte for the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers” while triggering a PDCP SN gap report.

The benefit is that the PDCP SN gap report can early and accurately notify more discard information to the peer receiving PDCP entity and it will prevent unnecessary re-ordering delay.

[PDCP-2] Proposal 2: When stopReTxDiscardedSDU is configured for AM DRB, PDCP triggers a PDCP SN gap report when at least one byte for the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers. (Adopt TP2)

- [UE capability-01]: Whether a UE supporting multipleEntryDelayStatusReport-r19 shall also indicate support of delayStatusReport-r18.

We have already agreed and implemented in running CR that if at least one LCG is configured with reporting threshold, R19 DSR will be used, and all the LCGs configured with triggering threshold, will be provided with reporting threshold(s). It implies that R19 DSR can work independently with R18 DSR, without any correlation between them, e.g., coexistence or fallback. Thus, UE capability for R19 DSR and R18 DSR can be deemed as independent capabilities without correlation.

[UE capability-01] Proposal 3: There is no dependency between multipleEntryDelayStatusReport-r19 and delayStatusReport-r18.

- [MAC-1] Discuss whether a pending DSR should be canceled if the UE has delay reporting SDUs but no delay critical SDUs or all delay critical SDUs have been reported.

If a UE has delay-reporting data but no delay-critical data, the current running CR will cancel the pending DSR. This is because the association between a PDCP SDU and a pending DSR is determined based on the triggering threshold. Since the R19 DSR reports information specifically related to delay-reporting data, the pending DSR remains valuable and should still be reported, even in scenarios where there is no delay-critical data but delay-reporting data is still pending for reporting.

For such purpose, we should modify the association criterion between a PDCP SDU and a DSR that the association should be determined based on the reporting threshold(s) if configured.

[MAC-1] Proposal 4: A PDCP SDU is considered to be associated with a DSR if it has not been transmitted in any MAC PDU and is a delay-reporting PDCP SDU associated with the logical channel which triggered the DSR.

- [MAC-2] Discuss whether all pending BSRs may be cancelled if all the data eligible for inclusion in a BSR MAC CE is reported in a DSR MAC CE.

If all the pending data has been reported by DSR, we see no reason to additionally report BSR (with the same buffer size information as DSR). To save signalling overhead, it is reasonable to cancel the pending BSR in such a case, considering the padding BSR can serve the similar purpose if UL resource is adequate.

[MAC-2] Proposal 5: All the pending BSRs should be cancelled if all the data eligible for inclusion in a BSR MAC CE is reported in a DSR MAC CE.

- [MAC-6] Whether any spec change is needed to capture the agreement on DSR cancellation in the DC case.

The agreements made in last RAN2 meeting on the same issue, captured what the common understanding is, without introducing any further spec change (as captured below), which was yielded by a contradictory discussion as a compromise. The further discussion for the same topic should not be re-initiated, to avoid duplicated contradictory discussion, considering the time budget for the other open issues that have not been discussed yet.

- => (MAC-03) For DC case, no further enhancements on DSR due to transmission of DSR MAC CE in the other MAC entity and this DSR MAC CE with the delay information of all the PDCP SDUs associated with the DSR. - => (MAC-03) In DC, no enhancements are needed when a pending DSR is cancelled because all its associated PDCP SDUs have been discarded or included in a MAC PDU. - => (MAC-03) An understanding is that if MAC PDU is sent in one MAC entity, then the other MAC entity will see that there is no PDCP SDU associated with DSR and will cancel the DSR.

[MAC-6] Proposal 6: No spec change is needed to capture the DSR cancellation in the DC case.

# 3 Conclusion

Based on the discussion above, request RAN2 to consider the following proposals:

[PDCP-1] Proposal 1: Move the text “and are not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i” to the definition section for both delay-reporting PDCP SDU and non-delay-reporting PDCP SDU. (Adopt TP1)

[PDCP-2] Proposal 2: When stopReTxDiscardedSDU is configured for AM DRB, PDCP triggers a PDCP SN gap report when at least one byte for the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers. (Adopt TP2)

[UE capability-01] Proposal 3: There is no dependency between multipleEntryDelayStatusReport-r19 and delayStatusReport-r18.

[MAC-1] Proposal 4: A PDCP SDU is considered to be associated with a DSR if it has not been transmitted in any MAC PDU and is a delay-reporting PDCP SDU associated with the logical channel which triggered the DSR.

[MAC-2] Proposal 5: All the pending BSRs should be cancelled if all the data eligible for inclusion in a BSR MAC CE is reported in a DSR MAC CE.

[MAC-6] Proposal 6: No spec change is needed to capture the DSR cancellation in the DC case.

# 4 Annex: Text Proposals

TP 1

TS 38.323 ## 3.1 Definitions …. Delay-reporting PDCP SDU: if pdu-SetDiscard is not configured, a delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold is a PDCP SDU for which the remaining time till discardTimer expiry is less than the i:th dsr-ReportingThreshold and larger than or equal to the i-1:th dsr-ReportingThreshold (if i>1) or larger than zero (if i=1). If pdu-SetDiscard is configured, a delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold is a PDCP SDU belonging to a PDU Set of which the PDU Set remaining time is less than the i:th dsr-ReportingThreshold and larger than or equal to the i-1:th dsr-ReportingThreshold (if i>1) or larger than zero (if i=1), and is not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i. ….. Non-delay-reporting PDCP SDU: a non-delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold is a PDCP SDU that will be transmitted prior to any of the delay-reporting PDCP SDUs associated with the i:th dsr-ReportingThreshold and that is not a delay-reporting PDCP SDU, and is not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i.

TP 2

TS 38.323 ### 5.16.1 Transmit operation For UM DRBs and AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink (sn-GapReport in TS 38.331 [3]), the transmitting PDCP entity shall trigger a PDCP SN gap report when: - PDCP SDU(s) are discarded as specified in clause 5.3; and - there is at least one stored PDCP SDU(s) which is associated with a COUNT value larger than the COUNT value associated to the discarded PDCP SDU(s); and - if stopReTxDiscardedSDU is configured for AM DRB and at least one byte for the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers; otherwise, the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers. ….