3GPP TSG-RAN WG2 Meeting #131	R2-2505694

Bangalore, India  Aug 25th – 29th, 2025

Agenda item:	8.12.3

Source:	Lenovo

Title:	Remaining issues on UEI report

WID/SID:	NR_MIMO_Ph5-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

There are some remaining issues on a scenario that the first PUCCH and second PUSCH are configured on different serving cells and the two serving cells may belong to different TAGs. the scenario was mentioned and suggested to be further discussed by Rapporteur.

In this document, we discuss the related issues and some more issues on another case.

# 2		Discussion

Besides the case on the scenario mentioned during the email discussion, we think there could be another case that the PUCCH resource and the type-1 CG are configured on different serving cells, while the serving cell on which the type-1 CG for mode-B UEIBR is configured is deactivated. Since the PUCCH resource and the type-1 CG resource are associated for UEI report, another one resource is useless if either one of the resources is unavailable.

Further, it could be 1) the BWP or serving cell that the type-1 CG is configured on is deactivated or 2) the TAT corresponding to a TAG to which the resource belongs expires considering the different cases that cause the either one resource unavailable. For the first case, the paired PUCCH resource is released once the BWP or CC on which the type-1 CG resource is configured is deactivated. For the second case, the other one should also be released/cleared if either one resource is released/cleared due to TAT expired.

Proposal 1: Once the BWP or serving cell on which the type-1 CG resource for UEI report is configured is deactivated, the associated PUCCH resource is released.

Proposal 2: If either one resource for UEI report is released/cleared due to TAT expired the associated another resource for UEI report is also cleared/released.

It has been agreed in RAN2#130 that the existing rule is applied to handle the overlapping/prioritization between the PUSCH of mode-A UE-initiated report and SR/other PUSCH in MAC. No MAC impact is expected. While it seems some clarification on the report itself is needed.

Considering the scenario for UEIBR trigger and transmission procedure, the UEIBR may be failed to transmission e.g. because of the resource collision. Further, new UEIBR may be triggered by new or same event before the previous UEIBR is transmitted successfully. It may be allowed to cancel a UEIBR e.g. the measurement result is included by other UEIBR if the UEIBR is triggered but has not been transmitted or has not been transmitted successfully.

Proposal 3: A UEIBR which is triggered but has not been transmitted or has not been transmitted successfully, may be cancelled.

For the overlapping case, RAN1 agreed on beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding priority rules for CSI report multiplexing/dropping, UEI beam report for both mode-A and mode-B is prioritized over Semi-persistent CSI reports on PUCCH and Periodic CSI reports on PUCCH

- UEI beam report for mode-A > Aperiodic CSI report > UEI beam report (for mode-B) > Semi-persistent CSI reports on PUSCH

It seems to be RAN2 scope to discuss on the case of type-1 CG resource for mode-B overlapping with another PUSCH resource. In legacy, lch-basedPrioritization is configured to prioritize the urgent traffic when overlapping occurs. Considering the reason to enhance beam report by UEI report, it is because UE has better and more-timely knowledge of beam quality changes, UE-initiated beam reporting procedure can lead to more timely beam reports yet with reduced reporting overhead. Under such a procedure, if the UE determines that e.g., current beam(s) quality becomes poor, UE can trigger beam reporting without the network needing to configure or trigger frequent reporting. To some extent, UEI report is also an urgent traffic. The base station misses the first PUCCH from UE and schedules a dynamic resource which overlaps with the second PUSCH in mode-B. If there is not rule similar with lch-basedPrioritization applied to UEI report, the beam report may be dropped which is unexpected.

Proposal 4: lch-basedPrioritization is enabled when mode-B UEIBR is configured.

Correspondingly, there is a scenario that new data arrives before a SR occasion and the SR occasion is ahead of the CG type 1 resource. In legacy, the SR may not be triggered because the CG type 1 resource is considered as available resource. If the CG type 1 resource is for UEI report. SR shall be triggered since the CG type-1 PUSCH carrying the beam report of Mode-B does not carry MAC PDU (i.e. UL-SCH). Then, the MAC entity considers the CG type-1 PUSCH resource configured for mode-B UEIBR is not available from the perspective of RAN2.

Proposal 5: To avoid the impact on triggering a SR for the new data arrival, the MAC entity considers the CG type-1 PUSCH resource configured for mode-B UEIBR is not available.

# 3	Conclusion

In summary, we have following proposals for UEIBR:

Proposal 1: Once the BWP or serving cell on which the type-1 CG resource for UEI report is configured is deactivated, the associated PUCCH resource is released.

Proposal 2: If either one resource for UEI report is released/cleared due to TAT expired the associated another resource for UEI report is also cleared/released.

Proposal 3: A UEIBR which is triggered but has not been transmitted or has not been transmitted successfully, may be cancelled.

Proposal 4: lch-basedPrioritization is enabled when mode-B UEIBR is configured.

Proposal 5: To avoid the impact on triggering a SR for the new data arrival, the MAC entity considers the CG type-1 PUSCH resource configured for mode-B UEIBR is not available.

# 4	References

[1] RAN1#121 chairman notes.

[2] RAN2#130 chairman notes.