3GPP TSG-RAN WG2 Meeting #131	R2-2505709

Bangalore, India, Aug 25th – 29th, 2025

Agenda item:	8.2.3

Source:	Spreadtrum, UNISOC

Title:	Discussion on left issues of A-IoT random access

Document for:	Discussion and Decision

1 Introduction

The document includes a list of open issues for A-IoT random access [1], some issues have been addressed. In this contribution, we will discussion the left issues to be discussed by company contributions.

2 Discussion

## 2.1 MSG2 monitor boundary in CBRA

In last meeting, the option of MSG2 transmission and any retransmission of MSG2 happens within a predefined time window (based on timer) has been excluded. A device expecting MSG2 assumes CBRA failure if its MSG2 is not received before a boundary, where the boundary can be further down selected between option B and C below. A device receiving MSG2 within this boundary transmits MSG3. The device does not process MSG2 (re)transmission received after the boundary.

RAN2#130 Agreement: further down selection between option B and C for MSG2 monitor window in CBRA. - Option B – the boundary is the reception of either the next R2D trigger message or the subsequent paging message - Option C – the boundary is the reception of either the kth R2D trigger message or the subsequent paging message (K is FFS) - Option A (the boundary being the subsequent paging only) is excluded. - For option C, further discuss in terms of complexity at the device vs reader flexibility.

We prefer option B, device assumes MSG1 success when receiving its MSG2 before the next R2D trigger message or the subsequent paging message. In this case, MSG2 only need to support multiplexing at most X*Y RN16s with less RN16 collisions and less latency for device in success case.

While option C, device assumes MSG1 success when receiving its MSG2 before the next K R2D trigger messages or subsequent paging, MSG2 need to support multiplexing at most K*(X*Y) RN16s with more RN16 collisions and more latency for device in success case. Furthermore, a single R2D triggering message can trigger up to 2*8 devices to connect. We believe that the reader has sufficient capacity to handle these device connection requests and respond with MSG2 before next R2D trigger message or the subsequent paging message.

Proposal 1: A device assumes CBRA failure if it’s MSG2 is not received before the next R2D trigger message or the subsequent paging message.

## 2.2 Frequency index in MSG2 to handle collision

In previous meetings, RAN2 discussed whether MSG2 need to include more information (i.e. frequency index) on top of the random ID to avoid random ID collision, but there was no consensus [2].

RAN2 #130 Agreement: Including frequency index along with RN16 in MSG2 to reduce collisions of MSG1 between different devices is feasible. FFS Discuss further whether to include it.

We prefer the frequency index of RN16 is not included in MSG2. First, the probability of RN16 collision in a MSG2 is very low, about 0.02289% according the contribution [3]. Second, the device that experienced the RN16 collision fails to access and then can re-accesses, with no significant impact.

Proposal 2: Don not support including frequency index along with RN16 in MSG2.

3 Conclusion

In this contribution we discuss the left issues of the random access, with the following proposals:

Proposal 1: A device assumes CBRA failure if it’s MSG2 is not received before the next R2D trigger message or the subsequent paging message.

Proposal 2: Do not support including frequency index along with RN16 in MSG2.

- References

- Remaining A-IoT MAC open issues, Huawei, HiSilicon, 3GPP RAN WG2 Meeting #131, 2025.

- Draft RAN2 #130 meeting report, RAN2 Chair (InterDigital), St.Julians, Malta, 19th –23rd May, 2025.

- R2-2503952 A-IoT random access procedure, Huawei, HiSilicon, St.Julians, Malta, 19th –23rd May, 2025.