3GPP TSG-RAN WG2 Meeting #131	R2-2505738

Bengaluru, India, 25-29 August, 2025

Agenda Item:	8.17

Source: 	Huawei, HiSilicon

Title: 	Remaining issues of IoT-NTN TDD

Document for:	Discussion and Decision

- Introduction

In RAN#105, a new work item was approved to introduce NB-IoT TDD mode exclusively for 1616-1626.5 MHz Mobile Satellite Services MSS band [1]. During the RAN1#119 meeting, some agreements were made for the TDD pattern and the corresponding frame structure [2], as listed below.

Agreement： Regarding operation within the same band as the TDD frame structure of legacy system in the 1.6GHz MSS band (shown below), RAN1 work in Rel-19 considers the following design constraints: - At the satellite, all downlink NB-IoT channels/signals in a cell can only use one of the downlink slots in the TDD frame structure (DL1, DL2, DL3 or DL4) across 90ms periods. - The same downlink slot is used in all the 90ms periods. - At the satellite, all uplink NB-IoT channels in a cell can only use one of the uplink slots in the TDD frame structure (UL1, UL2, UL3 or UL4) across 90ms periods. - The same uplink slot is used in all the 90ms periods. - The one uplink slot and one downlink slot in the TDD frame structure have the same index (DL1 & UL1, DL2 & UL2, DL3 & UL3, or DL4 & UL4). - NOTE: this does not imply that the only configuration(s) to be specified are according to these constraints.

The WID was revised in RAN#106 [3] to reflect RAN1 progress and narrow down the scope of the solutions.

In RAN2#130 meeting [4], the following agreements were reached for RAN2 impacts of this new WID.

Agreements: 1. The SI-message transmission can be postponed to the next valid D frame within the SI-Window 2. It is up to NW implementation to avoid SI-window overlap 3. SI repetitions will not overlap (in case of collision the subsequent SI repetition is postponed) 4. In IoT-NTN TDD mode, the RA-RNTI should be calculated based on the SFN of the first radio frame in which the Random-Access Preamble is transmitted (i.e. no spec change) 5. For the timer of ra-ResponseWindowSize and mac-ContentionResolutionTimer, the absolute value limitation for FDD (i.e., 10.24s) is used for IoT NTN TDD. 6. In IoT-NTN TDD mode the same formula as for RA-RNTI calculation for FDD is reused 7. No extension is needed on the value range of timer in unit of ms or s for IoT NTN TDD 8. The remaining paging repetitions falling on the invalid DL SFNs are postponed to the next valid DL SFNs. 9. It is up to network to configure the gap between two POs (i.e., parameter NB) to be sufficiently long such that it includes enough number of valid DL subframes for NumRepetitionPaging-r13 (no spec impact) 10. Introduce the following definition for IoT-NTN TDD mode in the impacted RAN2 specifications: IoT-NTN TDD mode: allows use of NB-IoT channels with TDD mode for NTN with fixed values of D non-overlapping usable contiguous DL subframes and set of U usable contiguous UL subframes separated by fixed guard period (can revisit this based on the TP being prepared by RAN1) 11. In Rel19, RAN2 will not work on any specific enhancements to ensure that the features being specified in IoT_NTN_Ph3-Core will also work for IoT NTN TDD mode 12. In IoT_NTN_Ph3-Core, RAN2 will not work on any specific enhancements to ensure that the features being specified in IoT_NTN_Ph3-Core will also work for IoT NTN TDD mode. RAN2 understands that, as part of the IoT_NTN_TDD WI, we can discuss on a case by case basis whether minor specific enhancements – not affecting other WGs - can be supported to ensure that (some of) the features being specified in IoT_NTN_Ph3-Core will also work for IoT NTN TDD mode

In this paper, we further discuss the remaining issue of IoT NTN TDD.

- Discussion

  - Impact on Measurement

For NB-IoT, inter-frequency measurements on neighbour can be performed in RRC_CONNECTED to reduce RRC re-establishment time, as specified in TS 36.300. This should also be allowed in an IoT NTN TDD cell.

TS 36.300: For NB-IoT, measurements in RRC_CONNECTED are optionally supported to reduce the time taken for RRC reestablishment. The following principles are applied: - The "current cell" above refers to the configured carrier in the serving cell. The "target cell" above refers to the anchor carrier in the target cell. For instance, for the definition of intra and inter frequency measurements, this means: - Intra-frequency neighbour (carrier) measurements: Neighbour carrier measurements performed by the UE are intra-frequency measurements when the configured carrier in the serving cell and the anchor carrier in the target cell operates on the same carrier frequency. The UE shall be able to carry out such measurements without measurement gaps. - Inter-frequency neighbour (carrier) measurements: Neighbour cell measurements performed by the UE are inter-frequency measurements when the configured carrier in the serving cell and the anchor carrier in the target cell operates on a different carrier frequency. The UE may not be able to perform such measurements without measurement gaps. - The eNB configures the criteria to perform measurements via broadcast signalling; - Dedicated measurements gaps are not supported. The UE may need to perform neighbour cell measurements during DL/UL idle periods that are provided by DRX or packet scheduling; - Measurement reporting is not supported.

For a UE in the IoT NTN TDD cell, it cannot perform any service outside the available UL and DL subframes due to the new frame structure. Therefore it is better to allow the UE to perform the inter-frequency neighboring cell measurement outside the available UL and DL subframes when there is no data transmission, so that the normal data transmission during the serving cell’s available UL and DL subframes wouldn’t be affected by the measurement.

Proposal 1: RAN2 to confirm that inter-frequency neighboring cell measurement can be performed outside the serving cell's available UL and DL subframes to avoid interfering the normal data transmission during the serving cell’s available UL and DL subframes.

  - Open issue CAP-1

RAN1 has indicated that the UE supporting band 249 must support this FG. And the supported bands will be indicated as part of the UE capabilities in supportedBandList-r13. In this case, if UE indicates support of band 249, it shall support IoT NTN TDD. Therefore, it is actually a conditionally mandatory feature as captured in the current running CR.

Proposal 2: Capability of IoT NTN TDD mode is defined as a conditionally mandatory feature as in the 36.306 running CR.

- Conclusion

The observations and proposals made in this contribution are summarized below:

Impact on Measurement

Proposal 1: RAN2 to confirm that inter-frequency neighboring cell measurement can be performed during the GAP configured outside the serving cell's available UL and DL subframes to avoid interfering the normal data transmission during the serving cell’s available UL and DL subframes.

Open issue CAP-1

Proposal 2: Capability of IoT NTN TDD mode is defined as a conditionally mandatory feature as in the 36.306 running CR.

- References

RP-242415, New WID on introduction of IoT-NTN TDD mode

Chair notes RAN1#119 eom1

RP‑243293, Revised WID on introduction of IoT-NTN TDD mode

R2-2504673, Report from Break-out session on NR-NTN and IoT-NTN