3GPP TSG-RAN WG2 Meeting #131						R2-2505742

Bangalore, India, August. 25–29, 2025

Source:	Continental Automotive

Title:                       AS ID release procedure for CFRA

Agenda Item:         8.2.4

Document for:	Discussion and Decision

# Introduction

In RAN2#130 meeting, following agreement were mode for release AS ID. [1]

Agreements

-	For CBRA, to avoid AS ID being occupied for unnecessary time and to keep alignment between reader and device on AS ID release, device can release AS ID upon receiving paging message with different transaction ID, no matter the paging message is for it or not.   FFS for CFRA

-	FFS for need for release message

In this contribution, we discuss remaining aspect that how device release AS ID for CFRA.

# Discussion

In the RAN2#130 agreement, it was agreed that for CBRA, the device shall release its AS ID upon receiving a paging message with a different transaction ID, regardless of whether the paging message is addressed to the device or not. This approach is reasonable for CBRA, especially since multiple devices may be active simultaneously. Introducing an explicit release message in CBRA would increase signaling overhead and complexity, which is not desirable.

However, in the case of CFRA, the situation is different. Since only one device communicates with the reader at a time, the reader has full control over the session and can determine precisely when the procedure for that device is complete. Therefore, it is more beneficial to introduce an explicit AS ID release message in CFRA. This approach ensures clear alignment between the reader and the device, avoids AS ID being held longer than necessary, reduces device complexity, and simplifies device behaviour.

Observation 1: In CBRA, implicit AS ID release via paging is efficient and necessary to avoid signaling overload. In contrast, in CFRA, since only one device is active per reader at a time, the reader has full control over the session.

Proposal 1: We propose introducing an explicit AS ID release message for CFRA, which the reader sends to the device to indicate the end of the session and trigger AS ID release.

# Conclusions

In this document, we summarize procedure to release AS ID for CFRA. Additionally, we request RAN2 to discuss the following observation and proposal:

Observation 1: In CBRA, implicit AS ID release via paging is efficient and necessary to avoid signaling overload. In contrast, in CFRA, since only one device is active per reader at a time, the reader has full control over the session.

Proposal 1: We propose introducing an explicit AS ID release message for CFRA, which the reader sends to the device to indicate the end of the session and trigger AS ID release.

# References

[1] RAN2#130 meeting chair note