3GPP TSG-RAN WG2 Meeting #131                                 R2-2505768

Bangalore, India, Aug.25th- 29th, 2025

Agenda item:		8.2.2

Source:		Samsung

Title:			Discussion on the remaining issues on A-IoT paging

Document for:	Discussion & Decision

- 1 Introduction

In RAN2#130, the AIoT paging was extensively discussed with the following agreements:

Agreements

- Rel-19 devices are not expected to receive parallel service request for overlapping reader scenario based on network implementation.   Capture this in stage 2 specification.

- The Rel-19 device always responds to the new service indicated by the received paging message applicable for that device.  Capture this in stage 3 specification.

- Send LS to RAN3 to notify them of agreements 1 and 2

- Parallel service request for overlapping reader scenario can be addressed in Rel-20

Agreements on CFRA

- For CFRA, as a baseline the fields related to the transaction ID, indication of paging ID present/absent and number of access occasions are absent.  FFS on the need for the transaction ID for command case.

- For CFRA, the device always responds to paging regardless of transaction ID (if we put a transaction ID) (i.e. as long as it is addressed to the corresponding device).

- To ensure forward compatibility for paging with multiple identifiers, introduce at least one R field.   FFS if more than one R bit is required.

- Rel-19 devices would ignore the content of future release instead of ignoring the whole paging message.

- Issue (1-4) For number of access occasions introduce exponential way, 4 bits, value range FFS

In this contribution, we will share our views on the remaining issues.

- 2 Discussion

Issue 1: Configuration mismatch between device and reader

For CBRA, the fields of “Number of Access Occasions” and “D2R Scheduling Info” are used by the device to determine the access occasions for Msg.1 transmission. For the same inventory process, multiple A-IoT paging messages can be sent with the same transaction ID. With the implementation of the inventory procedure, the number of un-inventoried devices is decreased so that the reader can update these two fields among A-IoT paging messages with the same Transaction ID. For example, in the following figure, the first A-IoT paging message includes Number of Access Occasions = 10 (=1024), and m = 16. Before the second A-IoT paging message, the reader has successfully inventoried many devices; thus, the second A-IoT paging message includes the updated information, i.e., Number of Access Occasions = 8 (=256), and m = 8. Such update can save the resource, and also the inventory procedure can be carried out efficiently.

Fig. 1 Information update in A-IoT paging message

If the update mentioned above is allowed, the configuration mismatch between device and reader may be possible if the device misses the A-IoT paging message. As shown in Fig. 2, one example is given:

- The lost of one Access Trigger message: the device cannot start the Msg. 1 transmission before the second A-IoT paging message;

- The configuration update in the second A-IoT paging message: number of Access occasions = 8, and m = 8;

- The lost of second A-IoT paging message: the device performs counting down operation and Msg. 1 resource selection according to the old configuration, i.e., the device selects wrong resource for Msg. 1 (the one in the second time domain resource)

Fig. 2 Configuration mismatch between device and reader

The above configuration mismatch may result in the interference to the Msg.2 transmission to other devices, i.e., the resource used by the Msg.2 to other devices are interfered by the Msg. 1 from the device that misses the second A-IoT paging message.

Proposal 1: RAN2 is kindly asked to discuss whether the configuration mismatch between reader and device exists due to the miss of the A-IoT paging message with the updated configurations (i.e., number of access occasions, D2R scheduling info. ).

If the validity of the above configuration mismatch can be confirmed, RAN2 may need discuss whether and how to address it. The following solutions can be considered:

- Option 1: do nothing

In this option, the device can use the old configuration to perform Msg.1 resource selection. The resultant interference may cause the Msg. 1 lost for the device with configuration mismatch or Msg.2 lost for the devices without configuration mismatch. Then, those devices can reuse the current procedure (Clause 5.5 in TS38.391) to identify the failure.

- Option 2: no configuration update among A-IoT paging message with the same Transaction ID

This option requires to limit the configuration at the reader side. Since each Paging message has the same configuration, the configuration mismatch cannot happen.

- Option 3: add configuration ID in A-IoT paging message and Access trigger message

In this option, the configuration ID can be used to associate the access trigger messages with the paging message, i.e., the paging message and the access trigger message(s) with the same configuration ID share the same configuration. When the device receives the paging message, it has to store the configuration ID included in the paging message. If the device receives the access trigger message with configuration ID different from the stored one, the device considers the CBRA procedure is failed.

Proposal 2: RAN2 is kindly asked to discuss whether and how to address the configuration mismatch issue, and, if needed, the following options can be considered:

- Option 1: do nothing

- Option 2: no configuration update among A-IoT paging message with the same Transaction ID

- Option 3: add configuration ID in A-IoT paging message and Access trigger message

Issue 2: length of transaction ID

RAN2 has agreed that “Rel-19 devices are not expected to receive parallel service request for overlapping reader scenario based on network implementation”. Thus, the length of transaction ID can be short. On the other hand, the Paging message has to additionally include 128bits security information, which increases the length of A-IoT paging message. To reduce the signalling overhead, it is better to have a short length of Transaction ID.

Proposal 3: RAN2 is kindly asked to agree the length of Transaction ID as 4 bits.

Issue 3: size optimization for frequency resource indication

During post-email discussion, some companies raise the concern on the number of bits used to indicate the bit duration and frequency resource indication in D2R scheduling information field, which can be 9 bits instead of 11 bits in the current running CR. The intention is to reduce the signalling overhead. If 9 bits are used, the device may have complex processing since the device need to determine the number of bits used for bit duration based on the first “1” in the bit duration & frequency resource indication. In addition, 9 bits design may not future proof if RAN2 defines new bit duration in the future. So, we propose to not have such optimization.

Proposal 4: RAN2 is kindly asked to use separate fields for bit duration and frequency resource indication (as the current running CR).

- 3 Conclusion

Based on the discussion above, we have the following proposals:

Proposal 1: RAN2 is kindly asked to discuss whether the configuration mismatch between reader and device exists due to the miss of the A-IoT paging message with the updated configurations (i.e., number of access occasions, D2R scheduling info. ).

Proposal 2: RAN2 is kindly asked to discuss whether and how to address the configuration mismatch issue, and, if needed, the following options can be considered:

- Option 1: do nothing

- Option 2: no configuration update among A-IoT paging message with the same Transaction ID

- Option 3: add configuration ID in A-IoT paging message and Access trigger message

Proposal 3: RAN2 is kindly asked to agree the length of Transaction ID as 4 bits.

Proposal 4: RAN2 is kindly asked to use separate fields for bit duration and frequency resource indication (as the current running CR).