3GPP TSG-RAN WG2 Meeting #131                                                              R2-2505773

Bangalore, India, Aug. 25th- 29th, 2025

Agenda item:	8.13.2

Source:	Samsung

Title:	Discussion on remaining issues of relay discovery and (re)selection

WID/SID:	NR_SL_relay_enh2

Document for:	Discussion and Decision

Introduction

In last RAN2 meeting, the relay discovery and (re)selection was extensively discussed for multi-hop SL relay, and reached the following agreement:

Agreements:

The notification message is used for the L2 child UE in case of upstream relay (re)selection triggered by AS layer failure cases (Uu/PC5 link failure/release, using legacy cause value) where the upper layer does not trigger link release towards the child.  For reselection triggered by upper layers we rely on the upper-layer notification/release mechanism already defined.

Remote UE handling of the notification is up to upper layer implementation.

Further handling of any inter-layer issues on this topic can be done in maintenance phase.

LS to SA2 to notify them of these agreements.

[Chair’s note: The second agreement above was clarified in a later session as indicated below, such that the remote UE handling of “the notification” in the second agreement refers to the upper-layer notification.]

Agreement:

Leaving remote UE handling of the notification to implementation applies to the upper-layer notification for remote UEs in RRC_IDLE/RRC_INACTIVE.  UEs in RRC_CONNECTED follow the later agreement to trigger RRC reestablishment.  No case is expected where a UE in RRC_CONNECTED receives the upper-layer notification.

Agreements:

Notification is always sent for AS failure cases at the intermediate relay UE (e.g., upstream RLF that prevents the intermediate relay from having a path to the network), using the legacy mechanism.  FFS new cause values.

In principle, RAN2 consider that no AS-layer notification is sent by an intermediate relay UE in RRC_IDLE/RRC_INACTIVE when the AS-visible parameters that could affect relay reselection are not changed.  FFS which parameters are related.  This issue can be revisited if it is found that there are parameters not visible to the AS layer that would affect reselection and force the remote UE to be notified.

FFS if notification behaviour can be optimized for intermediate relay UEs in RRC_CONNECTED.

Agreements:

Upon receiving the notification message or upon PC5 link release, remote UE in RRC_CONNECTED needs to initiate RRC Reestablishment as legacy.

When the intermediate Relay UE in RRC_CONNECTED receives the notification message from its parent intermediate Relay UE/last Relay UE or when the PC5 link is released, it initiates RRC Reestablishment.

In this contribution, we will address the remaining issues for the discovery and relay selection/re-selection.

Discussion

According the current agreements, the following table shows the failure handlings which has been agreed and those which needs further discussions:

|  | RRC state | Trigger condition | Resultant behaviour |
| --- | --- | --- | --- |
| Remote UE | RRC_CONNECTED | Reception of notification message; PC5 link release | RRC re-establishment |
|  | RRC_CONNECTED | AS failure case (e.g., PC5 failure) | RRC re-establishment |
|  | RRC_IDLE/INACTIVE | Upper layer notification | Remote UE implementation |
|  | RRC_IDLE/INACTIVE | AS failure case (e.g., PC5 failure) | Relay (re)selection |
| Intermediate relay UE | RRC_CONNECTED | Reception of notification message; PC5 link release | RRC re-establishment FFS (Issue 1: RRC-5): send notification message transmission |
|  | RRC_CONNECTED | AS failure case | Send notification message TBD (Issue 2): recovery procedure |
|  | RRC_IDLE/INACTIVE | Upper layer notification | No such case |
|  | RRC_IDLE/INACTIVE | AS failure case + the AS-visible parameters that could affect relay reselection are not changed | Not send notification FFS (Issue 3: RRC-11): which AS-visible parameters are related |

Failure handling

Issue 1 (RRC-5): the transmission of notification message at the intermediate relay UE (in RRC_CONNECTED) if a notification message from parent node is received

In RAN2#130, the following agreements were reached:

When the intermediate Relay UE in RRC_CONNECTED receives the notification message from its parent intermediate Relay UE/last Relay UE or when the PC5 link is released, it initiates RRC Reestablishment.

During RRC reestablishment, the intermediate relay UE will perform either cell selection or relay selection or both (if both a suitable cell and a suitable relay are available, the intermediate relay UE can select either one based on its implementation). According to the current specification, the cell selection or relay selection will not trigger the notification message transmission. Thus, a new triggering of notification message should be defined, i.e., the reception of notification message.

Proposal 1: RAN2 is kindly asked to agree a new trigger of notification message transfer, i.e., reception of notification message.

Then, the following-up question is how to set the content of the notification message by the intermediate relay UE. This is related to the FFS in the following agreements in RAN2#129bis:

- When the intermediate relay UE receives a notification message from the last relay UE indicating a failure on Uu, the intermediate relay UE may transmit a notification message downstream (towards the remote UE). FFS if the notification message is a forwarded copy of the original notification (same cause) or a regenerated message from the intermediate relay (cause might reflect its own reaction such as relay reselection or re-establishment). This does not change the agreement that it is up to relay implementation whether to release the downstream link, and if the link is released there is no downstream notification message.

Two options can be considered:

- Option 1: a copy of the notification from the parent node

  - Option 2: regenerate the notification message based on its own behavior

In our understanding, the intention of notification message is to reflect the status of the sending node rather than nodes indirectly connecting to the sending node. Thus, it is natural to use Option 2.

Proposal 2: after receiving notification message from parent node, the intermediate relay UE in RRC_CONNECTED regenerates the notification message based on its own behaviour.

As the following-up step, we need decide how to set the indication type in the notification message. This depends on the timing of sending the notification message:

- Option 1: the sending time is immediately after the initiation of re-establishment procedure at such intermediate relay UE

This option can trigger the re-establishment of the child node quickly. As we know, the re-establishment of such intermediate node can be triggered by PC5 link failure as well. The indication type is related to whether there is a benefit to differentiate two trigger conditions, i.e., PC5 link failure, and the reception of notification message. In our understanding, no matter which trigger condition is satisfied, the resultant behaviour of child node is the initiation of re-establishment procedure. Thus, the indication type can be set to relay UE re-establishment.

- Option 2: the sending time is after getting the results of re-establishment procedure

This option delays the initialization of re-establishment of child node if the re-establishment of the intermediate node is failed. On the other hand, if the re-establishment of such intermediate relay UE is success, the child node can carry out re-establishment via such intermediate node directly, which can speed up its re-establishment procedure. In a word, the re-establishment results can cause different behaviours of child node. Thus, in this option, the indication type can indicate the re-establishment results to the child node, e.g., re-establishment success, re-establishment failed.

Compared to Option 1, we prefer to Option2. The reason is that in practice, the re-establishment can be success in most of cases, which can speed up the re-establishment of child node.

Proposal 3: RAN2 is kindly asked to agree that, after reception of notification message from the parent node, the intermediate node carries out the following behaviors:

- Perform the re-establishment procedure

- Send notification message to its child node after deriving the re-establishment results

- Include the results of re-establishment results (i.e., success or failure) in the notification message

Issue 2: AS failure at the intermediate relay UE in RRC_CONNECTED

RAN2 has agreed the following:

Notification is always sent for AS failure cases at the intermediate relay UE (e.g., upstream RLF that prevents the intermediate relay from having a path to the network), using the legacy mechanism.  FFS new cause values.

However, how to recover the link is not determined. The procedure of remote UE can be applied, i.e., initialize RRC re-establishment procedure.

Proposal 4: RAN2 is kindly asked to agree that the intermediate relay UE in RRC_CONNECTED initializes the RRC re-establishment procedure after AS failure (i.e., PC5 failure).

Issue 3 (RRC-11): Relay reselection for intermediate node in RRC_IDLE/INACTIVE

In RAN2#130, the following agreement were reached:

In principle, RAN2 consider that no AS-layer notification is sent by an intermediate relay UE in RRC_IDLE/RRC_INACTIVE when the AS-visible parameters that could affect relay reselection are not changed.  FFS which parameters are related.  This issue can be revisited if it is found that there are parameters not visible to the AS layer that would affect reselection and force the remote UE to be notified.

The following Note is added to the running CR:

Note 1: The Notification Message may not be sent by an intermediate relay UE in RRC_IDLE or RRC_INACTIVE to its child UEs if there are no changes to AS-visible parameters (e.g., the hop count provided by upper layers or the serving cell of the last U2N relay UE) that could impact relay reselection. Editor’s Note: FFS which parameters are related. This issue can be revisited if it is found that there are parameters not visible to the AS layer that would affect reselection and force the remote UE to be notified.

In our understanding, if the hop count or the serving cell of the intermediate node is not changed, it means that the relay reselection does not change the path condition for the intermediate node if it wants to establish connection. Thus, it is unnecessary to trigger the notification message transmission.

Proposal 5: RAN2 is kindly asked agree that the intermediate relay UE in RRC_IDLE/INACTIVE can omit the notification message transmission if the relay reselection does not cause the change of the hop count or serving cell.

Issue 4 (RRC-12): cause value for relay reselection

The intermediate relay may trigger relay reselection due to its own link problem. This is different from the current indication type in notification message. Thus, it is better to add this cause information to indicate that the notification is sent due to its own problem.

Proposal 6: RAN2 is kindly asked to agree relay UE reselection as the new cause information in notification message

Admission control

In multihop relay, the number of direct/indirect child nodes for an relay UE may be large compared to the single hop case. If the intermediate node is overloaded, it cannot allow additional child node to access. Currently, the admission control is performed at the gNB only, which is determined the load at the gNB side without consideration of relay UE capability. Thus, the remote UE may access an overloaded intermediate relay UE. To avoid such case, it is better to allow the overloaded relay UE to indicate its capacity limitation to its child node so that the remote UE can carry out the relay selection/reselection to the relay UE with available capacity. To realize this, the notification message can include a new cause information, i.e., no access allowed, which indicates that remote UE access is not allowed due to capacity limitation.

Proposal 7: RAN2 is kindly asked to agree a new cause in notification message to indicate that the UE access is not allowed.

Conclusion

In this contribution, we address some issues for the relay discovery and (re)selection, and have the following proposals:

Proposal 1: RAN2 is kindly asked to agree a new trigger of notification message transfer, i.e., reception of notification message.

Proposal 2: after receiving notification message from parent node, the intermediate relay UE in RRC_CONNECTED regenerates the notification message based on its own behaviour.

Proposal 3: RAN2 is kindly asked to agree that, after reception of notification message from the parent node, the intermediate node carries out the following behaviors:

- Perform the re-establishment procedure

- Send notification message to its child node after deriving the re-establishment results

- Include the results of re-establishment results (i.e., success or failure) in the notification message

Proposal 4: RAN2 is kindly asked to agree that the intermediate relay UE in RRC_CONNECTED initializes the RRC re-establishment procedure after AS failure (i.e., PC5 failure).

Proposal 5: RAN2 is kindly asked agree that the intermediate relay UE in RRC_IDLE/INACTIVE can omit the notification message transmission if the relay reselection does not cause the change of the hop count or serving cell.

Proposal 6: RAN2 is kindly asked to agree relay UE reselection as the new cause information in notification message.

Proposal 7: RAN2 is kindly asked to agree a new cause in notification message to indicate that the UE accesses is not allowed.

References