3GPP TSG-RAN WG2 Meeting #131										R2-2505779

Bangalore, India, 25th – 29th August 2025

Agenda item		: 8.4.2

Title				: Open issues on LP-WUS in IDLE/INACTIVE mode

Source				: Tejas Networks

Document for		: Discussion and Decision

# Introduction

This contribution discusses the open issues related to LP-WUS operation in IDLE and INACTIVE mode.

# Discussion

## Evaluation period for Entry/Exit condition:

The main advantage of using LR to monitor wake up indication is that MR can move to ultra-deep sleep/ deep sleep state to save power. The currently agreed entry and exit conditions only consider the signal qualities of LP-SS/SSB measured by LR/MR. Considering just the thresholds might lead to ping-pong effect of moving between use of LR and MR resulting in UE not having sufficient time to move to between sleep states. This can even result in adverse effect as there is power penalty associated with transition between sleep states. In order to overcome this, we propose to reuse the similar principle used in Rel16 relaxed measurement criteria where there is a time period configured for which the evaluation criteria need to be satisfied to consider the condition as fulfilled.

Proposal 1: Introduce evaluation time period for which the entry/exit evaluation criteria should be satisfied to consider the Entry/Exit condition is met to switch between LR/MR.

## Enabling/Disabling of LP-WUS operation – (RRC-12)

During the post-email discussion [1], there was a discussion on how to enable/disable LP-WUS operation in IDLE/INACTIVE state of UE and following options were proposed:

    - Solution 1: NAS based solution

    - Solution 2: RRC based solution

    - Solution 3: Based on CN no configuring CN based subgrouping ID (Apple)

    - Solution 4: Disabling based on UE/NW comparing the UE i-DRX cycle length and lo_frame_offset

The key concern to have some solution specified for enabling/disabling LP-WUS feature was for the case of UE having services which are latency sensitive and/or have more frequent wake ups resulting in inefficient UE operation.

Analysis on the proposed solutions:

- Solution 1: RRC based solution has few issues which needs further discussion such as the behaviour on cell reselection, lack of full information on UE session at gNB.

- Solution 2: In case of NAS based solution, this appears to be straightforward but this needs more impact in other WGs (RAN3) as well to enable it. As pointed by companies during discussion, to have better decision for power saving vs latency trade-off at CN on whether to enable/disable LP-WUS, there could be a need for RAN specific configuration (lo_frame_offset etc) to be sent to CN.

- Solution 3: This solution is based on implicit disabling in case of CN based subgroup ID is not configured to a UE during the registration process. This relies on UE supporting CN based subgrouping and also required the gNB to not broadcast UE ID based subgrouping. This solution appears to have restrictions on both UE and NW and might not be suitable for all deployment cases.

- Solution 4: As proposed in [2], the solution does not require any additional signalling as the UE and NW just needs to compare the UE idle/inactive DRX cycle length and offset value applicable to the UE and determine if LP-WUS is monitored or not. It also is simpler to handle the issue of UE behaviour on reselecting to a new cell as UE applies the configurations applicable to that cell. This solution is also agnostic to the subgrouping method used as it applies equally to both UE ID based and CN based subgrouping. It also appears to have the least impact to specification and can be addressed by RAN2 WG alone.

Proposal 2: Consider the solution of using applicable DRX cycle and applicable lo_frame_offset value to determine if LP-WUS operation is enabled for a UE or not

# Conclusion

Proposal 1: Introduce evaluation time period for which the entry/exit evaluation criteria should be satisfied to consider the Entry/Exit condition is met to switch between LR/MR.

Proposal 2: (RRC-12) Consider the solution of using applicable DRX cycle and applicable lo_frame_offset value to determine if LP-WUS operation is enabled for a UE or not

# Reference

[1] Summary of [Post130][222][LPWUS] Potential solution to support enabling/disabling LP-WUS monitoring in IDLEI/NACTVE per UE (Huawei)

[2] R2-2503659 Remaining issues on LP-WUS in IDLE and INACTIVE, CATT