3GPP TSG-RAN WG2 Meeting #131										R2-2505782

Bengaluru, India, August 25th – 29th, 2025

Agenda item		: 8.4.4

Title				: Open issues on LP-WUS operation in CONNECTED mode

Source				: Tejas Networks

Document for		: Discussion and Decision

# Introduction

This contribution discusses the open issues related to LP-WUS operation in CONNECTED mode.

# Discussion

## On supporting LP-WUS and Dual DRX case:

In previous RAN2 meeting, the following working assumption regarding support of LP-WUS with dual DRX group. - Working assumption: LP-WUS can be configured on the PCell with secondary DRX. LP-WUS with secondary DRX is supported with option 1-1 and 1-2, i.e. the UE monitors LP-WUS before the on-duration occasion or periodically outside ActiveTime. When LP-WUS is detected, then UE starts the drx-onDurationTimer (with option 1-1) or the lpwus-PDCCHMonitoringTimer (with option 1-2) in both DRX groups.

During the post email discussion [1], companies discussed on UE behavior on LP-WUS monitoring occasion during the Active time of any of the DRX groups as based on RAN1 LS [2], MR and LR cannot operate simultaneously.

In such cases, it is a simpler design to have UE start the drx-onDurationTimer when UE is unable to monitor LP-WUS when LO occurs during active time (any of DRX groups)

Proposal 1: UE starts the next drx-onDurationTimer in both the DRX groups when the LO overlaps with active time of any of the DRX groups if dual DRX groups is configured.

## UE behavior when LR is unable to monitor LP-WUS occasion:

During the post email discussion [1] it was discussed on how to handle the case of UE being unable to monitor LOs associated with option 1-2 due to various collisions. In case we go in the direction of not starting the lpwus-PDCCHMonitoringTimer if the UE is unable to monitor LP-WUS, this might result in UE missing data meant for the UE. On the other hand, if we go with starting the timer if UE misses, and given that the case of periodicity for LO of option 1-2 could be smaller, this might result in unnecessary monitoring of PDCCH taking a hit on the energy efficiency. To have a consistent behavior, it would be simpler to make the behaviour configurable by the network as this can make sure that UE and network are in sync with the monitoring UE behaviour.

Proposal 2: UE behaviour of starting lpwus-PDCCHMonitoringTimer in case of UE unable to monitor LO associated with option1-2 is configured by network.

# Conclusion

Proposal 1: UE starts the next drx-onDurationTimer in both the DRX groups when the LO overlaps with active time of any of the DRX Groups in case of dual DRX being configured.

Proposal 2: UE behaviour of starting lpwus-PDCCHMonitoringTimer in case of UE unable to monitor LO associated with option1-2 is configured by network.

# Reference

- [Post130][213] [LPWUS] Running CR for 38.321

- R2-2505020	RAN1 Reply LS on LP-WUS in RRC_CONNECTED