3GPP TSG-RAN2 Meeting #131	R2-2505794

Bangalore, India, 25 - 29 August 2025

Agenda Item:	8.13.3

Source:	Ericsson

Title:	Discussion on control plane procedures

Document for:	Discussion, Decision

# 1	Introduction

For the WI on NR sidelink multiple-hop relay, the below objectives on control plane procedures have been defined in the WID [1].

The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3]

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays.

  - Relay discovery and (re)selection [RAN2]

  - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3]

  - Impact on SRAP and QoS handling for multi-hop [RAN2]

  - Control plane procedures [RAN2, RAN3]

We discuss aspects on control plane procedures in this paper. All discussions refer to L2 U2N Remote UE or L2 U2N Relay UE unless otherwise stated.

# 2	Discussion

## 2.1 Reflective SRAP configuration

RAN2 has made the below agreements regarding SRAP at RAN2#129bis

- SRAP configuration including remote UE ID and BEARER ID is configured by the network via dedicated RRC signalling as in R17.

- For the SRAP configuration at the relay UE (both last relay and intermediate relay UE), rely on network to provide SRAP configuration for at least the directly connected child UEs, based on the L2 IDs of the direct connected child UEs.

- For CR drafting, assume as a baseline that the child UE’s SRAP configuration can include entries for indirect child UE with associated local ID for next-hop determination.  We will review next meeting if this explicit mapping functionality is needed.

- FFS whether to introduce reflective bearer mapping as an optional feature.

And for how to configure the Remote UE ID and BEARER ID, in R17 U2N Relay, network configures the SRAP configuration for one or more remote UEs at the U2N Relay as follows:

-- ASN1START -- TAG-SL-L2RELAYUE-CONFIG-START SL-L2RelayUE-Config-r17 ::= SEQUENCE { sl-RemoteUE-ToAddModList-r17 SEQUENCE (SIZE (1..maxNrofRemoteUE-r17)) OF SL-RemoteUE-ToAddMod-r17 OPTIONAL, -- Need N sl-RemoteUE-ToReleaseList-r17 SEQUENCE (SIZE (1..maxNrofRemoteUE-r17)) OF SL-DestinationIdentity-r16 OPTIONAL, -- Need N ..., SL-RemoteUE-ToAddMod-r17 ::= SEQUENCE { sl-L2IdentityRemote-r17 SL-DestinationIdentity-r16, sl-SRAP-ConfigRelay-r17 SL-SRAP-Config-r17 OPTIONAL, -- Need M ... } -- TAG-SL-L2RELAYUE-CONFIG-STOP -- ASN1STOP

As shown in the above specification, the network configures the SRAP configuration for each remote UE at the U2N Relay UE based on the L2 ID of the remote UE.

RAN2 has agreed the baseline that the child UE’s SRAP configuration can include entries for indirect child UE with associated local ID for next-hop determination.  We will review next meeting if this explicit mapping functionality is needed.

Based on the above baseline, each relay UE (i.e., last relay UE and intermediate relay UE) is configured by the network with the SRAP configuration for each directly connected child UEs wherein the SRAP configuration entry is indexed by the L2 ID of each connected child UE. Each SRAP configuration entry of each directly connected child will also include a list of SRAP configuration entries for indirectly connected child UEs. The ASN.1 (as captured in the RRC running CR) is illustrated in the below

SL-RemoteUE-ToAddMod-r17 ::=       SEQUENCE {

sl-L2IdentityRemote-r17            SL-DestinationIdentity-r16,

sl-SRAP-ConfigRelay-r17            SL-SRAP-Config-r17                                                      OPTIONAL,    -- Need M

...,

[[

sl-SRAP-ConfigRelay-ToAddModList-r19     SEQUENCE (SIZE (1..maxNrofRemoteUE-r17)) OF SL-SRAP-Config-ToAddMod-r19      OPTIONAL,      -- Need R

sl-SRAP-ConfigRelay-ToReleaseList-r19     SEQUENCE (SIZE (1..maxNrofRemoteUE-r17)) OF SL-SRAP-ConfigId-r19               OPTIONAL      -- Need R

]]

For the above baseline approach, two drawbacks are observed

- Upon reception of a SRAP data PDU, the relay UE needs to first locate the matching SRAP configuration entry according to the local ID contained in the SRAP header, then to find the L2 ID of the directly connected child. The searching procedure is two-step wise, which is not efficient.

For the baseline approach, upon reception of a SRAP data PDU, the relay UE needs to first locate the matching SRAP configuration entry according to the local ID contained in the SRAP header, then to find the L2 ID of the directly connected child. The searching procedure is two-step wise, which is not efficient.

Instead, it is better to introduce a flatter structure. So, for each relay UE, the relay UE can be configured with SRAP configuration for each child UE separately/independently, regardless of it is directly connected or indirectly connected. In addition, such alternative may be easier for relay UE to search the matching entry when receiving a SRAP data PDU, the relay UE doesn’t need to perform nested search. In such alternative, we can introduce a new field nextHopL2ID-r19 in SL-RemoteUE-ToAddMod-r17, which is only present for indirectly connected child UE, the egress link for indirectly connected child UE is determined based on nextHopL2ID.

By adopting a flattened structure for the SRAP configuration of each relay UE, it is easier to find matching SRAP entity upon reception of an SRAP data PDU since the searching procedure is 1 step wise, in addition, it is feasible for the gNB to provide delta configuration for each indirectly connected child UE.

By adopting a flattened structure for the SRAP configuration of each relay UE, it is easier to find matching SRAP entity upon reception of an SRAP data PDU since the searching procedure is 1 step wise.

By adopting a flattened structure for the SRAP configuration of each relay UE, the legacy ASN.1 (i.e., remote UE add/mod/release list) can be reused without additional spec change needed.

For the SRAP configuration at the relay UE (both last relay and intermediate relay UE), re-use legacy remote UE add/mod/release list to configure indirect remote UE’s SRAP configuration in intermediate relay UE, i.e., the flattened structure for SRAP configuration is adopted.

In the flattened structure for SRAP configuration, at each relay UE, the SRAP configuration is configured for each indirectly connected child UE, in this case, an additional field is needed to indicate the next hop L2 ID in DL mapping, since the child UE’s own L2 ID is not feasible to be used for determining the egress link.

Regardless of if the baseline approach or the flattened structure for SRAP configuration is eventually adopted, reflective mapping is feasible to be supported for each relay UE.

Reflective mapping comprises reflective mapping for egress DL link and reflective mapping for egress DL relay RLC channel.

For the former, for each received SRAP data PDU, the relay UE can determine the egress link based on the PC5 link from which the local ID (of the SRAP data PDU) is received. Reflective mapping for determining the egress link can be always applied (if the relay UE supports it), since each child UE will always start with SRB transmission.

Reflective mapping for determining the egress link can be always applied (if the relay UE supports it), since each child UE will always start with SRB transmission.

For the latter, the relay UE can determine the egress RLC channel based on the RLC channel from which the RB ID (of the SRAP data PDU) is received. Reflective mapping for determining the egress RLC channel may not always be feasible (even if the relay UE supports it), since each child UE may not always start with UL transmission for an RB (e.g., SRB2 and/or DRBs). When the gNB provides SRAP configurations to each relay UE, it is not clear on whether a remote UE will start its UL transmission for an SRB2 and/or DRBs. Therefore, the gNB cannot determine whether reflective mapping can be applied for the RB. Based on this analyses, we think it is reasonable to not support reflective mapping for egress RLC channel determination

Reflective mapping for determining the egress RLC channel may not always be feasible (even if the relay UE supports it), since each child UE may not always start with UL transmission for an RB (e.g., SRB2 and/or DRBs).

Reflective mapping is only supported for egress link determination.

If reflective mapping for determining the egress link is applied (if the relay UE supports it), the SRAP configuration for a child UE can ignore the field sl-DLNextHop-L2Identity-r19 for the flattened structure. While the field sl-LocalIdentity-r17 is still needed to be included in the SRAP configuration for each child for indexing the corresponding SRAP configuration.

For reflective mapping, to discuss and down-select between below options.

  - Option 1 - reflective mapping is mandatory supported for indirectly connected child, no new field is needed to be included.

  - Option 2 - reflective mapping is optional, include a new field sl-DLNextHop-L2Identity-r19 in SRAP configuration, based on absence or presence of the field, it is indicated that the intermediate relay UE shall apply reflective mapping or not.

## 2.2 Open issues in running CR

Open issues have been summarized with proposals as below in the offline discussion:

[Post130][407][Relay] Rel-19 relay merged CR to 38.331 (Huawei)

Proposal 3: Update TS 38.331, Section 3.1, to include the following clarification in the definitions of Intermediate U2N Relay UE and First Relay UE:

“An Intermediate U2N Relay UE first establishes a connection to the network as a U2N Remote UE, before transitioning to operate as a U2N Relay UE.”

Proposal 4: Confirm that the Remote UE can request the SFN-DFN offset from the connected parent Relay UE and that the SFN-DFN offset provided at the L2 U2N Relay UE or the L2 Last U2N Relay UE can be forwarded by intermediate U2N Relay UEs within the multihop relay chain, as proposed in the current running CR.

Proposal 5 – RAN 2 should discuss if there are any parameter not visible to AS layer that could change and would affect reselection and force the remote UE to be notified.

Proposal 6 – RAN2 to confirm that the Intermediate Relay UE sets the indication type to a specific value,  relayUE-RelayReselection, to clearly signal that the notification is triggered by its own reselection action, which follows a notification of AS failure received from its parent relay UE.

Proposal 7 – RAN2 to discuss if we need to further fine tune the timers value calculation for the multihop scenario.

Proposal 8 – RAN2 should clarify in TS 38.300 or TS 38.331 that, for Scenario C and Scenario D, the Remote UE shall report only Relay UEs in RRC_CONNECTED state for path switching, based on the Relay State indication received in the discovery message RRC container.

For the above the issue related to P3,

We think it is unnecessary to update the RRC running CR to additionally include the update

“An Intermediate U2N Relay UE first establishes a connection to the network as a U2N Remote UE, before transitioning to operate as a U2N Relay UE.”

Since there is already text in stage 2 running CR to capture the same clarification, which is sufficient. We shall avoid duplicate texts in the RRC.

No need to include the below text in RRC the definitions of Intermediate U2N Relay UE and First Relay UE

  - An Intermediate U2N Relay UE first establishes a connection to the network as a U2N Remote UE, before transitioning to operate as a U2N Relay UE.

For the above the issue related to P4,

We agree with the issue and the solution that the rapporteur has suggested. In other words,

Proposal 4: Confirm that the Remote UE can request the SFN-DFN offset from the connected parent Relay UE and that the SFN-DFN offset provided at the L2 U2N Relay UE or the L2 Last U2N Relay UE can be forwarded by intermediate U2N Relay UEs within the multihop relay chain, as proposed in the current running CR.

Confirm that the Remote UE can request the SFN-DFN offset from the connected parent Relay UE and that the SFN-DFN offset provided at the L2 U2N Relay UE or the L2 Last U2N Relay UE can be forwarded by intermediate U2N Relay UEs within the multihop relay chain, as proposed in the current running CR.

For the above the issue related to P5, we see no need to further discuss if there are any parameter not visible to AS layer that could change and would affect reselection and force the remote UE to be notified. The existing agreed AS parameters are sufficient in this release.

No need to further discuss if there are any parameter not visible to AS layer that could change and would affect reselection and force the remote UE to be notified.

For the above the issue related to P7, we think it is sufficient to keep the current RAN2 agreement. Given there are only up to two additional hops of relays supported in this release. We think it is acceptable to scale the timers to up to 3 times in this release.

Keep the current RAN2 agreement on the timers i.e., Extend the T300, T301 and T319 timers for multi-hop U2N relay operation. The UE and gNB multiply the signalled timer value by the hop count.

For the above the issue related to P8, we see no strong motivation to change the spec on UE reporting behaviours. It can be up to UE implementation to handle/consider RRC state of candidate UEs (received in the discovery message).

It is up to (remote) UE implementation to handle/consider RRC state of candidate UEs (if received in the discovery message).

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	For the baseline approach, upon reception of a SRAP data PDU, the relay UE needs to first locate the matching SRAP configuration entry according to the local ID contained in the SRAP header, then to find the L2 ID of the directly connected child. The searching procedure is two-step wise, which is not efficient.

Observation 2	By adopting a flattened structure for the SRAP configuration of each relay UE, it is easier to find matching SRAP entity upon reception of an SRAP data PDU since the searching procedure is 1 step wise.

Observation 3	By adopting a flattened structure for the SRAP configuration of each relay UE, the legacy ASN.1 (i.e., remote UE add/mod/release list) can be reused without additional spec change needed.

Observation 4	Reflective mapping for determining the egress link can be always applied (if the relay UE supports it), since each child UE will always start with SRB transmission.

Observation 5	Reflective mapping for determining the egress RLC channel may not always be feasible (even if the relay UE supports it), since each child UE may not always start with UL transmission for an RB (e.g., SRB2 and/or DRBs).

Based on the discussion in the previous sections we propose the following:

Proposal 1	For the SRAP configuration at the relay UE (both last relay and intermediate relay UE), re-use legacy remote UE add/mod/release list to configure indirect remote UE’s SRAP configuration in intermediate relay UE, i.e., the flattened structure for SRAP configuration is adopted.

Proposal 2	Reflective mapping is only supported for egress link determination.

Proposal 3	For reflective mapping, to discuss and down-select between below options.

a.	Option 1 - reflective mapping is mandatory supported for indirectly connected child, no new field is needed to be included.

b.	Option 2 - reflective mapping is optional, include a new field sl-DLNextHop-L2Identity-r19 in SRAP configuration, based on absence or presence of the field, it is indicated that the intermediate relay UE shall apply reflective mapping or not.

Proposal 4	No need to include the below text in RRC the definitions of Intermediate U2N Relay UE and First Relay UE

a.	An Intermediate U2N Relay UE first establishes a connection to the network as a U2N Remote UE, before transitioning to operate as a U2N Relay UE.

Proposal 5	Confirm that the Remote UE can request the SFN-DFN offset from the connected parent Relay UE and that the SFN-DFN offset provided at the L2 U2N Relay UE or the L2 Last U2N Relay UE can be forwarded by intermediate U2N Relay UEs within the multihop relay chain, as proposed in the current running CR.

Proposal 6	No need to further discuss if there are any parameter not visible to AS layer that could change and would affect reselection and force the remote UE to be notified.

Proposal 7	Keep the current RAN2 agreement on the timers i.e., Extend the T300, T301 and T319 timers for multi-hop U2N relay operation. The UE and gNB multiply the signalled timer value by the hop count.

Proposal 8	It is up to (remote) UE implementation to handle/consider RRC state of candidate UEs (if received in the discovery message).

# 4	References

[1] RP-241609, “New WID on NR sidelink multi-hop relay”, 3GPP TSG RAN Meeting #104, Shanghai, China, 17 – 20 June 2024.