3GPP TSG-RAN WG2 #131	R2-2505797

Bangalore, India, 25th – 29th August 2025

Agenda Item:	8.13.4

Source:	Ericsson

Title:	Service Continuity for Multi-Hop Relays

Document for:	Discussion, Decision

# 1 	Introduction

In this contribution, we discuss remaining issues for service continuity.

# 2 	Discussion

Regarding RRC state of candidate first/intermediate/last relay UE on a relay path, RAN2 has made the below agreements at RAN2#130

- For scenario C, the legacy path-switching command and procedure are reused for the Remote UE and the target first Relay UE. Upon receiving the path-switching command, the Remote UE performs the legacy UE behavior toward the target first Relay UE. The (first)intermediate/last Relay UE belonging to the target path should be in the RRC-connected state.

- For scenario D, the legacy path-switching command and procedure are reused for the Remote UE and the first Relay UE in the target path. Upon receiving the path-switching command, the Remote UE performs legacy UE behavior toward the first Relay UE on the target path. The (first)intermediate/last Relay UE belonging to the target path should be in the RRC-connected state.

Based on the agreements, the gNB will only consider candidate first/intermediate/last relay UE which are in RRC_CONNECTED on a relay path. From the current RRC running CR, the above agreements have not been captured. Therefore, it is suggested to discuss whether these agreements need to be captured in specs or not.

To discuss whether the agreements on RRC state of (first)intermediate/last Relay UE need to be captured in spec or can be up to gNB’s implementation to handle.

In addition, companies have suggested in previous RAN2 meetings to further enhance remote UE measurement report to include number of hops or accumulated QoS for each candidate relay paths. In our view, it can be fully up to the gNB implementation on how to consider such E2E metrics. No further optimization on remote UE measurement report is needed in this release.

Not pursue further optimization to remote UE’s measurement report regarding E2E metrics, e.g., inclusion of number of hops or accumulated QoS in measurement report.

# 3 	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	To discuss whether the agreements on RRC state of (first)intermediate/last Relay UE need to be captured in spec or can be up to gNB’s implementation to handle.

Proposal 2	Not pursue further optimization to remote UE’s measurement report regarding E2E metrics, e.g., inclusion of number of hops or accumulated QoS in measurement report.