3GPP TSG-RAN WG2 #131	R2-2505822

# Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	8.8.4

Source:	Ericsson

Title:	Support for broadcast services in NR NTN

Document for:	Discussion, Decision

# 1	Introduction

The third WID on NTN for NR was agreed in [1], which includes an objective on specifying signalling for broadcast services, as follows:

- Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3]

- Specify SIB signalling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2]

- Specify the necessary signalling between CN and NG-RAN. [RAN3]

This contribution presents our views on the final enhancements needed to secure a smooth service continuity for MBS NTN.

# 2	Discussion

## 2.1	Service continuity due to satellite movement

### 2.1.1	Background

MBS Service continuity for UEs in RRC_IDLE or RRC_INACTIVE was enhanced with the following mechanisms to reduce service interruption and improve user experience (section 16.10.6.5 in TS 38.300):

- A list of neighbour cells (mbs-NeighbourCellList ) in MCCH (MBSBroadcastConfiguration) that may provide the same MBS session(s) as in the serving cell. This allows a UE to request unicast reception before moving to a cell not providing the MBS session it is interested in.

- The neighbour frequencies that provide an MBS session. SIB21 contains MBS Frequency Selection Area ID (FSAI) which identifies a preconfigured area within, and in proximity to, which the cell(s) announces the FSAI and the associated frequency. This allows the UE to avoid the need to read MBS broadcast related system information and potentially MCCH on neighbour frequencies (see section 16.10.6.5.1 in TS 38.300). In addition, the UE is allowed to make the desired frequency highest priority in cell reselection.

### 2.1.2	NTN specific issues

In NTN, mobility is primarily driven by satellite movement. The duration for which an NGSO satellite serves a quasi-earth-fixed cell is typically limited to a few minutes. Each new serving satellite generally corresponds to a new cell. Therefore, an MBS NTN UE often performs mobility procedures such as cell reselection or handover and (re-)acquires MBS-related configuration, e.g., MCCH. This scenario poses challenges concerning service interruption, as the UE must first acquire MCCH to receive MBS service and may also result in increased UE energy consumption.

The dynamic nature of NGSO in NTN scenarios leads to frequent changes in serving cells, causing UEs to repeatedly acquire MBS-related configurations.

This repetitive re-acquisition of MBS configurations results in increased service interruption and UE energy consumption.

Additionally, MCCH operates with a defined modification period, with its contents permitted to change only at specific radio frames, i.e., at the modification period boundary. A notification mechanism is employed to announce updates to MCCH contents, which may occur due to several factors: the start, modification, or stop of a broadcast session, or changes to neighbouring cell information. The change notification is transmitted within the same modification period as the MCCH update, prompting the UE to initiate acquisition of the new MCCH without delay.

Figure 1. NTN cell change due to NGSO satellite mobility.

Figure 1 illustrates a representative NTN scenario in which a quasi-earth-fixed cell (cell A) is replaced as a consequence of NGSO satellite movement. The serving satellite (SatN) transitions out of coverage, and a new satellite (SatN+1) assumes service over the same geographical area, resulting in the establishment of a new cell (cell B). Between the time intervals T0 and T2, there exists a brief overlap period (T1) during which both satellites simultaneously illuminate the area. During this interval, the network needs to update the neighbour cell list within MCCH to include new cell (cell B). This change triggers MCCH update notification and forces the UE to re-acquire MCCH. In addition, once the UE has completed cell re-selection to the new cell (cell B), the UE may need to re-acquire once again MCCH in the new cell. Consequently, the UE may be required to acquire MCCH multiple times in response to every single satellite mobility event: first, when new neighbour cell information is incorporated into the MCCH at T1, and again upon reselection to the new cell at T2.

The dynamic nature of NGSO-based NTN scenarios leads to frequent changes in neighbour cells and, consequently, to repeated MCCH updates and reacquisitions.

The previous scenario presumes alignment of the MCCH modification period with the usually non-periodic satellite transitions; however, such alignment can be difficult to achieve in real-world deployments. Should the MCCH modification period fail to coincide with the timing of satellite changes, it becomes infeasible to provide timely updates to neighbour cell information within MCCH, thereby exacerbating the risk of service disruption.

The MCCH modification period limits the time instances when its contents can be updated increasing the risk of service interruption during the frequent change of NGSO satellites.

One approach to address the unnecessary acquisition of MCCH prior to cell re-selection is to provide the future neighbour cells that provide the same MBS service(s) as the serving cell. This is viable in NTN scenarios where cell transitions occur deterministically due to satellite movement or feeder link switching and the UE can be considered to remain static within the same area. Unfortunately, the current ASN.1 structure only allows the indication of existing/present cells, with no option to specify future neighbours. In this way, to adapt MBS service continuity to the most common mobility scenario in NTN, the neighbour cell list (mbs-NeighbourCellList) provided in MCCH could be extended to include identifiers of upcoming satellites or cells that will provide the same MBS session(s). This enhancement has a minor specification impact which is presented in a text proposal in section 3.

Broadcast future neighbour cells providing the same MBS service(s) as the serving cell.

In mobility events triggered by satellite mobility, we can assume that the MCCH content may remain consistent (across cell A and cell B from Figure 1), and the UE remains in the same area. A way forward to avoid MCCH re-acquisition after cell re-selection is to apply uniform MTCH settings across a defined geographical area, irrespective of the specific serving satellite or cell. The UE can be informed that MTCH configuration remains static upon satellite switch. An example is presented in Section 3. Thereby minimizing service interruptions and lowering UE power consumption, as frequent re-acquisition of information upon each cell change would no longer be necessary.

In MBS NTN, a UE is informed whether the cell replacing the serving cell provides an MBS service with identical MTCH configuration parameters.

# 3	Text Proposal

-- ASN1START

-- TAG-MBSBROADCASTCONFIGURATION-START

MBSBroadcastConfiguration-r17 ::= SEQUENCE {

criticalExtensions                CHOICE {

mbsBroadcastConfiguration-r17     MBSBroadcastConfiguration-r17-IEs,

criticalExtensionsFuture          SEQUENCE {}

}

}

MBSBroadcastConfiguration-r17-IEs ::= SEQUENCE {

mbs-SessionInfoList-r17               MBS-SessionInfoList-r17                                              OPTIONAL,   -- Need R

mbs-NeighbourCellList-r17             MBS-NeighbourCellList-r17                                            OPTIONAL,   -- Need S

drx-ConfigPTM-List-r17                SEQUENCE (SIZE (1..maxNrofDRX-ConfigPTM-r17)) OF DRX-ConfigPTM-r17   OPTIONAL,   -- Need R

pdsch-ConfigMTCH-r17                  PDSCH-ConfigBroadcast-r17                                            OPTIONAL,   -- Need S

mtch-SSB-MappingWindowList-r17        MTCH-SSB-MappingWindowList-r17                                       OPTIONAL,   -- Need R

lateNonCriticalExtension              OCTET STRING                                                         OPTIONAL,

nonCriticalExtension                  MBSBroadcastConfiguration-v19xy                                      OPTIONAL

}

MBSBroadcastConfiguration-v19xy-IEs ::= SEQUENCE {

mbs-SessionInfoList-v19xy             MBS-SessionInfoList-v19xy                                            OPTIONAL,   -- Need R

mbs-FutureNeighbourCellList-r19       MBS-NeighbourCellList-r17                                            OPTIONAL,   -- Need R

nonCriticalExtension                  SEQUENCE {}                                                          OPTIONAL

}

-- TAG-MBSBROADCASTCONFIGURATION-STOP

-- ASN1STOP

-- ASN1START

-- TAG-MBS-SESSIONINFOLIST-START

MBS-SessionInfoList-r17 ::=      SEQUENCE (SIZE (1..maxNrofMBS-Session-r17)) OF MBS-SessionInfo-r17

MBS-SessionInfoList-v19xy ::=    SEQUENCE (SIZE (1..maxNrofMBS-Session-r17)) OF MBS-SessionInfo-v19xy

MBS-SessionInfo-r17 ::=          SEQUENCE {

mbs-SessionId-r17                TMGI-r17,

g-RNTI-r17                       RNTI-Value,

mrb-ListBroadcast-r17            MRB-ListBroadcast-r17,

mtch-SchedulingInfo-r17          DRX-ConfigPTM-Index-r17                      OPTIONAL, -- Need S

mtch-NeighbourCell-r17           BIT STRING (SIZE(maxNeighCellMBS-r17))       OPTIONAL, -- Need S

pdsch-ConfigIndex-r17            PDSCH-ConfigIndex-r17                        OPTIONAL, -- Need S

mtch-SSB-MappingWindowIndex-r17  MTCH-SSB-MappingWindowIndex-r17              OPTIONAL  -- Cond MTCH-Mapping

}

MBS-SessionInfo-v19xy ::=        SEQUENCE {

mbs-SessionAreaList-r19          SEQUENCE (SIZE (1..maxNrofMBS-AreaPerSession-r19)) OF MBS-IntendedAreaID-r19 OPTIONAL, -- Need R

mtch-StaticConfigSwitch-r19      BOOLEAN                                                            OPTIONAL, -- Need R

...

}

DRX-ConfigPTM-Index-r17 ::=          INTEGER (0..maxNrofDRX-ConfigPTM-1-r17)

PDSCH-ConfigIndex-r17  ::=           INTEGER (0..maxNrofPDSCH-ConfigPTM-1-r17)

MTCH-SSB-MappingWindowIndex-r17  ::= INTEGER (0..maxNrofMTCH-SSB-MappingWindow-1-r17)

MRB-ListBroadcast-r17 ::=            SEQUENCE (SIZE (1..maxNrofMRB-Broadcast-r17)) OF MRB-InfoBroadcast-r17

MRB-InfoBroadcast-r17 ::=            SEQUENCE {

pdcp-Config-r17                      MRB-PDCP-ConfigBroadcast-r17,

rlc-Config-r17                       MRB-RLC-ConfigBroadcast-r17,

...

}

MRB-PDCP-ConfigBroadcast-r17 ::=     SEQUENCE {

pdcp-SN-SizeDL-r17                   ENUMERATED {len12bits}                   OPTIONAL, -- Need S

headerCompression-r17                CHOICE {

notUsed                              NULL,

rohc                                 SEQUENCE {

maxCID-r17                           INTEGER (1..16)               DEFAULT 15,

profiles-r17                         SEQUENCE {

profile0x0000-r17                    BOOLEAN,

profile0x0001-r17                    BOOLEAN,

profile0x0002-r17                    BOOLEAN

}

}

},

t-Reordering-r17                     ENUMERATED {ms1, ms10, ms40, ms160, ms500, ms1000, ms1250, ms2750}    OPTIONAL -- Need S

}

MRB-RLC-ConfigBroadcast-r17 ::=      SEQUENCE {

logicalChannelIdentity-r17           LogicalChannelIdentity,

sn-FieldLength-r17                   ENUMERATED {size6}                       OPTIONAL, -- Need S

t-Reassembly-r17                     T-Reassembly                             OPTIONAL  -- Need S

}

-- TAG-MBS-SESSIONINFOLIST-STOP

-- ASN1STOP

# 4	Conclusion

In the previous sections we made the following observations:

Observation 1	The dynamic nature of NGSO in NTN scenarios leads to frequent changes in serving cells, causing UEs to repeatedly acquire MBS-related configurations.

Observation 2	This repetitive re-acquisition of MBS configurations results in increased service interruption and UE energy consumption.

Observation 3	The dynamic nature of NGSO-based NTN scenarios leads to frequent changes in neighbour cells and, consequently, to repeated MCCH updates and reacquisitions.

Observation 4	The MCCH modification period limits the time instances when its contents can be updated increasing the risk of service interruption during the frequent change of NGSO satellites.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Broadcast future neighbour cells providing the same MBS service(s) as the serving cell.

Proposal 2	In MBS NTN, a UE is informed whether the cell replacing the serving cell provides an MBS service with identical MTCH configuration parameters.

# 5	References

RP-240775, “Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3” 3GPP TSG RAN#103, Mar. 2024.