3GPP TSG-RAN WG2 #130	R2-2505823

# Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	8.9.2

Source:	Ericsson

Title:	Support for store and forward in IoT NTN

Document for:	Discussion, Decision

# 1	Introduction

Based on the conclusions from SA2 study report [1], the following objectives were defined in RAN for store and forward in the related work item [2]:

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore:

  - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2]

    - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3]

In this contribution, we aim to address the last remaining issues such as the use of the already agreed indications (time information and barring bit), its extension to neighbour cells, the impact on AS procedures such as cell re-selection and the LS response from CT1.

# 2	Discussion

## 2.1	Operation mode of neighbour satellites

During the last RAN2 meeting, there was an extensive discussion regarding the inclusion in system information of the neighbour satellite operation mode. Two primary topics were addressed: the potential benefits to the UE and the impact on system information updates. Finally, a working assumption was taken as follows:

In the neighbour cell list we introduce an indication whether the cell operates in S&F mode or not (FFS if we also include the transition time). This WA can only be confirmed if we converge on the corresponding UE behaviour.

After the LS response from CT1 [3], it is clear that this information is not relevant to upper layers. Nonetheless, we believe that it can enhance the efficiency of various AS procedures, such as cell re-selection. For instance, a UE situated close to the border between a cell served by a satellite in S&F mode and a cell served by a satellite in non-S&F mode can benefit from prioritising reselection toward the non-S&F satellite cells. This approach effectively prevents ping-pong scenarios between cells of different operation modes, thereby reducing unnecessary context transfers between network entities (e.g., between ground-based and on-board MMEs) and limiting unnecessary UE signalling, such as repetitive NAS Attach procedures. Moreover, Release 19 UEs that do not support S&F operation but can read the signalling may utilise this information to steer away from S&F cells, thus avoiding reading SIB1 where the cell may be barred.

A UE may benefit from down-prioritizing cells served by satellite in S&F operation mode.

Regarding concerns about increased system information update frequency, it is important to note that modifications to SIB33-NB do not impact the value tag. Even in scenarios where the operation mode transition time is not specified, and the information may become outdated, UEs should reacquire SIB33-NB due to the validity of neighbour satellite ephemeris. Therefore, we consider there is little of increased signalling due to system information update notifications.

Confirm the working assumption: “In the neighbour cell list we introduce an indication whether the cell operates in S&F mode or not”.

From a procedural perspective, we think that a mechanism similar to the prioritisation used in MBS can be adopted. This is already present in the specification, and it would be simple to extend to S&F NTN. Specifically, when candidate cells have equal priority, the UE should apply a temporary offset in its reselection algorithm to deprioritise S&F neighbour cells, particularly when the serving cell operates in non-S&F mode. Note that, as defined in TS 36.304, UEs operating in enhanced coverage scenarios, which is a frequent case in NTN environments, always perform cell ranking regardless of configured frequency priorities.

The UE applies a temporary RSRP offset to S&F neighbour cells in the candidate list for cell re-selection.

## 2.2	Handling of operation mode transition

In previous meetings, RAN2 has discussed whether and how to deliver to the UE the operation mode of the satellite, and made the following agreements:

- The dynamic indication that “the cell is operating in S&F mode” is called “S&F operation” indication.

- “S&F operation” indication is provided in SIB1.

- If the “S&F operation” indication is not broadcast, a Rel-19 UE (regardless whether supporting S&F or not) shall follow the legacy barring procedure.

- When present, the “S&F operation” indication has two possible settings:

  - ‘1’: the cell is operating in S&F mode for all UEs (Rel-19 UEs supporting S&F are allowed to access the cell)

  - ‘0’: the cell is operating in S&F mode for UEs in Connected mode (which are not required to monitor the “S&F indication”), but idle Rel-19 UEs supporting S&F are barred (Rel-19 UEs not supporting S&F will follow legacy barring procedure).

During last meeting, the scenario in which a satellite transitions between S&F and non-S&F operation modes happens without an accompanying cell change was discussed. We believe it raises significant concerns. Specifically, if only the barring/operation mode bit is changed, current agreements may yield unintended consequences.

### 2.2.1	System information update upon operation mode transition

In NB-IoT systems, the fixed SIB1-NB modification period of 4096 radio frames (40.96 seconds) frequently exceeds the service time of NGSO satellites. As a result, updates to critical parameters such as cellBarred-NTN and the “S&F operation” indication cannot be synchronized in real-time with satellite operation mode transitions, leading to improper barring of UEs.

The modification period for SIB1-NB is fixed at 4096 radio frames (i.e., 40.96 seconds) which is longer than the NGSO satellite service time.

The barring bit and the “S&F operation” indication in SIB1-NB may not be updated timely following the change of satellite operation mode.

A straightforward potential solution is to mandate immediate SIB1-NB update and re-acquisition by the UE upon operation mode transition (i.e., after t-ModeSwitching), ensuring the UE can accurately determine barring status based on the updated SIB1-NB information. This approach enables timely and reliable access control, aligning cell status with actual satellite operation mode and preventing UEs from accessing or being barred inappropriately.

Regardless modification period boundaries, SIB1-NB is updated and re-acquired immediately after the transition of the satellite operation mode.

### 2.2.2	Cell access upon barring due to operation mode transition

According to TS 36.304, when a UE identifies a cell as barred, it must exclude that cell from the cell reselection candidate list for 300 seconds. Depending on the intraFreqReselection parameter configuration, the UE may extend this exclusion to the entire frequency. This mechanism, while robust, presents challenges in certain geographical areas where cell operation modes frequently transition. For instance, if a UE that does not support the S&F feature finds a cell operating in S&F mode that is about to transition to non-S&F, it is still required to exclude this cell (or the whole frequency) from the candidate list for the full 300 seconds. This time is in the order of the short service time of LEO satellites. Therefore, it may prevent the UE from reselecting the cell even after it becomes available (the barring bit is deactivated), resulting in missed connectivity opportunities.

A UE excludes a barred cell (or frequency) from the candidate list for cell re-selection for 300 seconds.

During the service time of a LEO satellite, a UE may not re-select to a cell previously determined as barred, even if the satellite changes operation mode and the barring bit is deactivated.

To address this limitation, we propose to introduce the operation mode transition time (t-ModeSwitching) of neighbouring satellites within the neighbour satellite information list in SIB33-NB. In this way, if a neighbouring cell is determined as barred due to S&F operation prior to its t-ModeSwitching, the UE would exclude this cell only until the specified operation mode transition time. Following the transition, the UE would reassess the barring status based on the updated system information. This solution minimizes unnecessary exclusion periods and maximizes the opportunity for UEs to connect when appropriate, ultimately improving network efficiency and user experience.

Introduce the operation mode transition time of neighbour satellites in the neighbour satellite information list in SIB33-NB.

The UE only excludes the barred neighbour cell due to S&F operation as a candidate for cell reselection until the operation mode transition time.

## 2.3	Cell re-selection and NAS monitoring list

In the previous meeting, the interplay between AS-layer UE behaviour, the NAS monitoring list, and the S&F Wait Timer was discussed. Some companies suggested that a UE could determine a detected cell as unsuitable and remove it from the reselection candidate list if the associated satellite is operating in S&F mode and it is not present in the monitoring list provided by NAS. However, these aspects were already clarified earlier in both working groups. An LS was sent to SA2 [4] to confirm the following understanding:

RAN2 would like to request SA2 to provide feedback on whether the following RAN2 understanding can be confirmed:

- RAN2 understanding: The UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list.

RAN2 received an LS response [5] which confirms RAN2’s understanding and further clarifies the informative intention with the S&F NAS wait timer and the satellite monitoring list [6]. In summary, the use of these mechanisms is scenario-dependent and, per SA2, is left to UE implementation. UEs are not prevented from attempting access to satellites outside the monitoring list; however, such action is not advisable, and it may lead to increased signalling and service delays, particularly if the satellite operates in S&F mode.

NOTE A:	The S&F Wait Timer or S&F Monitoring List doesn’t affect the UE when accessing an eNodeB that does not broadcast an indication of operating in S&F Mode.

NOTE B:	How UE behaves when receiving the S&F Monitoring List is up to UE implementation. When a UE receives a S&F Monitoring List and the UE access a satellite that supports Store and Forward Satellite operation that is not on the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure. The UE can continue to use the previously provided S&F Monitoring List, if the MME did not send one and the UE has previously been provided with one.

Therefore, SA2 and RAN2 are already aligned that the NAS monitoring list is informative in nature and should not impact AS-layer procedures such as cell reselection. We propose that RAN2 formally clarify this assumption to prevent further discussion in future meetings and guide UE and network implementations.

RAN2 understands that AS idle mode tasks related to NTN (e.g., cell (re)selection, paging monitoring) are not affected by the NAS satellite monitoring list and S&F Wait Timer.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	A UE may benefit from down-prioritizing cells served by satellite in S&F operation mode.

Observation 2	The modification period for SIB1-NB is fixed at 4096 radio frames (i.e., 40.96 seconds) which is longer than the NGSO satellite service time.

Observation 3	The barring bit and the “S&F operation” indication in SIB1-NB may not be updated timely following the change of satellite operation mode.

Observation 4	A UE excludes a barred cell (or frequency) from the candidate list for cell re-selection for 300 seconds.

Observation 5	During the service time of a LEO satellite, a UE may not re-select to a cell previously determined as barred, even if the satellite changes operation mode and the barring bit is deactivated.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Confirm the working assumption: “In the neighbour cell list we introduce an indication whether the cell operates in S&F mode or not”.

Proposal 2	The UE applies a temporary RSRP offset to S&F neighbour cells in the candidate list for cell re-selection.

Proposal 3	Regardless modification period boundaries, SIB1-NB is updated and re-acquired immediately after the transition of the satellite operation mode.

Proposal 4	Introduce the operation mode transition time of neighbour satellites in the neighbour satellite information list in SIB33-NB.

Proposal 5	The UE only excludes the barred neighbour cell due to S&F operation as a candidate for cell reselection until the operation mode transition time.

Proposal 6	RAN2 understands that AS idle mode tasks related to NTN (e.g., cell (re)selection, paging monitoring) are not affected by the NAS satellite monitoring list and S&F Wait Timer.

# 4	References

TR 3GPP TR 23.700-29 v19.0.0

RP-241624 - WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3

C1-254119, Reply LS on S&F mode indications to NAS, CT1#155, May 2025.

R2-2411197, LS on Satellite IDs for store-and-forward operation, CATT, RAN2#128, Orlando, USA, November 2024.

S2-2502450