3GPP TSG-RAN WG2 #130	R2-2505825

# Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	8.8.1

Source:	Ericsson

Title:	Remaining RRC open issues for NR NTN Rel-19

Document for:	Discussion, Decision

# 1	Introduction

The following document includes a list of open issues according to the following email discussion:

[Post130][302][R19 NR NTN] RRC CR (Ericsson)

Scope: discuss the running RRC CR

Intended outcome: Endorsed CR and list of remaining open issues

Deadline: long

Companies are invited to provide feedback on open issue list by: Aug 8th 08:00 UTC 2025

# 2	Remaining open issues for specification TS 38.331

### DL coverage enhancements

Configuration of 6 SMTC offsets (and periodicities)

According to the following RAN2 agreements, the network can configure in system information up to 6 SMTCs of which the UE will select a number based on its capabilities:

- We introduce support for SMTC enhancements in idle/inactive mode in a backwards compatible manner. FFS signalling details (e.g., new field or extension of SMTC4).

- Introduce a new SMTC list instead of extending legacy smtc4list (can consider signalling optimizations for the new list to refer to the content of smtc4list to avoid signalling duplications)

Provided the change should be introduced in a backwards compatible manner, i.e., the network may be able to configure SMTC4, the rapporteur has suggested two lists:

- The new SMTC5 list allows the network to configure from scratch periodicity and offsets.

- The extension of SMTC4 list allows the network to add an additional periodicity to an offset in the original SMTC4 list, which can override the default SMTC periodicity.

This proposal is better explained with a scenario. For example, in the scenario with two cells, one with SSB periodicity of 20 ms (cell 1) and another with 160ms (cell 2), the network should configure the default SMTC periodicity with the largest periodicity, 160 ms, following RAN4 requirements. In addition, the new SMTC4 extended list allows the network to indicate Release 19 UE that, in reality, there is an offset (corresponding to cell 1) that has a different periodicity than the default SMTC periodicity. The periodicity in the extended SMTC4 list overrides the default periodicity. In this way, a legacy UE will measure cell 1 every 160ms since it will not understand the new signalling, but a Release 19 UE can effectively measure cell 1 every 20 ms.

smtc4list-v19xy                     SSB-MTC4List-v19xy                              OPTIONAL,        -- Need R

smtc5list-r19                       SSB-MTC5List-r19                                OPTIONAL,        -- Need R

SSB-MTC4-v19xy ::=           SEQUENCE {

periodicity-r19              ENUMERATED {sf10, sf20, sf40, sf80, sf160}

}

SSB-MTC5-r19 ::=             SEQUENCE {

pci-List-r19                 SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                          OPTIONAL,  -- Need M

periodicity-r19              ENUMERATED {sf10, sf20, sf40, sf80, sf160}

offset-r19                   INTEGER (0..159)

Some companies have proposed another alternative using delta configuration. The rapporteur notes that the delta is only needed for those STMCs which are defined in the SMTC4 list which has a limited size of 3. If we disregarding the SMTC for the serving cell (default SMTC), then the other new 2 SMTCs can be in a new list as already agreed. In conclusion, provided the confirmation of multiple periodicties per frequency layer requires RAN2 discussion of the RAN4 LS, the rapporteur encourages companies to take this implementation as a baseline and bring concrete Text Proposals to the next meeting.

FFS Stage 3 impact of the limit of maximum 2 SMTC periodicities per frequency layer and the configuration of 6 SMTC offsets per frequency. Companies are invited to bring concrete text proposals.

Clarification on the size of the lists

The rapporteur included different sizes for the extension of SMTC4 list and the new SMTC5 list and this has generated confusion. Here is an explanation. RAN2 agreed to support the configuration of 6 SMTCs in total. If we consider only intra-frequency neighbours, the first SMTC corresponds always to the serving cell and should be captured with the Rel-15 SMTC parameter. In Release 17, 3 additional SMTCs were included for NTN within STMC4 list. Therefore, the size of SMTC5 list depends on the configuration of SMTC4 list, i.e., whether the network wants to support legacy UEs. Provided Rel-15 SMTC is configured, if SMTC4 list is not configured, the network can configure up to 5 additional SMTCs. If SMTC4 list is configured, then the size of SMTC5 list is equal to 5 minus the size of SMTC4. In this way, the total number of configured SMTCs never exceeds 6.

SSB-MTC4List-v19xy::=               SEQUENCE (SIZE(1..3)) OF SSB-MTC4-v19xy

SSB-MTC5List-r19::=                 SEQUENCE (SIZE(1..5)) OF SSB-MTC5-r19

Question 1: companies are invited to comment whether SMTC5 list can be configured independently, thus having a maximum size of 5, or always need to be configured with SMTC4 list and SMTC4 extension list, thus having a maximum size of 2.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | See comments | Lets forget periodicity now. Old UEs use SMTC4 list. New UEs use SMTC5 list. So from index 1 to 3, the pci-List and offset values are repeated (same) in both SMTC4 and SMTC5 (it must be same). But we agreed optimization with delta signaling could be considered. This means the pci-List and offset in SMTC5 can be optional. If absent for index 1 to 3, the new UEs use the pci-List and offset from SMTC4. |
| ZTE | See comments | In our understanding smtc5 list is independently configured from smtc4 list. legacy UEs only read smtc4 list while R19 UEs supporting extended SMTC number will follow SMTC5list configuration. |
| vivo |  | A new SMTC5 list of max size 5 should be adopted, as per agreement that “To support SMTC enhancements for DL CE UE, introduce a new SMTC list instead of extending legacy smtc4list”. The SMTC5 list is independently configured. |
| CMCC | See comments | Agree to configure SMTC5 list independently. Based on the 129bis agreement, legacy UE will not be barred from DL CE cell, so NW need to configure SMTC for both legacy UE and R19 DL CE capable UE. From the perspective of minimizing the impact on legacy UE and providing more flexibility, it is preferable to introduce new SMTC5 list. |
| Ericsson |  | We agree that SMTC5 should be possible to configure independently. |

SMTC5 list can be configured independently. Delta signalling with SMTC4 list can be supported to avoid unnecessary overhead. (5/5)

In this regard, the rapporteur proposed a maximum size of 5 for the reference location list with the understanding that Rel-15 SMTC refers to the serving cell, which does not need location-based SMTC. Provided the comments from different companies, RAN2 should clarify whether the agreement of 6 configured SMTCs include the serving cell and whether the Rel-15 SMTC parameter is subject to location-based SMTC selection.

ReferenceLocationList-r19 ::= SEQUENCE (SIZE(1..5)) OF ReferenceLocation

Question 2: do the agreed 6 configured SMTCs include the serving cell SMTC, usually captured with the Rel-15 SMTC parameter.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes |  |
| Qualcomm | Yes |  |
| ZTE | See comments | Smtc list in SIB2/4 is only for neighbor cell measurements, UE follows SSB pattern to measure serving cell. But we agree that smtc5 contains 5 entries, together with smtc in SIB2/4, the maximum number SMTCs is 6. |
| vivo | Yes |  |
| CMCC | Yes |  |

The maximum number of 6 configured SMTCs include the SMTC of the serving cell (5/5)

Multiple periodicities for inter-frequency neighbour cells

In RAN2#129, RAN2 assumed that “it will be possible to have different SSB periodicity among neighbour cells in the same frequency layer”. In the rapporteur’s understanding, based on previous discussions and baseline scenarios considered, multiple SMTCs offsets are not needed for inter-frequency neighbors. For example, RAN2 also made the following agreement in RAN2#129: “RAN2 observes that if the cells active simultaneously are scattered, (for the case of intra-frequency neighbouring cells), the SSBs of surrounding neighbour cells may be transmitted at different times and existing SMTC mechanism may not be enough”. However, some companies have commented that multiple periodicities should be also added in SIB4. Therefore, companies are invited to address the FFS whether multiple SMTC periodicies are needed in frequency layers different from the serving cell frequency (e.g., inter-frequency neighbours) in a contribution for next meeting clarifying the scenarios where this enhancement would be beneficial.

FFS whether multiple SMTC periodicities are needed in frequency layers different from the serving cell frequency (e.g., inter-frequency neighbours) and clarify in which inter-frequency scenario(s) this enhancement is beneficial.

SMTC enhancements in connected mode

Following last meeting discussions, it is still RAN2 confirmation the working assumption: “We introduce a mechanism to assist the NW to configure the SMTCs in connected mode:

Alt 1: UE provide the closest N reference locations/neighbor cells to network.

Alt 2: UE reports the selected SMTCs from configured SMTC set to network.

Companies are invited to bring their analysis whether to confirm the working assumption (and decide between Alt1 or Alt2) comparing the alternatives with aspects such as signaling design, energy consumption, etc. or whether the current specification is sufficient and RAN2 does not introduce any enhancement.

FFS on the support of location-based SMTC selection in RRC_CONNECTED and the choice between Alt 1 or Alt2.

### Intended Service Area for MBS NTN

Service continuity

RAN2 has agreed that a UE may prioritize the frequency(ies) for an interested service when UE can only receive the service on those frequency(ies) and the UE is in intended service area associated with the service provided in the frequency(ies). In addition, RAN2 agreed last meeting that “do not introduce service area information specific for FSAI in SIB2” and “do not introduce service area per neighbour cell in MCCH”. Provided the Service Announcement will carry the Intended Service Area definition, companies are invited to answer the following question regarding the association between intended service area and frequencies for frequency prioritization in RRC_IDLE.

Question 3: whether an explicit association between ISA and frequencies for frequency prioritization is required in AS layer.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | No | It is sufficient with the Service Announcement for the UE to make the connection between FSA ID and ISA. |
| Qualcomm |  | This option should be possible to do for network, may be leave this to OAM how neighbor cell MBS frequency ISA is obtained. |
| vivo | No | We think the intentsion of having ISA in USD is only for MCCH reading optimization. For frequency, the UE do the prioritization based on SIBxx. |
| CMCC | No | Existing mechanism is sufficient. |

Existing mechanisms are sufficient to associate the ISA with relevant neighbour frequencies to perform MBS frequency prioritization (3/4).

Signalling details

RAN2 and SA4 have chosen different representations of the Intended Service Area in System Information and Service Announcement, respectively. SA4 has specified that the ISA within the Service Announcement should be represented using either the Polygon or PointUncertaintyCircle formats as defined in TS 29.572 [S4-251482]. In parallel, RAN2 has adopted a circular representation based on referenceLocation and distanceRadius directly specified in RRC, alongside the Polygon shape defined in TS 23.032 and TS 37.355.

Given that both approaches convey equivalent geospatial information, it is better to harmonize the specification by adopting the Ellipsoid-PointWithUncertaintyCircle, as defined in TS 37.355, for the circular ISA representation.

There is a small difference in the range of values supported. The current distanceRadius is a 16-bit field supporting values between 0 and 3000 kilometres. In contrast, the Ellipsoid-PointWithUncertaintyCircle has an uncertainty of 7 bits with a suitably useful range between 0 and 1800 kilometres. According to TS 23.032, this uncertainty is expressed in metres and follows this formula (K=127 since 7 bits are used for the encoding):

Provided the nature of the MBS enhancement, the rapporteur understands that the difference is not significant (Ellipsoid-PointWithUncertaintyCircle consumes 9 bits less) in comparison to maintain the specifications clean and aligned across working groups. An illustrative ASN.1 definition for this proposal is provided below.

SIBXX information element

-- ASN1START

-- TAG-SIBXX-START

SIBXX-r19 ::=                     SEQUENCE {

intendedServiceAreaList-r19     IntendedServiceAreaList-r19      OPTIONAL,  -- Need R

lateNonCriticalExtension        OCTET STRING                     OPTIONAL,

...

}

IntendedServiceAreaList-r19 ::=   SEQUENCE (SIZE (1.. maxNrofMBS-Area-r19)) OF IntendedServiceAreaInfo-r19

IntendedServiceAreaInfo-r19 ::=   SEQUENCE {

intendedServiceAreaId-r19       MBS-IntendedAreaID-r19,

areaCoordinates-r19             CHOICE {

polygonArea-r19                   OCTET STRING,

circleArea-r19                    OCTET STRING

}

}

MBS-IntendedAreaID-r19  ::= INTEGER (0.. maxNrofMBS-Area-r19)

-- TAG-SIBXX-STOP

-- ASN1STOP

SIBXX field descriptions

intendedServiceAreaList Contains a list of intended service areas, each associated to one or more MBS broadcast services provided in an NTN cell.

polygonArea Contains the Polygon IE as defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

circleArea Contains the Ellipsoid-PointWithUncertaintyCircle IE as defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

Question 4: Whether RAN2 should use the Ellipsoid Point with uncertainty Circle from TS 37.355 to define the circle shape of the ISA in the new SIBXX.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes | We consider it is important to harmonize the specifications to avoid implementation issues. |
| vivo | No | RAN-level SIBxx is likely to provide more accurate area information with finer configuration parameters, based on the network service deployment. |
| CMCC | No | Agree with vivo, RAN could provide more accurate information. |

Provided the diverging views on the adoption of the IE and the accuracy at RAN and CN levels, the rapporteur suggests continuing the discussion next meeting to clarify the understanding regarding accuracy and compare the two alternatives in next meeting contributions.

FFS to align RAN2 specifications and use the Ellipsoid Point with uncertainty Circle from TS 37.355 to define the circle shape of the ISA in the new SIBXX.

# 3	Other identified open issues

Companies are invited to describe any other identified open issues not currently included within this document

| Company | Other identified open issues? (please describe) |
| --- | --- |
| Ericsson | MBS Service continuity due to satellite movement. In NTN NGSO, satellites will change often. This will trigger MCCH updates due to new neighbor cells being added. Consequently, UE will need to acquire MCCH multiple times, first when the neighbour list is updated, then, we the UE performs cell reselection. [Qualcomm] When satellite is changed, UE anyway would have to reacquire the SIB19 for satellite information, why it is additional burden only for MBS? [ZTE: Agree with QC. When satellite changes UE anyway will need to perform cell reselection to find a suitable cell to camp on. Moreover, UE can use ISA to decide whether to skip acquiring MCCH. ] [Ericsson] The reasoning behind this enhancement is to reduce service interruption due to satellite mobility which will happen every few minutes and certainly will disrupt user experience. |
| vivo | Whether ISA in USD is considered for MRB estalishement/release/frequency prioritization (i.e. how to use the ISA in USD from RAN2 perspective)? |
| CMCC | Term ‘broadcast service area’ vs. ‘intended service area’ misalignment issue, which is raised in last meeting and needs to comeback in this meeting. And this may have impact on the MII section description. |

# 4	Conclusions

The following proposals have been provided based on feedback to the above document:

[Proposals for easy agreement]

SMTC5 list can be configured independently. Delta signalling with SMTC4 list can be supported to avoid unnecessary overhead (5/5)

The maximum number of 6 configured SMTCs include the SMTC of the serving cell (5/5)

Existing mechanisms are sufficient to associated the ISA with relevant neighbour frequencies to perform MBS frequency prioritization (3/4).

[Proposals for further discussion]

FFS Stage 3 impact of the limit of maximum 2 SMTC periodicities per frequency layer and the configuration of 6 SMTC offsets per frequency. Companies are invited to bring concrete text proposals.

FFS whether multiple SMTC periodicities are needed in frequency layers different from the serving cell frequency (e.g., inter-frequency neighbours) and clarify in which inter-frequency scenario(s) this enhancement is beneficial.

SMTC enhancements in connected mode for NTN DL coverage enhancements are pending RAN2 confirmation of the working assumption: “We introduce a mechanism to assist the NW to configure the SMTCs in connected mode”. It is FFS the choice between Alt 1 or Alt2.

FFS to align RAN2 specifications and use the Ellipsoid Point with uncertainty Circle from TS 37.355 to define the circle shape of the ISA in the new SIBXX.

FFS on the terminology misalignment between ‘broadcast service area’ vs. ‘intended service area’ and the potential impact on the MII procedure.