3GPP TSG RAN WG2 Meeting #131                          	   R2-2505845

Bengaluru, India, August 25th - 29th, 2025

Agenda item:	8.5.2

Source:	Qualcomm Incorporated

Title:	Discussion on remaining issues of on-demand SSB

WID/SID:	Netw_Energy_NR_enh-Core – Release 19

Document for:	Discussion

# Introduction

The objectives of on-demand SSB SCell are outlined with the following [1].

The objectives of the work item are the following: - Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

In this contribution, we discuss the remaining design issues of OD-SSB transmission signaling.

# Discussion

At RAN2 #127, the following agreements were made for signaling OD-SSB transmission [2].

Agreements on OD-SSB OD-SSB transmission indication: 2. RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration. 3. New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

At RAN2 #127bis, the following agreement was made for RRC based signaling [3].

Agreements on OD-SSB SCell

- No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

At RAN2 #129bis, the following agreement was made for MAC CE based signaling [4].

Agreements on OD-SSB

- For explicit activation/deactivation, the OD-SSB MAC-CE includes fixed sized bitmap to indicate whether OD-SSB is activated in each SCell (i.e., similar to legacy SCell A/D MAC-CE).

- A/D bit: “1” means activation, “0” means deactivation for explicit deactivation case.

- For explicit activation/deactivation, the OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells.

- OD-SSB MAC-CE includes a configuration index for each SCell activating OD-SSB.

- RAN2 aims to define single MAC CE format for both explicit and implicit OD-SSB deactivations. Details are FFS.

Observation 1. Both RRC configuration or reconfiguration message and MAC CE can be used for OD-SSB transmission indication.

At RAN1 #121, the following UE capabilities were agreed for OD-SSB [5].

Observation 2: A UE may support the capabilities of RRC indication, MAC CE indication, or both.

For a UE supporting both RRC signaling and MAC CE signaling, the UE needs to know if the RRC signaling or MAC CE signaling is used for SCell’s OD-SSB transmissions.

Observation 3:  For a UE supporting both RRC indication and MAC CE indication, the UE needs to know whether to take the RRC message or the MAC CE as the indication for OD-SSB.

Based on the agreements and observations, we propose the following proposals for UE’s behaviors with the least spec impact.

Proposal 1. With only one OD-SSB configuration in an RRC configuration message or RRC reconfiguration message, a UE takes the RRC message as the indication for OD-SSB activation or update.

Proposal 2. With more than one OD-SSB configuration in an RRC configuration message or RRC reconfiguration message, a UE takes the MAC CE including an OD-SSB configuration index as the indication for OD-SSB transmission or update.

Proposal 3. Do not support mixed RRC indication and MAC CE indication for OD-SSB transmission activation or update.

# Conclusion

In this contribution, we discussed the remaining design issues of OD-SSB transmission signaling and concluded with the following observations and proposals.

Observation 1. Both RRC configuration or reconfiguration message and MAC CE can be used for OD-SSB transmission indication.

Observation 2: A UE may support the capabilities of RRC indication, MAC CE indication, or both.

Observation 3:  For a UE supporting both RRC indication and MAC CE indication, the UE needs to know whether to take the RRC message or the MAC CE as the indication for OD-SSB.

Proposal 1. With only one OD-SSB configuration in an RRC configuration message or RRC reconfiguration message, a UE takes the RRC message as the indication for OD-SSB activation or update.

Proposal 2. With more than one OD-SSB configuration in an RRC configuration message or RRC reconfiguration message, a UE takes the MAC CE including an OD-SSB configuration index as the indication for OD-SSB transmission or update.

Proposal 3. Do not support mixed RRC indication and MAC CE indication for OD-SSB transmission activation or update.

# References

- RP-242354 “Revised WID: Enhancements of network energy savings for NR”, RAN Meeting #105, Melbourne, Australia, Sep 9-12, 2024.

- R2-2407571 “Report from session on V2X/SL, R18/19 MOB, and R19 NES” RAN2 #127, Maastricht, NL, August 19th – 23rd, 2024.

- “Report from session on V2X/SL, R18/19 MOB, and R19 NES” RAN2 #127bis, Hefei, China, October 14th – 18th, 2024

- “R2_129bis_R18 SL_R18R19 MOB_R19 NES_Session Notes_EoM”, RAN2 #129bis, Wuhan, China, Apr. 7th – 11th, 2025

- R1-2504927 “Session Notes of AI 9.15.4	Ad-Hoc Chair (AT&T)”, RAN1 #121	, St Julian’s, Malta, May 19th – 23rd, 2025