3GPP TSG-RAN WG2 Meeting #131	R2-2505873

Bangalore, India, August 25th – 29th, 2025

Agenda Item:	8.1.3

Source:	InterDigital Inc.

Title:	Remaining issues for NW side data collection

Document for:	Discussion

# 1. Introduction

Several agreements regarding network side data collection were made in previous RAN2 meetings [1]-[7]. The Annex lists the agreements regarding network side data collection during these meetings.

In this contribution, we address some of the open issues regarding network side data collection.

# 2. Discussion

## 2.1 Availability indication and reporting of collected data

This is related to open issues RRC-19 and RRC-39 in [8]:

Open issue RRC-19: Reporting assistance information related to logged measurements

Issue description: It is not yet clear what the otherConfig should contain to enable the UE to report assistance information via UAI, related to logging of radio measurements. For instance, should the low power, buffer full, and buffer threshold reached indications be all configured with a single bit, or should the configurations be separated?

Open issue RRC-39: The naming of IEs related to NW-side data collection

Issue description: The naming of IEs related to NW-side data collection can be revisited. We will have more CSI-based and other use cases, each with different measurement logs and we should minimize the risk of having to replicate the NW-side DC procedures and ASN.1 for every new use case.

The following agreements were made regarding availability indication the sending of collected data:

RAN2-127bis [3]:

UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection.

RAN2-129 [5]:

UE indicates availability of logged data during handover (i.e., within the RRCReconfigurationComplete message) (if data is retained in the UE).

RAN2-129bis [6]:

- The UE send a UAI that indicates:

  - Data is available

  - Reason for trigger (full buffer, threshold)

  - Low power indication

In RAN2-130 [7]:

- Data is collected on per data logging configuration basis and UE indicates data logging configuration ID. An indication of the “gap” is needed.  “Gap” is time interval larger than the configured logging periodicity.  FFS if timestamp and relative time stamp for each group is needed per “group”.

Considering the above agreements, our understanding is that the indication of availability of collected data and the request for a collected data should indicate the configuration ID of the data collection (as the UE may be configured with multiple data collection configurations). Thus, we propose the coding in the running CR [9] to be modified to:

Proposal 1: The data collection configuration ID is included in the UAI and RRCReconfigurationComplete messages.

Proposal 2: The data collection configuration ID is included in the UEInformationRequest message to indicate the network is requesting the UE to send the logged data according to the indicated configuration.

Since the UE will not send the collected data autonomously without the network requesting it, there is no need to indicate the configuration ID in the UEInformationResponse.

Proposal 3: There is no need to indicate the data collection configuration ID in the UEInformationResponse.

## 2.2 Time related content of logged data

This is related to open issues RRC-21 in [8]:

Open issue RRC-21: Time related content of logged data

Issue description: It has not yet been clarified what information needs to be included with the logged data, to indicate a time gap between the logged data entries (i.e. a gap that is longer than the logging data periodicity).

This issue refers to the RAN2#129bis agreement:

- For temporal domain, the network is made aware whether there is a gap between two consecutive samples.   FFS amount of gap and whether this is implicit or explicit

RAN2#130 agreement:

Data is collected on per data logging configuration basis and UE indicates data logging configuration ID. An indication of the “gap” is needed. “Gap” is time interval larger than the configured logging periodicity. FFS if timestamp and relative time stamp for each group is needed per “group”.

NR supports logging of time information in the logged MDT case, as shown below:

LogMeasReport-r16 ::=                SEQUENCE {

absoluteTimeStamp-r16                AbsoluteTimeInfo-r16,

traceReference-r16                   TraceReference-r16,

traceRecordingSessionRef-r16         OCTET STRING (SIZE (2)),

tce-Id-r16                           OCTET STRING (SIZE (1)),

logMeasInfoList-r16                  LogMeasInfoList-r16,

logMeasAvailable-r16                 ENUMERATED {true}                   OPTIONAL,

logMeasAvailableBT-r16               ENUMERATED {true}                   OPTIONAL,

logMeasAvailableWLAN-r16             ENUMERATED {true}                   OPTIONAL,

...

}

LogMeasInfoList-r16 ::=              SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF LogMeasInfo-r16

LogMeasInfo-r16 ::=                  SEQUENCE {

locationInfo-r16                     LocationInfo-r16                    OPTIONAL,

relativeTimeStamp-r16                INTEGER (0..7200),

servCellIdentity-r16                 CGI-Info-Logging-r16                OPTIONAL,

measResultServingCell-r16            MeasResultServingCell-r16           OPTIONAL,

measResultNeighCells-r16             SEQUENCE {

measResultNeighCellListNR            MeasResultListLogging2NR-r16    OPTIONAL,

measResultNeighCellListEUTRA         MeasResultList2EUTRA-r16        OPTIONAL

},

anyCellSelectionDetected-r16         ENUMERATED {true}                   OPTIONAL,

...,

[[

inDeviceCoexDetected-r17             ENUMERATED {true}                   OPTIONAL

]]

}

As can be seen above, there is one absolute time information corresponding to the absolute time when the logged measurement configuration was provided, and for each collected sample, there is a relative timestamp from this absolute timestamp.

If we adopt a similar strategy as logged MDT for including the absolute timestamp for the start of the logging, and by default the gap between two entries is the logging periodicity, then the gap information for each group whenever there is a gap doesn’t have to be an absolute time stamp. Also, instead of a timestamp, the gap can be specified in terms of the number of “missing” samples (i.e., samples not logged due to the event for logging being not fulfilled during that period). Thus, we propose:

Proposal 4: Choose between the following two options for indicating the length of a gap in the samples of logged measurements (due to logging event not being fulfilled):

- Add a relative timestamp that is equal to the time interval between the last sample of the previous group and the first sample of the new group

- Add a relative sample gap that corresponds to the number of missing (i.e., not logged) samples between the last sample of the previous group and the first sample of the new group

## 2.3 Data logging configuration

The support of L3 event for controlling the logging of L1 based information causes an issue, especially in the case of CU-DU split architecture, as the CSI-RS configurations are handled by the DU and L3 measurements are handled by the CU.

Two ways of resolving the issue were proposed:

- solution 1: L1 CSI framework enhanced to include logging information and information/linkage about L3 event [10]

- solution 2: L3 measurement framework enhanced to include CSI-RS logging information [11]

An email discussion was carried out regarding these two approaches and there was no agreement on which solution to adopt (though more companies preferred solution 2) [12].

Our understanding is that solution 1 is going against the functional/protocol split of the CU-DU architecture, as it requires the DU to set the parameters for the L3 event (e.g., A1/A2 threshold, TTT) and modify the CSI-MeasConfig or CSI-Report.

Solution 2 requires some F1-AP enhancements as currently the CU transparently encapsulates the CSI-MeasConfig/CSI-Report configurations generated by the DU in an RRC message and transports it (transparently) to the UE. That is, CU has now to be enabled to know the CSI configuration is for logging purposes and then associate it with a L3 measurement event.

It is desirable to have a solution that is future proof. For example, we would like to have a solution that can be easily reused for the RRM measurement prediction and measurement event prediction use cases of AI/ML mobility, where logging L3 measurements is required (along with L1 measurements, depending on the RRM sub-use case). Solution 2 is more flexible to support such cases, as all that is needed will be now be to associate the L3 logging/measurement configuration with the L3 event (the same way the L1 logging/resource configuration is done for the BM case), while for solution 1 we have to redesign a new way for a L3 event to control the logging of L3 measurements.

Observation 1:	Data collection configuration based on the L3 measurement framework provides a future proof design that is easily extendable for other use cases (e.g., AIML mobility).

We observe a couple of issues the proposed TP for solution 2 in [11].

In [11], an approach similar to legacy measurement report configuration is used, wherein the logging can be periodic, or event based

loggingType-r19   CHOICE {

eventTriggerdLogging-r19	FFS,

Periodic				BOOLEAN

},

In our understanding, this choice structure is not needed as the UE doesn’t need to choose periodic logging versus event-based logging but rather log periodically whenever the event is fulfilled.

Another issue is that even though we are designing this for the BM use case, it would be good to define the framework in such a way that we don’t have to duplicate the whole structure for each new use case (i.e.,  have a generic DataLoggingConfig that can be used for BM or other use cases, wherein IEs within this will be linked to the particular measurements that will be logged).

Thus, we propose:

Proposal 5: The network side data collection configuration to be based on L3 measurement framework proposed in [11], with the following modifications:

- use a generic DataLoggingConfig IE (instead of the proposed BM-DataLoggingConfig) that can be used for any use case, wherein IEs within this will be linked to the measurements to be logged for each use case

- remove the CHOICE structure for periodic vs event-based logging, and instead have an optional event configuration for controlling the logging

# 4. Conclusion

In this contribution, we discussed some of the open issues regarding data collection for network sided model for the BM use case and the following observations and proposals were made:

Observation 1:	Data collection configuration based on the L3 measurement framework provides a future proof design that is easily extendable for other use cases (e.g., AIML mobility).

Proposal 1: The data collection configuration ID is included in the UAI and RRCReconfigurationComplete messages.

Proposal 2: The data collection configuration ID is included in the UEInformationRequest message to indicate the network is requesting the UE to send the logged data according to the indicated configuration.

Proposal 3: There is no need to indicate the data collection configuration ID in the UEInformationResponse.

Proposal 4: Choose between the following two options for indicating the length of a gap in the samples of logged measurements (due to logging event not being fulfilled):

- Add a relative timestamp that is equal to the time interval between the last sample of the previous group and the first sample of the new group

- Add a relative sample gap that corresponds to the number of missing (i.e., not logged) samples between the last sample of the previous group and the first sample of the new group

Proposal 5: The network side data collection configuration to be based on L3 measurement framework proposed in [11], with the following modifications:

- use a generic DataLoggingConfig IE (instead of the proposed BM-DataLoggingConfig) that can be used for any use case, wherein IEs within this will be linked to the measurements to be logged for each use case

- remove the CHOICE structure for periodic vs event-based logging, and instead have an optional event configuration for controlling the logging

# 5. References

[1]	RAN2-126 Chairman Notes, Fukuoka, Japan, May 2024

[2]	RAN2-127 Chairman Notes, Maastricht, The Netherlands, August 2024

[3]	RAN2-127bis Chairman Notes, Hefei, China, October 2024

[4]	RAN2-128 Chairman Notes, Orlando, USA, November 2024

[5]	RAN2-129 Chairman Notes, Athens, Greece, February 2025

[6]	RAN2-129bis Chairman Notes, Wuhan, China, April 2025

[7]	RAN2-130 Chairman Notes, Malta, May 2025

[8]	R2-2505778, Remaining RRC open issues in feature AIML PHY, Ericsson, RAN2-131, Bangalore, India, August 2025.

[9]	R2-2504349, Draft Running RRC for AIML PHY, Ericsson, RAN2-129, Malta, May 2025.

[10]	R2-2504644, Ericsson, Nokia, Huawei, T-Mobile USA, BT Plc., “Discussion on NW-side data collection framework”, 3GPP TSG-RAN WG2 #130, Malta, May 2025.

[11]	R2-2503849, ZTE Corporation, Apple, MediaTek, Samsung, OPPO, Lenovo, Xiaomi, CMCC, China Telecom, vivo, NTT DOCOMO, Sanechips, “Discussion On the NW Side Data Collection RRC Framework”, 3GPP TSG RAN2 Meeting #130, Malta, May 2025.

[12]	R2-2505859, Report of email discussion [POST130][031][AI PHY] NW side data collection, Ericsson/ZTE, RAN2-131, Bangalore, India, August 2025.

# 6. Annex (Agreements regarding network side data collection)

RAN2-126 agreements [1]:

Agreements for beam management

1. 	For gNB centric and OAM centric (for RRC signaling between UE and gNB), reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature. FFS how to handle case when single RRC message is not sufficient. FFS if there will be any further enhancement needed pending RAN1 agreement.

2.	Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model

3.	Enhance the immediate MDT framework to support periodical reporting. FFS whether and what event-based reporting is supported and FFS on network request reporting.

RAN2-127 agreements [2]:

Agreements

- As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements.  The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started.  FFS if multiple configurations can be provided to the UE.  FFS if dynamic activation/deactivation is support.

- UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size.  This is across all use cases

- When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging.

- Measurements for data collection purposes and logging based can be controlled based on power state of the UE.  It is up to UE implementation how the UE determines power state.  FFS whether the UE stops autonomously or if it reports to the network.

- FFS whether AS buffer event based reporting is supported.  FFS if we send availability indication or full report if it is supported

- FFS on event-based data collection/logging

- On-demand request from the network is supported.   FFS details on signalling

Agreements

- The UE implementation can determine how many entries to include in the list radio measurements information, such that the maximum PDCP SDU size is not exceeded. No standardized RRC segmentation procedure is needed (as for the logged MDT measurements)

- Data collection report will not be transmitted over SRB1.  FFS which SRB is used.

RAN2-127bis agreements [3]:

Agreements on NW side data collection

- Periodic logging is supported for training data collection procedure in R19

- Event-triggered data logging will be supported.  At least radio condition based event triggered logging will be supported.  FFS the details of radio condition based event.  FFS if other events are supported.

- Periodic reporting of logged data is not supported.

- On-demand reporting of the logged measurements will be specified

- UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection.   FFS of details of the message

- The UE can indicate the availability of logged data to the network to assist network to trigger UEInformationRequest.  FFS trigger/definition of availability indication.   and FFS how data availability indication is sent to the network.

- Low priority SRB will be used.    FFS new SRB or use of SRB4

- For data collection for both NW-sided/UE sided BM model training, at least L1-RSRPs and/or beam-IDs needs to be collected by UE.  FFS if other data needs to be collected based on RAN1 progress.

RAN2-128 agreements [4]:

Agreements on NW side data collection: - Focus on the following three radio condition event-based logging: - L3 serving cell measurement based (e.g. X1/X2 similar to A1/A2) - Beam based events (e.g. beam becomes top-1 beam and number of measurements is less than configured value) - L1 beam level measurement - Measurements on aperiodic CSI resources are not reported for NW sided data collection. - Data collection is controlled by the network. The UE will not autonomously stop when low power state is detected. - The UE reports to the network when the power state is low. We will not specify how the UE determines low power state. The network should de-configure the data collection (this can be captured in stage 2). - The UE reports to the network when buffer is or may become full. FFS when it reports (before and/or after). - The UE can report the reason for triggering of indication for the status (e.g. low power state, low memory). FFS how this is signalled and if the reporting can be part of availability indication.

RAN2-129 agreements [5]:

Agreements on NW side data collection: - Support the use of L3 measurement event triggered (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE performs logging or not. L1 measurement event triggered will not be supported.FFS what to log - Low power bit indication is supported - Data availability indication is supported. FFS when this would be triggered - As baseline, the UEInformationResponse contains one or more logged measurement entries in chronological order (i.e. starting from the oldest measurement entries stored in the UE memory), and an availability indication if there are further data available for transmission. Same principles as for logged MDT. - UE retains logged data during handover (HO). FFS if there is scenarios where the UE needs to release the data and how does the UE know and if control from network is needed - UE indicates availability of logged data during handover (i.e., within the RRCReconfigurationComplete message) (if data is retained in the UE). - FFS how to handle idle/inactive and RLF cases and whether we have a unified.

RAN2-129bis agreements [6]:

Agreements on NW side data collection: - Introduce 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration during/before HO. Source gNB decides whether the data should be kept. The indication is provided in RRCReconfiguration (i.e. not in RRC Reconfiguration from target cell). FFS signaling details. - Upon going to RRC_IDLE, RLF, or RRC_INACTIVE, UE discards any logged data - Availability indication can be triggered due to: - Full buffer being reached (if configured) - Buffer threshold being reached (if configured). - Low power (if configured) - The UE send a UAI that indicates: - Data is available - Reason for trigger (full buffer, threshold) - Low power indication - The encoding of the data is available/UAI and the cause value is FFS - NOTE: it is up to UE Implementation how buffer threshold reached and low power is - The measurement configuration of AI/ML data collection can configure measurements for multiple sets of resources and use cases (e.g. BM, Mobility, etc) - For temporal domain, the network is made aware whether there is a gap between two consecutive samples. FFS amount of gap and whether this is implicit or explicit - New SRB can be configured for NW-side data collection (with lower priority) - Next meeting proponents should work together and bring complete proposals to show specification impact and consider future use cases.

RAN2-130 agreements [7]:

Agreements on NW side data collection:

- (RRC-18) 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration with sync is introduced.  In case of HO, the source sends the 1-bit indication to target cell in HO preparation message. This 1-bit indication is included in HO command by target cell (if the target cell wants to keep the data).   We should have single UE behaviour, when it receives the indication it keeps it, otherwise it removes it.   Notify RAN3

- Buffer threshold to trigger data availability indication should be set based on specific size, e.g., KB instead of percentage

- UAI related to buffer status or low power state is triggered only once when specific conditions are met (e.g., buffer full/threshold, and low power state). A prohibit timer is not necessary for UAI related to buffer status or low power state

- No additional signaling from the UE is required when the low power issue is resolved

- No additional signaling from the UE is required when the buffer full issue is resolved

- As a starting point, the data logging is captured in RRC specs.

- Data is collected on per data logging configuration basis and UE indicates data logging configuration ID. An indication of the “gap” is needed.  “Gap” is time interval larger than the configured logging periodicity.    FFS if timestamp and relative time stamp for each group is needed per “group”.

- The UE should report the CGI of the serving cell whenever feasible. If CGI is unavailable, the UE shall log PCI-ARFCN as a fallback.