3GPP TSG-RAN WG2 Meeting #131	R2-2505883

# Bengaluru, India, 25th – 29th August 2025

Agenda Item:	8.7.6

Source:	Ericsson

Title:	Remaining Issues on XR Rate Control

Document for:	Discussion, Decision

# 1	Introduction

In UE capabilities, we believe that the current radio access capability for support of XR rate control is not sufficient in view of the CN providing information of which QoS flows are rate adaptable.

Thus, in this contribution, we discuss UE capabilities and their awareness in the CN.

# 2	Discussion

## 2.1	UE Capabilities

### 2.2.1	CN Awareness of UE Capability

From SA2’s LS reply [1], they have confirmed the CN can indicate to the gNB that a QoS flow is subject to uplink rate control and will discuss further on the whether to reuse existing mechanisms or come up with a new mechanism. It was also agreed in the last meeting that a new radio access UE capability is introduced to indicate the support for the XR rate control MAC CE. Then, the functional representation of the end-to-end XR rate control solution is as follows:

Figure 1: Functional Representation of the end-to-end XR rate control solution

In Figure 1, we see that the CN determines the rate adaptability of a QoS flow with the input from the UE and corresponding AF, then provides the information to the RAN. The RAN then based on the UE capability information can determine whether it can signal the XR rate control MAC CE. However, even if the UE does not support this capability, the CN still performs the determination of QoS flow rate adaptability and provides the information to the RAN, only to be ignored i.e., the CN is unaware of the contents of the UE radio access capabilities.

- CN is unaware of the contents of the UE radio access capabilities and whether it can support the XR rate control radio access capability, so will always perform the determination of QoS flow rate adaptability.

This rate adaptability is also used to setup a dedicated QoS flow to prevent multiplexing of non-rate adaptable flows, which could be unnecessary if the UE does not support the corresponding capability. Thus, RAN2 should discuss a solution to overcome this issue.

Discuss the issue on the CN being unaware of UE’s XR rate control capability when determining the rate adaptability of the QoS flow.

As we are discussing awareness in the CN, RAN2 should send an LS to SA2 to discuss this issue.

Send an LS to SA2 to discuss the issue about awareness of UE’s XR rate control capability in the CN.

# 3 	Conclusion

Based on the discussion in the previous sections, we have the following observations:

Observation 1	CN is unaware of the contents of the UE radio access capabilities and whether it can support the XR rate control radio access capability, so will always perform the determination of QoS flow rate adaptability.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	Discuss the issue on the CN being unaware of UE’s XR rate control capability when determining the rate adaptability of the QoS flow.

Proposal 2	Send an LS to SA2 to discuss the issue about awareness of UE’s XR rate control capability in the CN.

# 4 	References

- S2-2504252, LS reply on uplink rate control, 3GPP SA WG2 #168, Goteborg, Sweden.

- R3-250805, Reply LS on XR UL Rate Control, 3GPP RAN WG3 #127, Athens, Greece.

# 5	Annex

The following are the agreements from RAN2#128 meeting on the XR rate control objective:

Agreements on XR rate control - RAN2 confirms it is feasible for RAN to estimate the congestion information at both per-DRB and per-QoS flow level. - gNB can be indicated which QoS flows can be throttled. FFS whether this is indicated from UE/CN - Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB. - RAN2 will not discuss/support rate indication for DL unless WID is updated to include it by RANP. - RAN2 assumes that the congestion situation can be known at the gNB without any indication from the UE - FFS whether UL MAC CE rate query/preference is supported as UE recommendation to the NW or whether legacy MAC CE can serve this already. FFS in which scenarios this is useful.

The following agreement were made in RAN2#129 on the XR rate control objective:

Agreements on XR rate control - RAN2 assumes for XR rate control, the gNB receives QoS flow information from the CN, specifying which QoS flows are subject to uplink rate control (i.e., Option 2). Send an LS to RAN3 and SA2. We may revisit UAI option based on SA2/RAN3 reply - Specify a new table for XR rate control. FFS distribution (exponential, linear), codepoints etc. - We will try to design a table first and check whether it is possible to meet the required range/granularity. Afterwards, we can check whether multipliers are needed - Working assumption: - Support rate query MAC CE with the target to use same design that we will agree for rate indication MAC CE. - The rate query MAC CE is configurable by the network, i.e. the network may turn it off completely (same as legacy). Companies to check with their SA4 colleagues whether there are any issues with this

The following agreements were made in RAN2#129bis on the XR rate control objective:

Agreements on XR rate control - Bit rate in Rel-19 UL Rate Control MAC CE is a physical-layer bit rate. - We specify a single table. - As a starting point, we aim to have a table with 8 bits and exponential distribution and no multiplier. This can be subject to further agreements on MAC CE contents/design. Overhead will be considered. - The working assumption is changed to agreements on rate query MAC CE. - From RAN2 point of view per flow indication in MAC CE is preferred, but there are concerns on F1 impact which needs to be verified by RAN3, so RAN2 will go with per flow approach unless R3 has issues with this. - FFS how QFI is indicated, e.g. with DRB ID + QFI ID.