3GPP TSG-RAN WG2 Meeting #131	R2-2505894

Bengaluru, India, 25 - 29 August 2025

Agenda Item:	8.6.4

Source:	Huawei, HiSilicon

Title:	Intra-CU conditional LTM

Document for: Discussion and Decision

# Introduction

At RAN #107, the WID for Rel-19 NR mobility enhancements Phase 4 was updated. According to the updated WID [1], conditional LTM (C-LTM) should be specified for the intra-CU case in Rel-19, with limitations defined for the scenario where the UE is in a non-DC case.

RP-250339, Revised Work Item: NR mobility enhancements Phase 4 - Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC

In RAN2 meetings, RAN2 has already made some agreements. In this contribution, we discuss the remaining open issues listed the previous online/offline discussion.

# 2	Discussion

## 2.1	Co-existence between network triggered LTM and C-LTM

| RAN2#129bis meeting Network can send an LTM Cell Switch Command MAC CE indicating a CLTM candidate configuration (no specification change). |  |
| --- | --- |
| For MAC-14, Rapporteur suggests not to discuss this issue in this meeting, and leave it open (it will not impact the WI completion). It could be considered based on companies’ contributions in the future meetings. |  |
| MAC-14 | FFS for the case when CLTM candidate TAT timer is running, an (Enhanced) LTM Cell switch Command MAC CE is received Editor’s NOTE: FFS for the case when CLTM candidate TAT timer is running, an (Enhanced) LTM Cell switch Command MAC CE is received. [Rapp]: New added based on Anil’s comments. |

In RAN2#129bis meeting, it has been agreed that the network can trigger the UE to perform LTM cell switch with an LTM CSC MAC CE, even when the UE is configured with conditional LTM. According to the discussion in the last meeting, we should further discuss the TAT handling in this scenario. When a CLTM candidate TAT timer is running and the UE receives an LTM CSC MAC CE, we should clarify how the UE should handle the ongoing C-LTM TAT.

Q-1: How to handle three different TA values: the TA indicated in LTM CSC, the UE-measured TA, and the TA stored for CLTM?

For the conditional C-LTM procedure, the UE runs the C-LTM TAT upon reception of the TA value in the early TA MAC CE. If the UE is configured for UE-based TA measurements for an LTM candidate, the UE can calculate the TA value of the target cell autonomously. Additionally, the UE may receive the LTM CSC containing a TA value. Therefore, we need to determine the UE behaviour when these three TA values occur simultaneously.

According to the current procedure in TS 38.321 clause 5.18.35, the UE will apply the TA in the LTM CSC, unless the indicated value is FFF, in which case the UE will apply the autonomously determined TA if any, otherwise the UE will perform RACH (triggered by TS 38.331 for SCG LTM, and by transmission of the RRCReconfigurationComplete message to the target cell for MCG LTM).

This procedure seems perfectly suitable because:

-	if the network has already sent an early TA MAC CE to the UE, the network can store the TA value and know whether it is still valid, and if it is still valid the network can send it again;

-	if the UE is not configured with UE-based TA measurements for the LTM candidate cell and there is no valid TA available, the network can send the value FFF, and the UE will perform RACH,

-	if the UE is configured with UE-based TA measurements for the LTM candidate, if for some reason the network also has sent an early TA command MAC CE, the network can also send it again, and otherwise, the network can send FFF.

Proposal 1: (MAC-14) If the LTM CSC MAC CE is received indicating an LTM candidate for which execution condition is configured, the UE follows the unmodified Rel-18 procedure for the processing of the LTM CSC MAC CE: use the TA indicated in LTM CSC if the value is not FFF, used the UE-measured TA if available and the value is FFF, otherwise do not perform RACH-less LTM cell switch (i.e., RACH will be triggered)..

Q-2: How to handle the PTAG TAT in this scenario?

In Rel-18, the UE starts the PTAG TAT after the LTM cell switch is executed. For simplicity, the same behaviour should apply, regardless whether any execution condition was configured for the candidate LTM configuration for which the LTM SCS MAC CE is received.

Proposal 2: (MAC-14) Follow the legacy Rel-18 procedure: the PTAG TAT is started after the LTM cell switch.

Q-3: How to handle the stored TA and its corresponding TAT for candidate cells after the CSC MAC CE is received?

In the subsequent LTM, the candidate cells are maintained on the UE unless they are released by the network. After the LTM cell switch, the UE can maintain the valid TA values and the corresponding TAT for the candidate cells so that there is no need for the network to trigger the early TA acquisition procedure again for these candidate cells as long as their TA values remain valid, as ensured by the associated running TAT. Keeping these TA values will reduce the steps for TA acquisition for any further LTM cell switch, particularly eliminating the need for the UE to send a preamble to other candidate cells, which saves energy for the UE. Thus, it is beneficial to maintain the valid TA and its corresponding TAT for candidate cells (other than the target cell).

Proposal 3: (MAC-14) Upon cell switch execution triggered by reception of an LTM CSC MAC CE, the UE keeps the stored TA values and for other candidate cells and keeps the early C-LTM TA timer running for candidate cell(s).

## 2.2	DC configuration with C-LTM

R2-2503733 Report of [POST129b][119][Mob] Stage-2 CR 38.300 (Apple) Apple discussion Rel-19 NR_Mob_Ph4-Core Proposal 1: RAN2 to discuss and confirm if it is valid to have the UE configured with DC, but the CLTM configuration does not have any CLTM candidate cells with DC configured (i.e., explicit configuration for each of the CLTM candidates to inform the UE to release DC). [Apple]: Will have unofficial offline with interested companies. [Ericsson]: Support the proposal and understand there is no additional stage-3 spec impact to allow it. Comeback in CB session. Postpone it to the next meeting. [CB session]: [Apple]: We can postpone it to the next meeting. Apple plan to prepare some TP.

In Rel-18, the UE in DC can be configured with MCG candidate cells, either without any DC configuration or with an explicit indication to release the SCG. If there is no DC configuration, at MCG LTM cell switch execution, the SCG is unchanged. If there is an indication to release the SCG, at MCG LTM cell switch execution, the UE will release the SCG.

In the draft TS 38.331, there is a single procedure for LTM cell switch execution, regardless whether it is triggered by an execution condition that is fulfilled, or by reception of an LTM CSC MAC CE. In the draft 38.321 CR, there are some differences, but in Rel-18, for MCG LTM, whether or not the UE is configured with DC, and if it is, whether the SCG is released or kept, is transparent to TS 38.321.

Therefore, supporting for C-LTM all the MCG LTM DC scenarios that are supported in Rel-18 should not have any impact to TS 38.321 and TS 38.331, the only possibly impact is to UE capabilities, so the additional complexity seems rather low.

Proposal 4: C-LTM supports all the MCG LTM DC scenarios that are supported in Rel-18. No impact to TS 38.331 or TS 38.321 is expected.

## 2.3	Early DL synchronization

R2-2504283 Discussion on conditional LTM Nokia discussion Rel-19 NR_Mob_Ph4 Proposal 1: In FR2, after TCI state activation for a candidate cell, the UE shall stop L1 based condition evaluation for candidate cells without active TCI states. [MediaTek]: Support the proposal. [Ericsson]: It is clearly specified in RAN4. Do not see a need to specify anything in RAN2 spec. Comeback in CB session. Noted.

At the last meeting, it was argued that it should be clarified in RAN2 specifications that in FR2, after TCI state activation for a candidate cell, the UE should stop L1-based condition evaluation for candidate cells without active TCI states.

The RAN4 specification states that "There is no requirements are defined for any other cell(s) whose TCI state(s) is not in the active TCI state list." This means that the UE is cannot be required to perform L1-based condition evaluation. In the current draft RAN2 CRs, performing L1 measurements is covered in TS 38.321 as follows:

### 5.x.2	Performing measurement

An RRC_CONNECTED UE obtains L1 beam level measurement results by measuring one or multiple RSs as configured by the network as specified in [RAN1 REF] for the LTM candidate cell(s) with the candidate ID configured in ltm-CandidateReportConfigList. For each L1 beam level measurement result in RRC_CONNECTED, the UE applies the layer 1 filtering by implementation, before using the measured results for evaluation of reporting criteria and measurement reporting. The MAC entity performs the evaluation of reporting criteria as specified in 5.x.3 based on the L1 measurement results from lower layer. For the LTM candidate cell(s) with the candidate ID not configured in ltm-CandidateReportConfigList, the UE is not required to perform measurement and event evaluation in 5.x.3 on the reference signaling belonging to the candidate ID for the corresponding event.

For L1 beam level event triggered measurements report, the network can configure SS/PBCH block(s) or CSI-RS as event evaluation RS type, and L1-RSRP as trigger quantity. Reporting quantities is the same as the trigger quantity.

In TS 38.331, the L1 measurements are specified as follows:

3>	else if l1-Conditions is included within ltm-ServingCellExecutionCondition:

4>	request lower layers to initiate the LTM cell switch conditions evaluation based on L1 measurements according to the received field ltm-ServingCellExecutionCondition;

Therefore, RAN2 specifications either "request lower layers to initiate LTM cell switch conditions evaluations" or indicate that L1 beam level measurements are obtained as specified in physical layer specifications. How often the UE obtains such measurements depend on there physical layer specifications and, in some circumstances, such measurements may not be obtained at all. Such a situation does not seem to affect RAN2 specifications though.

One may suggest adding a note in RAN2 specifications about this aspect. If anything would be added, it should not duplicate anything from other specifications, so at most, a reference to RAN4 specifications could be added.

Proposal 5: No changes are needed to the RAN2 specification regarding the case where the UE is not required to perform L1 measurements according to RAN4 specifications (e.g., for candidate cells without active TCI states). At most, RAN4 specification would be referenced in the sentence in TS 38.321 that says "the UE obtains L1 beam level measurement results as specified in".

# 3	Conclusion

This contribution makes the following proposals:

Co-existence between network triggered LTM and C-LTM

Proposal 1: (MAC-14) If the LTM CSC MAC CE is received indicating an LTM candidate for which execution condition is configured, the UE follows the unmodified Rel-18 procedure for the processing of the LTM CSC MAC CE: use the TA indicated in LTM CSC if the value is not FFF, used the UE-measured TA if available and the value is FFF, otherwise do not perform RACH-less LTM cell switch (i.e., RACH will be triggered).

Proposal 2: (MAC-14) Follow the legacy Rel-18 procedure: the PTAG TAT is started after the LTM cell switch.

Proposal 3: (MAC-14) Upon cell switch execution triggered by reception of an LTM CSC MAC CE, the UE keeps the stored TA values and for other candidate cells and keeps the early C-LTM TA timer running for candidate cell(s).

DC configuration with C-LTM

Proposal 4: C-LTM supports all the MCG LTM DC scenarios that are supported in Rel-18. No impact to TS 38.331 or TS 38.321 is expected.

Early DL synchronization

Proposal 5: No changes are needed to the RAN2 specification regarding the case where the UE is not required to perform L1 measurements according to RAN4 specifications (e.g., for candidate cells without active TCI states). At most, RAN4 specification would be referenced in the sentence in TS 38.321 that says "the UE obtains L1 beam level measurement results as specified in".

# 4	Reference

- RP-250339, Revised Work Item: NR mobility enhancements Phase 4 Apple Inc, China Telecom.

- R2-250xxxx, Discussion summary and list of MAC open issue for Mob Ph4