3GPP RAN WG2 Meeting #131	R2-2505906

Bangalore, India Aug 25th – 29th, 2025

Agenda Item:	8.4.2

Source:	InterDigital, Inc.

Title:	Remaining issues on LP-WUS operation in RRC_IDLE/INACTIVE modes

Document for:	Discussion

# 1. Introduction

In RAN2#130 [1], following agreements were made:

Agreement The entry/exit condition for LP-WUS monitoring is mandatory in LP-WUS configuration, if the LP-WUS configuration is provided by the NW. Full coverage for LP-WUS is not precluded, e.g., if there needs to be a threshold value so that the condition is always fulfilled for all LPWUS UEs. It is up to UE implementation to choose whether SSB measurement based or OOK LP-SS measurement based conditions are used for LP-WUS monitoring entry/exit condition, if UE support both measurement types. RAN2 aim at supporting enabling/disabling LP-WUS monitoring in IDLE/INACTVE per UE, if the solution can be concluded in the August meeting.

In this contribution, we discuss LP-WUS operation in RRC_IDLE/INACTIVE mode.

# 2. Discussion

Indication of (de-)activation signaling for monitoring LP-WUS

When the UE is monitoring LP-WUS, the UE needs an additional latency to wake up MR and monitor PO from LP-WUS monitoring. The additional latency can be problematic for some use cases which are delay sensitive, e.g., the additional latency can be longer than the required delay. To avoid the additional latency, UE (de-)activation signaling needs to be supported. For example, when a UE is expected to receive DL data with delay sensitive traffic (e.g., XR) frequently, then the network can deactivate the LP-WUS monitoring (i.e., not to start LP-WUS monitoring) via the dedicated deactivation signaling so that the UE can receive the delay sensitive data  without additional wake-up delay from LP-WUS monitoring. This (de-)activation signlaing can be configured with the RRC release message, similar to one of the legacy configuration using for idle/inactive state (e.g., suspendConfig, cellReselectionPriorities) via the RRC release message. Another option to (de-)activate indication can be delivered via NAS message between UE and core network. In [2], RAN2 discussed these both solutions. From our view, RAN2 can support both solutions because each solution can operate with different signaling and layer, independently. Moreover, RAN2 can leave NW implementation which one of the signaling is used for the UE.

Proposal 1: Support both NAS and RRC signaling for (de-)activation indication LP-WUS monitoring per UE.

Proposal 2: Support (de-)activation indication LP-WUS monitoring per UE via RRC message if down-selection is neccessary.

Entry condition with relaxed LR measurements

Since both measurements (e.g., serving cell) via MR and LR can be considered to determine satisfying the entry condtion, RAN2 considers further whether/how to use both metrics via MR and LR (e.g., RSRP and (LP-)RSRP) for the determination when both MR-based and LR-based thresholds are configured. If RAN2 decides to consider always two metrics, additional power consumption with LR measurement and complexity of UE behavior (i.e., when to measure LP-RSRP via LR) cannot be avoided.

As a solution, RAN2 considers some conditions when to use metric(s) via MR measurement only with relaxed (or skipping) LR measurement for determining whether satysifying the entry condtion. For example, RAN2 can consider to use metric via MR only when the measurement configuration/parameters for MR and LR is similar (e.g., similar bandwidth, carrier frequency, band) that assumes results of MR measurement is similar to the results of LR measurement. For LR measurement relaxation, another MR-based threshold (e.g., entry condition with an offset) can be considered. For example, if measured value of MR measurement is good enough (e.g., cell-center) with satisfying the threshold, then the UE can start LP-WUS monitoring without utilizing LR-based measurement (e.g.,  given that LR-based threshold assumed to be satisfied).

Proposal 3: For satisfying LP-WUS entry condition, support to use MR measurement only with LR measurement relaxation (i.e., skipping LR measurement).

Exit condition with LR measurement

RAN2 agreed that entry/exit condition for LP-WUS monitoring is configured by SIB. To support entry/exit conditions for LP-WUS monitoring, the UE is configured with conditions. For example, a threshold of LP-RSRP/LP-RSRQ value of received LP-SS can be configured. The satisfied condition (e.g., above the threshold) of received LP-SS can be regarded as successful reception and the UE can keep monitoring LP-WUS. In contrast, if the condition is not satisfied by detecting number of reception failures (e.g., below the threshold of LP-RSRP) or measured value is below the threshold during a period of time (e.g., avoiding ping-pong effect), the UE stops LP-WUS monitoring and starts to monitor the legacy PO.

Proposal 4: For satisfying LP-WUS exit condition, support a number of LP-WUS reception failures.

Proposal 5: To avoid ping-pong effect, support to measure a quality of LP-WUS reception during a period time.

Information for LP-WUS configuration

According to the evaluation results from the TR, it is obvious that offloading RRM measurement from MR to LP-WUS is essential to avoid frequent wake up of MR from ultra deep sleep and achieve power saving gain from LP-WUS. Considering limited capacity of LP-WUR and reduced power consumption, providing information on neighboring cells (e.g., whether to provide LP-WUS related configuration) should be considered for LP-WUR based RRM measurement. Also, this information may be beneficial that the UE can (re-)select cell with supporting LP-WUS configuration while performing the cell (re-)selection procedure.

Proposal 6: Information on neighboring cells (e.g., LP-WUS capable) should be supported.

# 3. Conclusion

In this contribution, the following proposals were made:

Proposal 1: Support both NAS and RRC signaling for (de-)activation indication LP-WUS monitoring per UE.

Proposal 2: Support (de-)activation indication LP-WUS monitoring per UE via RRC message if down-selection is neccessary.

Proposal 3: For satisfying LP-WUS entry condition, support to use MR measurement only with LR measurement relaxation (i.e., skipping LR measurement).

Proposal 4: For satisfying LP-WUS exit condition, support a number of LP-WUS reception failures.

Proposal 5: To avoid ping-pong effect, support to measure a quality of LP-WUS reception during a period time.

Proposal 6: Information on neighboring cells (e.g., LP-WUS capable) should be supported.

# 4. References

[1] Chairman’s notes, 3GPP TSG RAN WG2 #130.

[2] R2-2505381, Summary of [Post130][222][LPWUS] Potential solution to support enabling/disabling LP-WUS monitoring in IDLEI/NACTVE per UE (Huawei).