3GPP TSG-RAN WG2 Meeting #131	R2-2505909

Bangalore, India  Aug 25th – 29th, 2025

Agenda item:		8.2.2

Source:		Futurewei

Title:		A new issue on A-IoT paging

Document for:		Discussion and Decision

# Introduction

RAN2 has reached the following highlighted agreements with respect to AS ID release:

in RAN2#130 [1]:

Agreements on AS ID release

-	For CBRA, to avoid AS ID being occupied for unnecessary time and to keep alignment between reader and device on AS ID release, device can release AS ID upon receiving paging message with different transaction ID, no matter the paging message is for it or not.   FFS for CFRA

-	FFS for need for release message

in RAN2 129-bis [2]:

Agreements on AS ID

1	AS ID is applied for Inventory + command case;

2	AS ID is not included in D2R message except Msg 1 (RN16 in Msg 1 has been agreed.

3	For both CFRA and CBRA, the AS ID size is same as RN 16, i.e. 16 bits.

4	Do not specify the reader behaviour on how exactly the ASID is generated.

5	The device releases the AS ID upon power off (no stage 3 specification impact);

6	The device only keeps one AS ID at a time.

7	For CFRA, command message is used for AS ID assignment

8	For CBRA, Msg 2 is used for AS ID assignment

9	The device releases the AS ID at least:

- upon receiving Paging with new transaction id for that device, i.e. different session/service

- when it triggers new msg1 transmission as a result of receiving Paging message (i.e. it has to generate a random ID for CBRA)

- FFS other cases for release ASID to avoid keeping it indefinitely.

In addition, in the latest running CR for AIoT MAC spec [3], it is also captured that the device releases the stored AS ID upon receiving a paging message indicating CFA in the Access Type field, as a result of running CR review.

Meanwhile, the authentication procedure recently adopted by SA3 may have complicated the situation with respect to AS ID retention/release. In this contribution, we discuss the potential impact of the authentication procedure to AS ID retention/release. We propose a solution to prevent the authentication procedure from interfering with AS ID retention/release, with TP for running CR of TS 38.391 in the Annex.

# Discussions

## SA3 specification involving AIoT paging message

The current TS 33.369 (v0.2.0) specifies the following [4]:

#### 4.2.1.3	Requirements for command protection

The AIoT device shall support confidentiality protection of AIoT NAS messages between the AIoT device and the AIOTF.

Confidentiality protection of AIoT NAS messages between the AIoT device and the AIOTF is optional to use.

The AIoT device shall support the following ciphering algorithms:

NEA0 and 128-NEA2 as specified in Annex D of TS 33.501 [5].

The AIoT device shall support integrity protection of AIoT NAS messages between the AIoT device and the AIOTF.

Integrity protection of AIoT NAS messages between the AIoT device and the AIOTF is mandatory to use.

The AIoT device shall support the following integrity algorithms:

128-NIA2 as specified in Annex D of TS 33.501 [5].

Editor’s Note: For this clause, the replay protection, privacy and authentication related requirements are FFS.

…

### 5.3.2	Security procedure on information protection during command procedure

Figure 5.3.2-1:  Security procedure on the information protection during command procedure

1.	The command procedure is initiated as specified in step 1-6 of clause 6.2.3 of TS 23.369 [2].

2.	The procedure as described in clause 5.2.2 shall be performed. The device and AIOTF acquire the KAIOTF key to be used for command protection.

Although the detailed description of KAIOTF key derivation is still missing, in accordance with the figure extracted from clause 5.2.2 in [4], as below, AIoT paging message is used in the procedure for Authentication and KAIOTF key derivation.

### 5.2.2	Authentication procedure

The authentication procedure is aligned with inventory procedure and command procedure in 6.2.2 and 6.2.3 of TS 23.369[2].

Figure 5.2.1-1: Authentication procedure

TS 33.369 v0.2.0 further specifies the following [4]:

5.3.3     Input parameters to integrity algorithm

The input parameters to the integrity algorithm as described in Annex D.3 in TS 33.501[5] shall be set as follows.

The KEY input is equal to the KAIOTF key.

Editor’s Note: whether the KAIOTF key is fresh for each round of inventory-command procedure is FFS.

The DIRECTION bit is set to 0 for uplink and 1 for downlink.

The BEARER is set to all zeros.

The COUNT is set to all zeros.

Key-nonce reuse is generally considered as a serious security violation. With integrity protection on commands being mandatory and with the COUNT value being fixed at zero, either the KAIOTF key needs to be refreshed or a new pair of RANDAIOT_n and RANDAIOT_d need to be established (and used as the nonce values) between the device and AIoT CN for each new command request and/or new command response. In either case, a new paging message needs to be transmitted for each new command request and/or new command response.

Observation 1. In accordance with the AIoT authentication procedure adopted by SA3, a new paging message needs to be transmitted for each new command request and/or new command response.

## Potential Impacts to AS ID retention/release

## A new paging message sent for a different “inventory-command” procedure has no impact to AS ID retention/release as the devices need to release the stored AS ID anyway. However, a new paging message sent in the mid of an “inventory-command” procedure may interfere with the AS ID retention/release by the devices. For example, a procedure may require more than one pair of command request and command response to complete. For another example, the NAS signaling based solution to the delayed NAS response problem would also require at least two command requests, e.g., the original Write command and the follow-up Read command. In these cases, the second paging message sent before the second command request for the intended device(s) to refresh either key or nonce values in the mid of the same “inventory-command” procedure may cause devices unintended for the second paging message to release their stored AS ID prematurely.

## Observation 2: The second paging message sent before the second command request for the intended device(s) to refresh either key or nonce values in the mid of an “inventory-command” procedure may cause devices unintended for the second paging message to release their stored AS ID prematurely.

## Solutions to avoid interference to AS ID retention/release

There can be a few simple solutions to prevent paging messages sent for the intended device(s) to refresh key or nonce values in the mid of an “inventory-command” procedure from interfering with the AS ID retention/release at devices unintended for the paging message. The first solution is to introduce a new R2D message to replace the paging message specified in the authentication procedure by SA3. However, this may increase the number of steps that the devices need to go through to complete an “inventory-command” procedure, not mentioning that another R2D message needs to be defined by RAN2 and further interaction with other WGs such as SA3, SA2, and RAN3 may also be needed.

The second solution is to explicitly specify, in TS 38.391, that devices unintended (as determined based on the Paging ID) for a paging message do not release their respectively stored AS ID (if any), if the paging message is sent for the intended device(s) to refresh either key or nonce values in the mid of an “inventory-command” procedure. First, a paging message sent for the intended device(s) to refresh either key or nonce values in the mid of a same “inventory-command” procedure is a paging message indicating CBRA with the TID value same as the TID value sent in the first paging message that started this “inventory-command” procedure, and the field containing the RANDAIOT_n is included in the paging message. Secondly, devices can determine whether the field containing the RANDAIOT_n is included in a paging message based on 1) the upper layers of the device, after receiving the Inventory Request from the AIoT MAC layer, indicate that the Inventory Request includes the security random number RANDAIOT_n; or 2) a new presence bit in the paging message indicates that the field containing the RANDAIOT_n is included in the paging message. We think 2) is reasonable as we may need to introduce a presence bit indicating whether the field containing the RANDAIOT_n is included in the paging message or not anyway. On the other hand, if the presence bit is not to be introduced in the paging message (e.g., if the inclusion of RANDAIOT_n in the paging massage is completely transparent to the AIoT MAC layer), then the first approach is also doable.

Proposal 1. Introduce a new presence bit in the paging message to indicate whether the field containing the security random number RANDAIOT_n is included in the paging message or not. The new presence bit can be referred to as the CN Security RN Presence field. The field containing the security random number RANDAIOT_n can be referred to as the CN Security RN field.

Proposal 2. Upon receiving a paging message with the CN Security RN Presence field indicating that the CN Security RN field is included in the paging message, the Access Type field indicating CBRA, and the TID field indicating the same value as the stored TID value, devices unintended for such paging message, as determined based on the Paging ID, do not release their stored AS ID.

To ensure that an intended device, which had previously successfully responded to the first paging message sent with a TID value for the first command request and the first command response, still responds to the second paging message sent with the same TID for the device to refresh key or nonce values for the second command request and/or second command response in the mid of the same service procedure, the previous agreement on how to avoid duplicated response may need to be revised. For example, if the second paging message is sent with the same TID as the stored TID, for which the device has successfully responded before, and the second paging message does not include the CN Security RN field, then even if the device is still considered as an intended device based on the Paging ID, this device still would not respond to the second paging message. On the other hand, if the second paging message includes the CN Security RN field and the device is an intended device for the second paging message (as determined based on the Paging ID), then this device needs to participate in responding to the second paging message.

Proposal 3. Upon receiving a paging message with the Access Type field indicating CBRA and the TID field indicating the same value as the stored TID value, for a device intended for the paging message (as determined based on the Paging ID), if the paging message does not include the CN Security RN field, the device does not respond to the paging massage; otherwise, if the paging message includes the CN Security RN field, the device participates in responding to the paging message.

TP for revising the running CR for TS 38.391 in accordance with Proposals 1-3 is included in the Annex.

Proposal 4. RAN2 to consider adopting the TP in the Annex to revise the running CR for TS 38.391.

# Conclusions

Observation 1. In accordance with the AIoT authentication procedure adopted by SA3, a new paging message needs to be transmitted for each new command request and/or new command response.

## Observation 2: The second paging message sent before the second command request for the intended device(s) to refresh either key or nonce values in the mid of an “inventory-command” procedure may cause devices unintended for the second paging message to release their stored AS ID prematurely.

Based on these observations, we propose the following:

Proposal 1. Introduce a new presence bit in the paging message to indicate whether the field containing the security random number RANDAIOT_n is included in the paging message or not. The new presence bit can be referred to as the CN Security RN Presence field. The field containing the security random number RANDAIOT_n can be referred to as the CN Security RN field.

Proposal 2. Upon receiving a paging message with the CN Security RN Presence field indicating that the CN Security RN field is included in the paging message, the Access Type field indicating CBRA, and the TID field indicating the same value as the stored TID value, devices unintended for such paging message, as determined based on the Paging ID, do not release their stored AS ID.

Proposal 3. Upon receiving a paging message with the Access Type field indicating CBRA and the TID field indicating the same value as the stored TID value, for a device intended for the paging message (as determined based on the Paging ID), if the paging message does not include the CN Security RN field, the device does not respond to the paging massage; otherwise, if the paging message includes the CN Security RN field, the device participates in responding to the paging message.

Proposal 4. RAN2 to consider adopting the TP in the Annex to revise the running CR for TS 38.391.

# References

R2_130_Chair_minutes_2025-05-23_13-00.

R2_129b_ChairNotes_25-04-11_16-45 final.

Draft_MAC running CR_v33_Rappv5_cl (to be uploaded as R2-2505520).

TS 33.369 v0.2.0.

# Annex: TP for revising the running CR of 38.391 [3]

-----------------Start of the First Change-----------------

## 5.2	A-IoT paging

The purpose of this procedure is to transmit A-IoT Paging message to one or more devices. The reader may include the Paging ID field to select a specific device or a group of devices, or may not include Paging ID field to select all devices.

The device monitors for the A-IoT Paging message, and determines whether the device is selected to initiate the access procedure.

Upon receiving the A-IoT Paging message, the A-IoT MAC entity shall:

1>	if the Access Type field in the A-IoT Paging message indicates CBRA:

2>	if the device has no stored Transaction ID; or

2>	if the value of the Transaction ID field is different from the stored Transaction ID; or

2>	if the value of the Transaction ID field is the same as the stored Transaction ID, and the previous procedure was determined as failed for this Transaction ID as specified in clause 5.5:

3>	if the device has no stored Transaction ID or if the value of the Transaction ID field is different from the stored Transaction ID:

4>	release the stored AS ID if any;

3>	store the received value in Transaction ID field, if the device has no stored Transaction ID, or replace the previously stored Transaction ID with the current received value, if the value of the Transaction ID field is different from the stored Transaction ID;

3>	if the Paging ID Presence Indication field indicates Paging ID field is absent:

4>	consider the device is selected and indicate to the upper layers;

3>	else:

4>	forward the value of the Paging ID field to the upper layers;

4>	if the upper layers indicate that the Paging ID is matched:

5>	consider the device is selected;

3>	if the device is selected:

4>	if the device has no stored Transaction ID or if the value of the Transaction ID field is different from the stored Transaction ID or if the CN Security RN Presence field is set to 1 in the A-IoT Paging message:

5> initiate Contention-Based Random Access procedure as specified in clause 5.3.1;

3>	else (i.e., if the device is not selected):

4>	if the value of the Transaction ID field is the same as the stored Transaction ID, and the previous procedure was determined as failed for this Transaction ID as specified in clause 5.5, and the CN Security RN Presence field is set to 0 in the A-IoT Paging message:

5>	release the stored AS ID if any;

1>	else (i.e., the Access Type field in the A-IoT Paging message indicates CFA):

2>	release the stored AS ID if any;

2>	release the stored Transaction ID, if any

2>	forward the value of the Paging ID field to the upper layers;

2>	if the upper layers indicate that this Paging ID is matched:

3>	consider the device is selected;

3>	initiate Contention-Free Access procedure as specified in clause 5.3.2.

-----------------End of the First Changes-----------------

-----------------Start of the Second Changes-----------------

#### 6.2.1.1	A-IoT Paging message

Figure 6.2.1.1-1 and 6.2.1.1-2 show the formats of the A-IoT Paging message.

The fields in this message are defined as follows:

-	R2D Message Type: This field indicates the message type. See the Table 6.1-1. The length of the field is 3 bits.

-	R: This field is a future extension indication. The length of the field is 1 bit, with the value set to 0 in this release.

-	Access Type (AT): This field indicates CBRA (when set to 1) or CFA (when set to 0). The length of the field is 1 bit.

-	D2R Scheduling Info: This field contains the physical layer parameters used for D2R transmission. The child fields are defined in clause 6.2.1.6.

For CBRA, the following fields are further included:

-	Paging ID Presence Indication (PIPI): This field indicates whether Paging ID and Length of Paging ID are present (when set to 1) or absent (when set to 0). The length of the field is 1 bit.

-	Paging ID Length: This field indicates the length of the Paging ID field in unit of bit when Paging ID field is present. The length of the field is 8 bits.

-	Paging ID: This field contains AIoT Identification Information (as defined in TS 23.369 [4], clause 5 and TS 23.003 [5]). .

-	Transaction ID: This field associates an inventory procedure or command procedure as specified in TS 38.300 [3]. The length of the field is xxx bits.

-	Number of Access Occasions: This field indicates the number of access occasions. The length of the field is 4 bits. The value 0 (i.e., 0000) indicates the number of access occasions is 20. The value 1 (i.e., 0001) indicates the number of access occasions is 21. The value 2 (i.e., 0010) indicates the number of access occasions is 22. And so on. The maximum number of access occasions is 215 when this field is set to 15 (i.e., 1111).

-	Fill Bits: This field is of variable size and is optionally present. It can be used to pad for byte alignment (1-7 bits) and/or contain future extensions. In this release, the device shall ignore the values of this field.

-	CN Security RN Presence: This field indicates whether the CN Security RN field is present (when set to 1) or absent (when set to 0). The length of the field is 1 bit.

-	CN Security RN: This field, when present, contains the security random number RANDAIOT_n. When present, the length of the field is 128 bits.

Editor’s Note: add the above new fields in Figure 6.2.1.1-1: MAC PDU of A-IoT Paging message indicating CBRA.

-----------------End of the Second Changes-----------------