3GPP TSG-RAN WG2 Meeting #131	R2-2505910

Bangalore, India  Aug 25th – 29th, 2025

Agenda item:		8.2.3

Source:		Futurewei

Title:		Remaining issue on A-IoT Random Access

Document for:		Discussion and Decision

# Introduction

After RAN2 meeting#130 and the open issue discussion during [POST130][027][AIoT] MAC Running CR e-mail discussion, the following issue with respect to A-IoT random access remains open [1]:

| Issue 2-4: CBRA failure detection | Further down selection between option B and C for msg2 monitor window in CBRA. - 1 Exclude the option of MSG2 transmission and any retransmission of MSG2 happens within a predefined time window (based on timer) - 2 A device expecting MSG2 assumes CBRA failure if its MSG2 is not received before a boundary, where the boundary can be further downselected between option B and C below. A device receiving MSG2 within this boundary transmits MSG3. The device does not process MSG2 (re)transmission received after the boundary. - Option B – the boundary is the reception of either the next R2D trigger message or the subsequent paging message - Option C – the boundary is the reception of either the kth R2D trigger message or the subsequent paging message (K is FFS) - Option A (the boundary being the subsequent paging only) is excluded. - For option C, further discuss in terms of complexity at the device vs reader flexibility. - 3 Including frequency index along with RN16 in MSG2 to reduce collisions of MSG1 between different devices is feasible. FFS Discuss further whether to include it. - Status in running CR: the common part of option B and C is captured, while the difference part is in [] and highligted. |
| --- | --- |

In this contribution, we provide our view on this issue, considering the latest progress made in SA3.

# Discussions

## Down selection between options B and C for Msg2 monitoring window

We consider Option B as a special case of Option C with K value being restricted to 1. Allowing K>1 provides some flexibility to the reader for scheduling purpose. If the K value is sufficiently small, the complexity added to the devices may be limited.

In addition, SA3 has recently specified an authentication procedure where the AIoT paging message and D2R Upper Layer Data Transfer message (as Msg3) are used, as shown in the extraction below from [2].

### 5.2.2	Authentication procedure

The authentication procedure is aligned with inventory procedure and command procedure in 6.2.2 and 6.2.3 of TS 23.369[2].

Figure 5.2.1-1: Authentication procedure

In Step 5 as shown above, the Msg3 includes not only the Device ID but also the 128-bit security random number RANDAIOT_d and the authentication challenge response RESAIOT. Depending on hardware implementation at the devices, it may be questionable whether all devices can finish the computation of RANDAIOT_d and RESAIOT in time to be included in the earliest Msg3 opportunity that the reader may schedule for the device, if K=1. Although one may argue that the device can generate the RANDAIOT_d in advance, the device must generate RESAIOT on the fly using the RANDAIOT_n provided by the AIoT CN as an input to the algorithm. Therefore, it may be necessary to support K>1 to allow more time for devices to compute the security related response in time to be included in Msg3 in the authentication procedure specified by SA3.

Observation: it may be necessary to support K>1 to allow more time for devices to compute the security related response in time to be included in Msg3 in the authentication procedure specified by SA3.

Making K a variable parameter may add the burden for the reader to configure the parameter and the complexity for devices to handle a variable configuration. Therefore, we support the K being a fixed but small value, such as 4.

Proposal.  RAN2 to adopt Option C with K being fixed to a small integer value, such as 4, for Msg2 monitoring window in CBRA.

# Conclusions

Observation: it may be necessary to support K>1 to allow more time for devices to compute the security related response in time to be included in Msg3 in the authentication procedure specified by SA3.

Based on the observation, we propose the following:

Proposal.  RAN2 to adopt Option C with K being fixed to a small integer value, such as 4, for Msg2 monitoring window in CBRA.

# References

R2-2505521, Summary of A-IoT MAC open issues (outcome of [POST130][027][AIoT] MAC Running CR).

TS 33.369 v0.2.0.