3GPP TSG-RAN WG2 Meeting #131	R2-2505911

Bengaluru, India,25th  – 29rd  August,  2025

Agenda item:	8.1.2.2

Source:	Nokia

Title:	LCM for UE-side Beam Management

WID/SID:	NR_AIML_air-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In this paper, we discuss open issues in [1], including specifying the cause of inapplicability, responding to RAN4 on activation delays after reporting applicability, how to capture the prevention of activation of an inapplicable periodic CSI-ReportConfig, implementation of RAN1 parameters in CSI-ReportConfig, and applicability reporting for inference-related parameters corresponding to full inference configurations. Additionally, we discuss the inclusion of the serving cell index in the applicability report. The annex includes a draft LS response to RAN4 regarding the activation delay.

# 2	Open Issues

The rapporteur provided a list of open issues for each agenda item prior to this meeting, RAN2#131.We will discuss those relevant to 8.1.2.2 in this section which are in line with Rapporteur proposals and summaries corresponding to each open issue collected in [1] .

## 2.1	(RRC-1) Cause of inapplicability

This section addresses the following open issue, RRC-1.

RRC-1 Issue description: It is FFS how to define the simple cause value of inapplicability related to model availability and how to capture it in the spec.

The following was agreed in RAN2#130

Agreement

Introduce “release configuration” flag instead of inapplicability cause to indicate UEs preference to release a configuration (e.g. due to model in availability in the local device FFS reason to be specified)

During discussion in RAN#130, it was evident that RAN2 could not conclude on reporting causes for inapplicability. The reasons provided weren’t invalid, but it was not clear how the gNB could use the information. Examples from the previous meeting included: mismatch in NW-side additional conditions; mismatch in UE-side additional conditions; UE-side internal conditions; model unavailable (e.g., due to time gap between transferring the model from the server to the on-chip memory at the UE); and model download duration (e.g., the download duration from the cloud to the UE)  in progress. The purpose of the release flag is to indicate to the network that the UE will not be capable of executing the AI/ML-enabled configuration or an AI/ML-enabled configuration based on a set of inference-related parameters during the present RRC session. The release flag enables the gNB to de-configure inapplicable configurations and sets of inference-related parameters, which are not expected to become applicable.

Observation 1: There is no need to specify anything about the cause for the UE providing the release configuration flag.

## 2.2 (RRC-15) and LS response to RAN4

The issue description for RRC-15 relating to a time requirement to produce reports from an AI/ML-enabled configuration based on CSI-ReportConfig is copied from the open issues list below.

Issue description: The UE needs to synchronize the time duration for an AI functionality to become available for inference with the network when the UE reports applicability for either a full inference configuration or a set of inference parameters.

Unlike conventional non-AI features, where the algorithm size is fixed, AI algorithms can vary significantly in size depending on their generalization performance and specific use cases. These sizes can range from several kilobytes to tens of megabytes. Additionally, UE implementations may use various types of memory to balance cost and performance, and these different memory types and constraints can affect the access speed of stored models.

The UE should indicate the time duration for the AI functionality to become available when reporting applicability to the network. This time duration refers to how long it takes for the UE to load the AI models into RAM or another accessible memory for inference. By providing this information, the network can better understand the timeframe within which the configured radio resources and AI model can be utilized, enabling more efficient and effective resource management, especially for periodic CSI.

Another option is to let RAN4 address this time duration in terms of requirements.

The excerpt from the related RAN4 LS [2] as follows:

Companies in RAN4 have different interpretations about the following RAN2 agreement: “the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4”. - Interpretation 1: the applicable functionalities are already activated (i.e., the applicable functionalities activation is completed) before reporting applicable functionalities via RRCReconfigurationComplete in step 4 - Interpretation 2: UE starts to activate the applicable functionalities (i.e., the applicable functionalities activation is not completed) upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 RAN4 kindly requests RAN2 to clarify which interpretation is aligned with RAN2 agreements.

RAN2 is discussing two open issues which are related to this:

RRC-15 discusses whether UE to indicate the time duration for an AI/ML configuration to become available for inference after reporting applicability to the network via RRCReconfigurationComplete. On the other hand, RRC-17 discusses the processing timing requirement between the reception of full inference configuration or inference related parameters via RRCReconfiguration and applicability/inapplicability reporting via RRCReconfigurationComplete.

From these two open issues, we observed that AI/ML configuration will become available for inference, in other words, UE can activate the applicable functionalities after UE sends the applicability report via RRCReconfigurationComplete in Step 4. No additional indication related to time is needed. Moreover, according to Rapporteur proposal in [1], the processing latency requirement of 16ms (RRC-17) should be enough to report applicability to network. We, therefore, acknowledges that Interpretation 2 made by RAN4 is similar to our view.

Proposal 1: (RRC-15) RAN2 confirms that interpretation 2 of the RAN4 LS on AI/ML functionality activation is correct.

Proposal 2: Send an LS response to RAN4 confirming interpretation 2. The draft LS is attached in the Annex.

## 2.3 (RRC-45) How/where to capture activation/deactivation of periodic inference CSI-ReportConfig in specifications

Issue description: RAN2 agreed that the UE should activate a periodic inference configuration in CSI-ReportConfig only if it reports that it is applicable. There are several possibilities to capture this in the specifications, as commented by several companies, and it should be discussed how to best do this. Furthermore, it should be clarified if the solution can be captured only in RRC specifications or whether RAN1 should also capture (part of) the solution.

Some solutions suggested by companies for several aspects are:

-	Where exactly in RRC to capture the solution:

a)	In Section 5.3.5.3 when setting the content of RRCReconfigurationComplete, or

b)	In Section 5.3.5.3 when sending RRCReconfigurationComplete to the lower layers

-	What to write in RRC specifications:

i.	Upon including the applicable status in RRCReconfigurationComplete, indicate to lower layers to activate the configuration, or

ii.	Submit/do not submit the configuration to lower layers, which would be the equivalent of activation/no activation.

RAN1 specifications do not allow for periodic CSI-ReportConfig to be activated or deactivated whereas a semipersistent CSI-ReportConfig can be activated or deactivated. The desired behavior is different when it comes to an inapplicable periodic CSI-ReportConfig. To the lower layers, it should be as if the periodic configuration was never provided since it can’t be activated later anyway. Therefore, we prefer option b.ii: “Submit/do not submit the configuration to lower layers, which would be the equivalent of activation/no activation.”

Observation 2: Option b.i would require the RAN1 specification to implement an activation and/or deactivation command for periodic CSI-ReportConfig. We do not see this as necessary and that we can accomplish the same effect in our RAN2 specifications.

Proposal 3: (RRC-45) When sending the RRCReconfigurationComplete to lower layers, for an inapplicable periodic AI/ML-enabled CSI-ReportConfig, do not submit the configuration to the lower layers.

## 2.5 (RRC-47) How to implement RAN1 parameters in CSI-ReportConfig

Issue description: RAN1 LS (R2-2505000) needs implementation and adoption to ASN.1 principles and RAN2 agreements. There are several ways to implement these parameters in CSI-ReportConfig in ASN.1.

There are two main solution directions:

-	List all parameters (for inference, performance monitoring, UE data collection) one after another without any hierarchy; or

-	Group some of the parameters with a CHOICE structure, to clarify which parameters are allowed to be configured together. For instance, the latest version of the RRC running CR proposes a structure with a CHOICE branch for inference (and UE data collection) and a second one for performance monitoring.

Grouping of CSI-ReportConfig parameters with a CHOICE structure replicates many parameters. We understand the reason or trying to group the parameters, but it is already the case that the gNB could provide the UE invalid configurations, since not all combinations of parameters can be configured. Therefore, we propose to follow the precedent, trusting the gNB to provide valid configurations to the UE depending on what is being configured.

Additionally, the grouping is only valid for the first version of the specification. Unless we determine we want to support extension markers inside of extension markers inside of CSI-ReportConfig, adding one new parameter applicable to all groups

Observation 3: The grouping of parameters for inference, performance monitoring and UE data collection is only valid for the first version of the specification and subsequent updates to each would require the creation of new groups. In the worst case if the same parameter is added to all three groups, to maintain consistency, three new groups matching the initial three would need to be added in the future release.

Proposal 4: (RRC-47) RAN2 to support ungrouping RAN1 parameters and benefit from using extension markers and adding new fields, new sets of parameters for inference, monitoring and data collection.

## 2.6 (RRC-48) Applicability reporting for inference related parameters sets after the corresponding full inference configuration is provided

Issue description: Based on current agreements, the UE reports the applicability of both full inference configurations and corresponding inference related parameter sets, even if some of them may correspond to each other.

The issue was raised that once the network provides full inference configuration for applicable inference related parameter set, it would be highly inefficient if UE keeps reporting both full inference configuration ID and inference related parameter set config ID upon change in applicability. One solution is to allow the UE to either provide the applicability of inference related parameter set or inference configuration once the inference configuration is provided based on the applicability of inference related parameter set.

We understand the intention of optimizing the reporting of the applicability of full inference configurations and on sets of inference-related parameters. However, the overhead of signaling the applicability of a set of inference-related parameters at the same time as signalling the applicability of a full inference configuration would be approximately the size of identifier, which could be as large as 8 bits. We do not see it as valuable to save 8 bits in the applicability report.

Observation 4: The overhead of signalling the applicability of a set of inference-related parameters at the same time as signalling the applicability of a full inference configuration would be approximately the size of identifier, which could be as large as 8 bits, which is not significant.

It has not been discussed how to associate a full inference configuration with a set of inference-related parameters, so we assume that the intention is that the UE would implicitly make the determination. Because we have not agreed to an explicit link, we think that it would be safer to leave the applicability reporting procedures for full inference configurations and sets of inference-related parameters as independent so that we do not have to introduce conditions to the procedures for applicability reporting.

Observation 5: It would be safer to leave the applicability reporting procedures for full inference configurations and sets of inference-related parameters as independent so that we do not have to introduce conditions to the procedures for applicability reporting.

Therefore, we propose not to introduce a link, explicit or implicit, between a full inference configuration and a set of inference-related parameters.

Proposal 5: (RRC-48) Do not introduce a link, explicit or implicit, between a full inference configuration and a set of inference-related parameters. If the applicability of a full inference configuration changes and there is a corresponding set of inference-related parameters whose applicability changes at the same time, the UE shall report the applicability of both.

# 3	Use of ServCellIndex for applicability determination

Applicability reporting is cell specific. In the case of beam management and CSI prediction, the CSI-ResourceConfig is specific to the cell which transmits the CSI-RS whose measurements are to be reported based on a CSI-ReportConfig. In case of DL-only component carriers, only the PCell has uplink capability. Therefore, the CSI report would be sent over the PCell, while the CSI-RS would be measured from the SCell. The applicability of the reporting configuration corresponds to the UE’s ability to make inference on the CSI-RS for a specific SCell. Therefore, the cell ID reported for applicability should be that of the SCell.

However, the CSI-ReportConfig already includes a field called carrier, which is the ServCellIndex or absent if the reporting is on measurements of the PCell. Therefore, reporting the applicabilityCellId is redundant.

Observation 6: The applicabilityCellId refers to the ServCellIndex of the cell which transmits the CSI-RS, and the carrier field in CSI-ReportConfig refers to the ServCellIndex of the cell which transmits the CSI-RS, making the applicabilityCellId redundant.

Proposal 6: Remove applicabilityCellId from ApplicabilityReport-r19.

# 4	Conclusion

This document has made the following observations:

Observation 1: There is no need to specify anything about the cause for the UE providing the release configuration flag.

Observation 2: Option b.i would require the RAN1 specification to implement an activation and/or deactivation command for periodic CSI-ReportConfig. We do not see this as necessary and that we can accomplish the same effect in our RAN2 specifications.

Observation 3: The grouping of parameters for inference, performance monitoring and UE data collection is only valid for the first version of the specification and subsequent updates to each would require the creation of new groups. In the worst case if the same parameter is added to all three groups, to maintain consistency, three new groups matching the initial three would need to be added in the future release.

Observation 4: The overhead of signalling the applicability of a set of inference-related parameters at the same time as signalling the applicability of a full inference configuration would be approximately the size of identifier, which could be as large as 8 bits, which is not significant.

Observation 5: It would be safer to leave the applicability reporting procedures for full inference configurations and sets of inference-related parameters as independent so that we do not have to introduce conditions to the procedures for applicability reporting.

Observation 6: The applicabilityCellId refers to the ServCellIndex of the cell which transmits the CSI-RS, and the carrier field in CSI-ReportConfig refers to the ServCellIndex of the cell which transmits the CSI-RS, making the applicabilityCellId redundant.

And the following proposals:

Proposal 1: (RRC-15) RAN2 confirms that interpretation 2 of the RAN4 LS on AI/ML functionality activation is correct.

Proposal 2: Send an LS response to RAN4 confirming interpretation 2. The draft LS is attached in the Annex.

Proposal 3: (RRC-45) When sending the RRCReconfigurationComplete to lower layers, for an inapplicable periodic AI/ML-enabled CSI-ReportConfig, do not submit the configuration to the lower layers.

Proposal 4: (RRC-47) RAN2 to support ungrouping RAN1 parameters and benefit from using extension markers and adding new fields, new sets of parameters for inference, monitoring and data collection.

Proposal 5: (RRC-48) Do not introduce a link, explicit or implicit, between a full inference configuration and a set of inference-related parameters. If the applicability of a full inference configuration changes and there is a corresponding set of inference-related parameters whose applicability changes at the same time, the UE shall report the applicability of both.

Proposal 6: Remove applicabilityCellId from ApplicabilityReport-r19.

# 5	References

[1] [POST130][026][AI PHY] 38.331 CR (Ericsson) R2-25xxxxx_Remaining RRC open issues in feature AIML PHY

[2] R2-2505045, LS on AI/ML functionality activation (R4-2508085; contact: CMCC)

# Annex

3GPP TSG-RAN WG2 Meeting #131	R2-250xxxx

Bengaluru, India, 25 – 29 August 2025

Title:	[DRAFT] Reply LS on AI/ML functionality activation

Response to:	R2-2505045/R4-2508085

Release:	Release 19

Work Item:	NR_AIML_air-Core

Source:	Nokia [TSG RAN WG2]

To:	TSG RAN WG4

Cc:	TSG RAN WG1

Contact Person:

#### Name:	First Last

E-mail Address:	first.last@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 would like to thank RAN4 for the LS in R2-2505045/R4-2508085 on applicability functionality activation.

RAN2 clarifies that interpretation 2 is aligned with RAN2 agreements.

2. Actions:

To RAN WG4.

ACTION: 	RAN2 respectfully asks RAN4 to take the above agreements for consideration.

3. Date of Next TSG-RAN WG2 Meetings:

RAN2#131bis	from 2025-10-13	to 2025-10-17		Prague, CZ

RAN2#132	from 2025-11-17	to 2025-11-21		Dallas, USA