3GPP TSG-RAN WG2 Meeting #131	R2-2505913

Bangalore, India  Aug 25th – 29th, 2025

Agenda item:		8.2.4

Source:		Futurewei

Title:		Discussion on handling the delayed NAS response

Document for:		Discussion and Decision

# Introduction

In RAN2#130, RAN2 discussed the timing of AS/NAS interaction when a command is sent to the A-IoT device, and reached the following agreement [1]:

- The device is expected to send a MAC response to the reader in the D2R occasion.   The MAC response contains the NAS message if available at the D2R occasion.   If there is no NAS message available to transmit at the D2R occasion then the response contains MAC with 0 SDU and padding as needed.

This agreement does not address the issue that, when the NAS response is delivered to the device’s MAC layer later, there are no D2R radio resources provided by the reader for the device to transmit the delayed NAS response. RAN2 sent an LS to CT1, asking if CT1 can address this issue based on NAS signaling [2].

In this contribution, we discuss some potential issues associated with NAS signaling based solution and propose an alternative solution that is completely based on AIoT MAC specification.

# Discussions

## NAS signaling based solution and its drawbacks

A NAS signaling based solution means that the procedure would involve at least two command requests, e.g., the original Write command and a follow-up Read command sent to solicit the delayed NAS response from the device. It is unclear at this point, in addition to the delayed NAS response, whether CT1 will specify an immediate NAS response for the device to convey the delaying of the final NAS response so that the AIOTF can initiate the follow-up Read command based on the immediate NAS response, instead of based on the null NAS response.

The current TS 33.369 (v0.2.0) specifies the following [3]:

#### 4.2.1.3	Requirements for command protection

The AIoT device shall support confidentiality protection of AIoT NAS messages between the AIoT device and the AIOTF.

Confidentiality protection of AIoT NAS messages between the AIoT device and the AIOTF is optional to use.

The AIoT device shall support the following ciphering algorithms:

NEA0 and 128-NEA2 as specified in Annex D of TS 33.501 [5].

The AIoT device shall support integrity protection of AIoT NAS messages between the AIoT device and the AIOTF.

Integrity protection of AIoT NAS messages between the AIoT device and the AIOTF is mandatory to use.

The AIoT device shall support the following integrity algorithms:

128-NIA2 as specified in Annex D of TS 33.501 [5].

Editor’s Note: For this clause, the replay protection, privacy and authentication related requirements are FFS.

To protect the two command requests required in the NAS signaling based solution, e.g., the original Write command and the follow-up Read command, a unique pair of key-nonce values need to be used for each command request. If there is an immediate NAS response in addition to the delayed NAS response, the same unique pair of key-nonce values used on the command request can be used on the respective command response. However, the current TS 33.369 (v0.2.0) also specifies the following [3]:

5.3.3     Input parameters to integrity algorithm

The input parameters to the integrity algorithm as described in Annex D.3 in TS 33.501[5] shall be set as follows.

The KEY input is equal to the KAIOTF key.

Editor’s Note: whether the KAIOTF key is fresh for each round of inventory-command procedure is FFS.

The DIRECTION bit is set to 0 for uplink and 1 for downlink.

The BEARER is set to all zeros.

The COUNT is set to all zeros.

With the COUNT value fixed at zero, either the KAIOTF key needs to be refreshed after the immediate MAC response and before the follow-up Read command, using the authentication procedure described in [3] and illustrated below, or another value that is variable can be used as the nonce value. For example, the RANDAIOT_n and RANDAIOT_d generated during the authentication procedure can be used to replace the COUNT as the nonce value used in the security algorithms.

### 5.2.2	Authentication procedure

The authentication procedure is aligned with inventory procedure and command procedure in 6.2.2 and 6.2.3 of TS 23.369[2].

Figure 5.2.1-1: Authentication procedure

In either case, such authentication procedure involves, in the AS layers, the transmission of a paging message by the reader for carrying RANDAIOT_n (i.e., the 128-bit random number generated by the core network) to the device and the transmission of a D2R Upper Layer Data Transfer message by the device for carrying at least RANDAIOT_d (i.e., the 128-bit random number generated by device) to the core network, with significant amount of energy consumed by the device in the process before the device can send its delayed NAS response, hence jeopardizing its chance to successfully complete the procedure.

Observation 1: A NAS signaling based solution for the delayed AIoT NAS response requires refreshing key or nonce values at the device for the follow-up Read command, with significant amount of energy consumed by the device in the process before the device sends its delayed NAS response, hence jeopardizing its chance to successfully complete the procedure.

Finally, RAN2 is to complete the Rel-19 AIoT WI in this meeting. However, it is unclear if a NAS signaling based solution will be specified timely by CT1.

## AIoT AS-only based solution

Therefore, an AIoT AS-only based solution is considered below as an alternative solution (or at least as a backup solution just in case the NAS signaling based solution is not timely available). An AIoT AS-only based solution means that the reader doesn’t send the Command Response to the AIOTF until it retrieves the delayed NAS response from the device and the R2D message sent to retrieve the delayed NAS response is triggered by the reader itself, not by the AIOTF. Compared to the NAS signaling based solution, the AS-only based solution does not require refreshing the security key in the mid of the procedure, because no extra command request or command response is involved (i.e., the reader does not send the immediate (null) Command Response initially after receiving the immediate MAC response with 0 SDU and the AIOTF does not send the follow-up Read command to retrieve the delayed NAS response).

In the AS-only based solution, the reader needs to solely rely on the initial MAC response from the device to determine:

- whether the original command request is received by the device;

- whether the NAS response is currently available;

- if no NAS response is currently available, whether the NAS response will be available later; and

- if the NAS response will be available later, when to schedule the D2R to retrieve the delayed NAS response.

The initial MAC response with 0 SDU already provides sufficient indication for the reader to determine: 1) the original command request is received by the device, and 2) the NAS response is currently unavailable. However, 0 SDU does not necessarily indicate that the device will have data to send at a later time. For example, in a case that does not necessarily involve a delayed NAS response, the device may have sent the previous D2R Upper Layer Data Transfer message with 1 SDU and MDI=0 but the reader receives it as if MDI=1 so that the reader schedules another D2R Upper Layer Data Transfer message for the device to send the remaining segment, for which the device needs to respond by sending 0 SDU with MDI=0 to indicate that the device has no more bits (after the Offset number of bits) to send. A false CRC-check with the wrong MDI value may occur when the previous D2R transmission is the last segment that is so short that CRC-6 is used on the previous D2R transmission. Therefore, we may need to preserve the combination of 0 SDU and MDI=0 to indicate the case where the device truly has no data to send.

Observation 2: we may need to preserve the combination of 0 SDU and MDI=0 to indicate the case where the device truly has no data to send.

Therefore, we think it is better to use the combination of 0 SDU and MDI=1 to indicate that the NAS response is currently unavailable but will be available later. Some companies have raised concern that MDI=1 will cause the reader to immediately schedule another D2R Upper Layer Data Transfer message for the device, which may be wasted again. We think that the reader should decide to schedule another D2R Upper Layer Data Transfer message for the device whether immediately or only after some delay based on the combination of SDU number (being 0 SDU or 1 SDU) and MDI=1, given the readers are far more sophisticated devices than the AIoT devices. The table below describes the different combinations of SDU number and MDI value.

| SDU # in D2R | MDI in D2R | Status at the Device | Reaction of the Reader |
| --- | --- | --- | --- |
| 0 | 0 | Device has no data to send. | Nothing special. |
| 0 | 1 | Device has NAS PDU to send but the NAS PDU is not available yet. | Send R2D at a later time to schedule the device to send another D2R. |
| 1 | 0 | Device has sent the entire NAS PDU without segmentation. | Nothing special. |
| 1 | 1 | Device has sent a segment of the NAS PDU while there is remaining segment ready for transmission. | Immediately send R2D to schedule the device to send another D2R. |

Although the MDI value is currently generated by the device internally only due to segmentation status, we think it is not difficult for the device to internally generate a delay status of the NAS response and then use the logic OR of the segmentation status and the delay status to derive the MDI value.

As to the question of how the reader determines when to schedule the D2R Upper Layer Data Transfer message to retrieve the delayed NAS response, we think the amount of time to be delayed can be based on the reader’s experience. For example, the reader can, based on experience, send the follow-up R2D Upper Layer Data Transfer message at a later time that is sufficiently late to retrieve the delayed NAS response. The reader can also test the water by initially scheduling the follow-up D2R Upper Layer Data Transfer message with a D2R TBS that may be smaller than what is required to retrieve the entire NAS response. For example, the D2R TBS indicated in the follow-up R2D Upper Layer Data Transfer message may be sufficient to retrieve only one byte of the delayed NAS response. In this case, if the NAS response is still unavailable at the device’s MAC layer, the wasted D2R resource is limited, while if the NAS response indeed becomes available, the device will send the first byte of the delayed NAS response in the SDU field with MDI=1. Then, the reader can send another follow-up R2D Upper Layer Data Transfer message to schedule another D2R Upper Layer Data Transfer message, with the D2R TBS indicated in the follow-up R2D Upper Layer Data Transfer message sufficient to retrieve the rest of the delayed NAS response. All of these (i.e., time delayed and D2R TBS selected in the follow-up R2D Upper Layer Data Transfer message) can be handled by reader implementations.

Finally, as to the question of whether the follow-up R2D Upper Layer Data Transfer message should include the original command or the Offset (aka Received Data Size) value, since the 0 SDU in the initial MAC response already serves as an acknowledgement that the device has received the command, we think there is no need to repeat the command in the follow-up R2D Upper Layer Data Transfer message to avoid duplicated execution of the command by the device or the extra complexity on the device to avoid duplicated execution of the command. Instead, the follow-up R2D Upper Layer Data Transfer message should include the Received Data Size field, with the Received Data Size field initially set to value 0, so that the device will transmit the delayed NAS response, if available, from the beginning. This R2D MAC PDU format is consistent with the segmentation case where the R2D Upper Layer Data Transfer message is sent to retrieve the remaining D2R segment in response to MDI=1 in the previous D2R Upper Layer Data Transfer message.

Proposal 1. RAN2 to consider an AIoT AS-only based solution for the delayed NAS response case, as an alternative or at least as a backup to the NAS signaling based solution, where the reader does not send an immediate Null Command Response to the AIOTF when 0 MAC SDU is received in response to the command initially.

Proposal 2. MDI is set to 1 in the device’s initial MAC response with 0 SDU when the NAS response is currently unavailable but will be available later.

Proposal 3. The reader, in response to 0 SDU and MDI=1 in the device’s MAC response, will send a follow-up R2D Upper Layer Data Transfer message at a later time to schedule another D2R Upper Layer Data Transfer message from the device. The follow-up R2D Upper Layer Data Transfer message includes the Received Data Size field with the Received Data Size field set to value 0, without including the original command.

Proposal 4. If the delayed NAS response becomes available at the MAC layer of the device, the device responds to the follow-up R2D Upper Layer Data Transfer message by sending a D2R Upper Layer Data Transfer message with 1 SDU (where the MDI is set based on the segmentation status); otherwise, if the delayed NAS response is still unavailable at the MAC layer of the device, the device sends a MAC response with 0 SDU and MDI=1 again.

Proposal 5. The reader sends the Command Response to the AIOTF after the reader has retrieved the delayed NAS response from the device.

# Conclusions

Observation 1: A NAS signaling based solution for the delayed AIoT NAS response requires refreshing key or nonce values at the device for the follow-up Read command, with significant amount of energy consumed by the device in the process before the device sends its delayed NAS response, hence jeopardizing its chance to successfully complete the procedure.

Observation 2: we may need to preserve the combination of 0 SDU and MDI=0 to indicate the case where the device truly has no data to send.

Based on these observations, we propose the following:

Proposal 1. RAN2 to consider an AIoT AS-only based solution for the delayed NAS response case, as an alternative or at least as a backup to the NAS signaling based solution, where the reader does not send an immediate Null Command Response to the AIOTF when 0 MAC SDU is received in response to the command initially.

Proposal 2. MDI is set to 1 in the device’s initial MAC response with 0 SDU when the NAS response is currently unavailable but will be available later.

Proposal 3. The reader, in response to 0 SDU and MDI=1 in the device’s MAC response, will send a follow-up R2D Upper Layer Data Transfer message at a later time to schedule another D2R Upper Layer Data Transfer message from the device. The follow-up R2D Upper Layer Data Transfer message includes the Received Data Size field with the Received Data Size field set to value 0, without including the original command.

Proposal 4. If the delayed NAS response becomes available at the MAC layer of the device, the device responds to the follow-up R2D Upper Layer Data Transfer message by sending a D2R Upper Layer Data Transfer message with 1 SDU (where the MDI is set based on the segmentation status); otherwise, if the delayed NAS response is still unavailable at the MAC layer of the device, the device sends a MAC response with 0 SDU and MDI=1 again.

Proposal 5. The reader sends the Command Response to the AIOTF after the reader has retrieved the delayed NAS response from the device.

# References

R2_130_Chair_minutes_2025-05-23_13-00.

R2-2504967 LS to CT1 on delayed NAS PDU.

TS 33.369 v0.2.0.