3GPP TSG-RAN WG2 Meeting #131	R2-2505919

Bengaluru, India, 25th – 29th Aug, 2025

Agenda item:	8.17

Source:	Samsung

Title:	On open issues on IoT NTN TDD

WID/SID:	IoT_NTN_TDD

Document for:	Discussion and Decision

# Introduction

In RAN#106 the WID for IoT NTN TDD was agreed [1]:

This work item includes the following objectives: - Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern, necessary adaptation (if needed) and associated UE procedures [RAN1, RAN2] - Support a pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period.RAN1 to consider whether there is a need for a mechanism to achieve an adjustable guard period for the purpose of allowing deployment with the TDD frame structure of the legacy system operating in the target MSS allocated band. - Other necessary impacts on higher layers [RAN2] - RRM and RF core requirements [RAN4] - Specify a new NB-IoT TDD operating NTN band for the MSS allocation spanning 1616-1626.5 MHz for DL and UL, based on the outcome of the study, to be used as example band for this WI [RAN4]. - Specify band numbering - Specify SAN and UE RF characteristics - Specify DL and UL channelization. - Specify channel bandwidth as 200 kHz - Note1: No NTN-NTN coexistence study needed. - Note2: Leverage existing work as much as possible for TN-NTN coexistence of adjacent bands

In this contribution, we provide some initial discussion on IoT NTN TDD.

# Discussion

- The key aspect of the work item is to adapt NB-IoT NTN to a special TDD band n249, which can be seen in Figure 1. The UL and DL frames are 8.28 ms, the simplex frame is 20.32 ms and with guard bands the total comes in to 90 ms.

Figure 1.

- The challenging aspects of this specific frame is that the periodicity of the TDD bands has a periodicity that is not aligned with 3GPP periodicities and frame structures, and that each UL or DL frame being 8.28 ms makes allocating the subframes difficult. Another challenging aspect has to do with the fact that legacy services prevent all of the UL and DL frames to be available for NB-IoT. This means that very few subframes are available, potentially interrupting or delaying procedures. Due to the existing services, it was agreed in the WID that only a single UL and DL frame with 8 subframes each, are available every 90 ms.

## Extended k-Mac

- In RAN2#129bis, the following is agreed:

6.	For IoT NTN TDD mode, support k-Mac with a value range up to 1023 ms (add corresponding a restriction in the field description)

- This has been implemented in the latest CR:

- However, one issue with this is that the value range currently encompasses already existing value range of k-Mac. This may make it confusing for an implementer, who may believe that k-Mac-r19 always need to be implemented. So we propose that k-Mac-r19 shall only be used for extension purposes. In other words, the value range of k-Mac-r19 shall be 513 to 1024.

Proposal 1: Change the value range of k-Mac-r19 to 513...1024.

## GNSS measurement gaps

- In RAN1#120bis, RAN1 had the following agreements:

Agreement

For GNSS measurement gap, RAN1 to further discuss at least the following options:

- Option 1: No enhancement on the Rel-18 timeline for the start of GNSS gap

- Option 2: The start of the GNSS measurement gap starts at a non-D/non-U subframe

- Option 3: GNSS measurement gap does not apply for NB-IoT NTN TDD

- And then in RAN2#121, RAN1 subsequently took the following decision:

Agreement

For GNSS measurement gap:

- GNSS measurement gap can be applied in NB-IoT NTN TDD.

- RAN1 does not specify any enhancement on the Rel-18 timeline for the start of GNSS gap.

- GNSS measurement gaps were partly specified by RAN1, but mostly implemented in RAN2. This feature which on the surface seems simple has a lot of conditions and special cases and it took a considerable effort from RAN2 to specify it. NB-IoT NTN TDD use case, where at most 8 DL and UL subframes are available for data transmissions every 90 ms, will not allow for higher data rates and it is thus unlikely that a UE in such a use case will be in connected mode for long enough where the GNSS is no longer valid. With this in mind, we propose that RAN2 can make a decision to down-prioritize any enhancements on GNSS gap.

Proposal 2: RAN2 does not specify any enhancements related to GNSS measurement gaps for IoT NTN TDD.

## Neighbour cell assistance information

In RAN4#114bis the following was agreed:

Agreement: For the cell reselection for TDD NB-IoT in NTN, - The requirements are applicable only if UE is provided with the active periods of the neighbour cells configured for the measurements. - For earth fixed cell and earth moving cell: - The existing values for Tdetect, Tmeasure, and Tevaluate in Table 4.6A.2.2-1 and Table 4.6A.2.2-2 can be applicable if it is confirmed that the current requirement is achievable without requiring more than one NPSS/NSSS sample for half the DRX cycle length [due to the TDD pattern] - Further discuss the values if -1dB will be used. Ksatellite = 1 for the cells measured in DL opportunities of a neighbour satellite that are in non-overlapping with DL opportunities to be measured in the serving satellite

In our understanding, the UE can calculate the difference in timing between a serving and a neighbouring satellite based on the satellite ephemeris. This means that the UE would only need to know an timing offset of the active periods of the neighbouring cell. If this offset can be assumed to be zero, then the satellite ephemeris would only be required. However, RAN4 has yet to send RAN2 an LS despite 2 meetings have passed, so we think that RAN2 shall wait for RAN4 before doing any work.

Proposal 3: RAN2 to wait for LS from RAN4 on providing UE with active periods of neighbouring cells.

# Conclusion

In this contribution, we provided some proposals on IoT NTN TDD:

Proposal 1: Change the value range of k-Mac-r19 to 513...1024.

Proposal 2: RAN2 does not specify any enhancements related to GNSS measurement gaps for IoT NTN TDD.

Proposal 3: RAN2 to wait for LS from RAN4 on providing UE with active periods of neighbouring cells.

# Reference

RP-243293, Revised WID for introduction of IoT NTN TDD mode, Iridium Satellite LLC, RAN#106, Madrid, Spain, December 2024.