3GPP TSG-RAN WG2 Meeting #131	R2-2505922

Bengaluru, India, 25th – 29th Aug, 2025

Agenda item:	8.8.3

Source:	Samsung

Title:	On OCC applicability to RACH-less handovers

WID/SID:	NR_NTN_Ph3-Core

Document for:	Discussion and Decision

# Introduction

In RAN#106 the latest NR-part of the WID NR NTN Phase in Rel-19 was agreed [1] where the relevant parts to this contribution are:

2. Uplink Capacity/Cell Throughput Enhancement for FR1-NTN [RAN1, RAN2, RAN4] - Specify Orthogonal Cover Codes (OCC) for DFT-s-OFDM PUSCH at least for multiplexing 2 or 4 UEs when PUSCH repetitions are used - Specify necessary signalling - Update RF requirements accordingly, if needed - Notes for this objective: - The enhancement is not targeting improvements/impacts of MU-MIMO capability - The enhancement is not targeted to PUSCH DMRS - No enhancement for initial access - Enhancements to PRACH are not in scope. - This feature may be applicable for UEs operating in terrestrial networks based on a common design

In this contribution, we provide a discussion on RAN2 aspects of uplink capacity/cell throughout enhancement for FR1-NTN.

# Discussion

## OCC for RACH-less handovers

As is made clear from the WID and the agreements, the enhancement targets PUSCH. One aspect where whether OCC is applicable that has yet to be agreed is whether OCC can apply to PUSCH transmissions during RACH-less handovers. We think that allowing for OCC to be applied during PUSCH transmission for RACH-less can be beneficial, especially in a non-terrestrial network where system-wide there is expected to be a lot of handovers being performed, thus it is important to ensure that the handovers are performed efficiently. In fact this in our view is one of the key argument for introducing RACH-less handovers for NTN Release 18.

Some relevant RAN1 agreements are the following:

Agreement

OCC length and OCC sequence for OCC with CG PUSCH Type 1 are configured by RRC higher-layers.

- Up to RAN2 whether to signal this with one or two RRC parameters.

- Note: OCC lengths and sequences to be provided in the table of RRC parameters to be prepared by the WI rapporteur.

In the last meeting the following was discussed:

R2-2504352	On RAN2 aspects for Uplink OCC	Samsung	discussion	Rel-19	NR_NTN_Ph3-Core

Observation 1: Applying OCC for RACH-less handovers can make the handovers more efficient.

Proposal 2: RAN2 to confirm OCC is applicable for RACH-less handovers.

-	vivo thinks this is up to RAN1 to decide

-	QC thinks this is applicable and there is no need to ask RAN1. CATT agrees and we can leave this to NW configuration

Can come back to see whether we need to clarify anything for this in the specs

Currently, the OCC configuration has been implemented in the running RRC CR as part of the rrc-ConfiguredUplinkGrant in ConfiguredGrantConfig [2]. This means that it may in principle be applicable to RACH-less handovers by setting the cg-RRC-Configuration and the related fields in rach-LessHO in ReconfigurationWithSync. Therefore we believe that RAN2 can confirm that OCC configured for Type 1 CG is applicable to CG RACH-less. We think that this shall be confirmed as RAN1 have not discussed it, neither have an agreement confirming this.

Proposal 1: RAN2 to confirm OCC is applicable for CG RACH-less handovers by suitable network configuration.

In order to indicate this possibility to an implementer, we believe that it can be useful to document it in 38.300. An example of this can be found in Annex A.

Proposal 2: RAN2 to consider documenting OCC being applicable for RACH-less handovers in Stage 2 as seen in Annex A.

# Conclusion

In this contribution we discussed issues related to uplink capacity/cell throughput enhancements.

Proposal 1: RAN2 to confirm OCC is applicable for RACH-less handovers by suitable network implementation.

Proposal 2: RAN2 to consider documenting OCC being applicable for RACH-less handovers in Stage 2 as seen in Annex A.

# Reference

RP-243300, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT, RAN#106, Madrid, Spain, December 2024.

R2-250xxxx, Introduction of NTN Phase 3 enhancements, Ericsson, RAN2#131, Bengaluru, India, August 2025.

R2-250xxxx, Stage 2 running CR for NR NTN phase 3, Thales, RAN2#131, Bengaluru, India, August 2025.

# Annex A – 38.300 TP

-------------- Based on 38.300 running CR implementation [3] --------------

### 16.14.X	Support for NR NTN capacity enhancements

To improve uplink capacity in NTN, the following enhancements are supported for PUSCH transmission [56]:

- Inter-slot OCC with OCC length 2 to multiplex up to two UEs

- Inter-slot OCC with OCC length 4 to multiplex up to four UEs

OCC is applicable to RACH-less handovers with configured UL grant, as described in clause 9.2.3.6.

-------------- Based on 38.300 running CR implementation [3] --------------