# 3GPP TSG-RAN WG2 Meeting #131		                                                      R2-2505944

# Bangalore, India, 25th – 29th Aug. 2025

# Agenda item:	8.5.3

# Source:	CMCC

# Title:	Discussion on open issues of OD-SIB1

# Document for:	Discussion, Decision

- Introduction

Following are the open issues identified in the email discussion of RRC running CR[1]:

Open issue list: OD-SIB1: Issue 1: Q12 Issue 2: Vivo002

In this paper, we further discuss open issues related to OD-SIB1.

- Discussion

  - Validity of si-BroadcastStatus

Q9 is about the validity of si-BroadcastStatus:

si-BroadcastStatus Indicates if the SI message is being broadcasted or not. Change of si-BroadcastStatus should not result in system information change notifications in Short Message transmitted with P-RNTI over DCI (see clause 6.5). The value of the indication is valid until the end of the BCCH modification period when set to broadcasting. When SIB19 is scheduled in an NTN cell, the si-BroadcastStatus for the mapped SIB19 is set to broadcasting. When SIB22 is scheduled in an ATG cell, the si-broadcastStatus for the mapped SIB22 is set to broadcasting. FFS: how to capture that a CONNECTED MODE UE supporting OD-SIB1 who is in a cell that does not broadcast SIB1, understands that the stored SIB1 is the latest SIB1. E.g. “The UE supporting OD-SIB1 in RRC_CONNECTED considers the stored SIB1 as the latest SIB1.” Q12: Can “The UE supporting OD-SIB1 in RRC_CONNECTED considers the stored SIB1 as the latest SIB1” be added to the field description of si-BroadcastStatus to address the FFS? Conclusion: FFS remains as open issue

In our opinion, the intention of this open issue is to clarify that the IE si-BroadcastStatus included in SIB1 acquired by on-demand request is still valid, or latest.  It’s fine to add such description. However, the description proposed by rapporteur is not straightforward enough, therefore, we prefer to clarify in the new added description that the IE si-BroadcastStatus in the stored SIB1 is the latest.

Proposal 1: Add description to clarify that the UE supporting OD-SIB1 in RRC_CONNECTED considers IE si-BroadcastStatus in the stored SIB1 is the latest.

  - OD-SIB1 acquisition in NES cell

Issue 2 is about OD-SIB1 acquisition in NES cell:

| vivo002 | 5.2.2.3.1 Acquisition of MIB and SIB1 2> else if SIB1 acquisition is required for the UE and ssb-SubcarrierOffset indicates that SIB1 is not scheduled in the cell: 3> if the UE has a stored valid version of od-SIB1-Config for this cell: 4> if the UE is in RRC_IDLE or in RRC_INACTIVE; or 4> if the UE is in RRC_CONNECTED while T311 is running: 5> perform the actions as specified in clause 5.2.2.3.3x; 3> else: 4> perform the actions as specified in clause 5.2.2.5. NOTE 1: The UE in RRC_CONNECTED is only required to acquire broadcasted SIB1 if the UE can acquire it without disrupting unicast or MBS multicast data reception, i.e., the broadcast and unicast/MBS multicast beams are quasi co-located. The UE in RRC_INACTIVE state while T319a is running, is only required to acquire broadcasted SIB1 and MIB if the UE can acquire them without disrupting unicast data reception, i.e. the broadcast and unicast beams are quasi co-located. NOTE 2: UE in RRC_INACTIVE that does not support inactiveStateNTN-r17 enters RRC_IDLE upon cell reselection between TN cell and NTN cell, and initiates the NAS signalling connection recovery (see TS 24.501 [23]). NOTE x: It is up to UE’s implementation to check if SIB1 is currently being broadcasted for that cell before triggering OD-SIB1 request procedure of that cell. [comment] During last meeting, at offline session [AT130][104][NES] (Ericsson), P6 in R2-2504037 about the UE behavior after receiving SI change notifcation was discussed. Rapporteur responded that a condition was added to address the issue: 5> if SIB1 is not broadcasted 6> perform the actions as specified in clause 5.2.2.3.3x; And therefore, at the Comeback session, it was first agreed that: Keep RRC CR as it is for the short message and UE behaviour.. Then, it is commented that the high-lighted part mandates the UEs to check whether SIB1 is broadcasting before requesting OD-SIB1 (not the case upon receiving SI change notification), which is against the RAN1 agreement that it is up to the UE’s implementation to check if SIB1 is broadcasting. Thus, the high-lighted part was removed, and instead the NOTE x is added. However, RAN2#126 agrees that: Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.. Therefore, we encourage RAN2 to review the agreement again ‘Keep RRC CR as it is for the short message and UE behaviour.’, whether the NOTE x can cover the above RAN2#126 agreement. The discussion is comprised of the following questions: Q1. For RAN2#126 agreement ‘ Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.’, does it mean that the UE always assumes the NES cell will turn from transmitting NCD-SSB to transmitting CD-SSB for SIB1 update? [HONOR] For Q1, we understand that this agreement simply shows the UE can acquire the updated SIB1 from the NES cell without returning to Cell A. It does not imply that the NES cell will immediately transmit CD-SSB for SIB1 update upon broadcasting a SIB change notification (e.g., at the start of the next MP). Q2. If the answer is ‘NO’ for Q1, i.e. the NES cell can remain transmitting NCD-SSB and at the same time transmit the updated SIB1, how does the UE camping on the NES cell to receive the updated SIB1? Shall we rely on the NOTE x to say it’s up to UE’s implementation to handle this? [HONOR] For Q2, we think the UE camping on the NES cell can initiate OD-SIB1 request based on the WUS configuration to acquire the updated SIB1. We think for Q2, if the CORESET#0 in the stored OD-SIB1 request configuration has changed, the UE camping on the NES cell will have no way by implementation to receive the updated SIB1. As a result, all the UEs camping on the NES cell will trigger OD-SIB1 request since SIB1 cannot be received, which is not correct. Therefore, we propose Q1/Q2 to be listed as open issues for further discussion. [HONOR] Same view as Vivo, i.e., add Q1, Q2 and the issue of WUS configuration changes caused by CORESET#0 modifications to open issues for further study. | Listed as open issue as requested since it can be seen there would be conflicting agreements. |
| --- | --- | --- |

For Q1 listed here is how to understand the agreements made in RAN2#126[2]:

Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.

In our opinion, this agreement is to clarify that the UE can acquire SIB1 and updated other SIBs from its camping NES cell, but not to reselect to Cell A. And since the UE can camp on the NES cell, it means SIB1 and WUS configuration stored in the UE is valid. While with valid WUS configuration, the SSB transmitted in the NES cell can be considered as CD-SSB  even OD-SIB1 transmission is ended, since the UE can acquire SIB1 in the cell if needed. What’s more, considering the UE is camping on NES Cell, we think the UE can acquire updated WUS configuration and to acquire OD-SIB1,  and RAN2 also made related agreements in RAN2#127 meeting: Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures [3].

Proposal 2: The agreement in RAN2#126 is to clarify that the UE can acquire SIB1 and updated other SIBs from its camping NES cell, but not to reselect to Cell A.

Proposal 3: The SSB transmitted in NES cell can be considered as CD-SSB if the UE has valid WUS configuration, even OD-SIB1 transmission is ended.

Proposal 4: The UE camping on a NES cell means it has latest WUS configuration and SIB1, else it cannot camp on the cell.

  - SUL on NES Cell

SUL is used to improve UL coverage for high frequency scenarios. And considering the potential commercial deployment of NES, NES cell may be deployed on high frequency, therefore, SUL can be configure for NES cell for UL coverage improvement.

Proposal 5: SUL can be configured in NES cell supporting OD-SIB1.

In legacy OD-SI, it can be performed on NUL or SUL. Similarly, it can be considered to support OD-SIB1 request on SUL, and if supported, the related information about SUL should be included in WUS configuration.

Proposal 6: OD-SIB1 request on SUL can be supported, and SUL related information should be included in WUS configuration to achieve this.

- Conclusion

In this contribution, we discuss configuration/indication of OD-SSB transmission and measurement related issues. Following are the proposals made in this contribution:

Proposals:

Proposal 1: Add description to clarify that the UE supporting OD-SIB1 in RRC_CONNECTED considers IE si-BroadcastStatus in the stored SIB1 is the latest.

Proposal 2: The agreement in RAN2#126 is to clarify that the UE can acquire SIB1 and updated other SIBs from its camping NES cell, but not to reselect to Cell A.

Proposal 3：The SSB transmitted in NES cell can be considered as CD-SSB if the UE has valid WUS configuration, even OD-SIB1 transmission is ended.

Proposal 4: The UE camping on a NES cell means it has latest WUS configuration and SIB1, else it cannot camp on the cell.

Proposal 5: SUL can be configured in NES cell supporting OD-SIB1.

Proposal 6: OD-SIB1 request on SUL can be supported, and SUL related information should be included in WUS configuration to achieve this.

- References

- Report of [POST130][107][NES]: Collection of comments to 38.331 CR for NES (Ericsson);

- Report of 3GPP TSG RAN WG2 meeting #126;

- Report of 3GPP TSG RAN WG2 meeting #127.