3GPP TSG-RAN WG2 Meeting #131                               	                                R2-2505955

Bangalore, India  Aug 25th – 29th , 2025

Agenda Item:	8.7.5

Source: 	CMCC

Title:  	Discussion on the open issue of RLC enhancements

WID/SID:	          NR_XR_Ph3-Core

Document for:	Discussion and Decision

Introduction

In the last RAN2 #130 meeting, the RLC enhancements has been discussed and the following related agreements have been approved.

Agreements on RLC timely retransmissions - (RLC-8) We just keep the current specifications for polling triggering, i.e. no need to specify that polling shall only be triggered once per RLC SDU when its remaining time falls below a specified threshold, unless an issue is identified with this - (RLC-9) No additional conditions are needed for the polling enhancement. - (RLC-7) RAN2 agrees that autonomous retransmissions should trigger an increment of the RETX_COUNT. - (PDCP-2) The UE shall trigger remaining-time-based RLC retransmission and polling in PDCP SDU level, no matter pdu-SetDiscard is configured or not. No specification change is needed. - (RRC-3, RRC-4) Autonomous retransmission and polling is triggered only based on discardTimer, i.e. not based on discardTimerForLowImprotance - (RRC-8) The remaining time thresholds (for both autonomous retransmission and polling) are configured per PDCP entity. Agreements for unnecessary retransmission avoidance in RLC - (RLC-6) There is no specification impact foreseen for RLF triggering due to RLC maximum retransmission. - Working assumption: (RLC-11) No need to address window stalling issue with polling retransmission (TBC next meeting) - Assumption from companies is that this should not happen due to SR triggering from Rx side, but it needs to be checked whether this will always work - (UE capability-2) Define an (optional) per-UE capability with signalling for the Rx-side aspect, where an outdated SDU is abandoned based on a new RLC timer and the abandoned SDUs are positively acknowledged in an RLC status report. - (UE capability-2) Define an (optional) per-UE capability with signalling for the Tx-side aspect, where the Tx side stops transmissions for an outdated SDU based on an indication from the PDCP. FFS A UE supporting this feature shall also indicate the support of Rx-side aspect.

In this contribution, we provide our views on the open issue listed in [1] for RLC enhancements.

Discussion

Issue RLC-10: Whether/How to introduce any limitations or associations for the configuration of stopReTxObsoleteSDU and DL t-RxDiscard?

In the last meeting, UE capabilities for RLC Tx and Rx side aspect were defined separately.  One FFS was listed that a UE supporting the Tx side feature shall also indicate the support of Rx-side aspect. If stopReTxObsoleteSDU in Tx side is configured, the Tx side stops the retransmissions for an outdated SDU based on an indication from the PDCP.  And if the t-RxDiscard is not configured in Rx side, the missing outdated SDU or segments will not be recognized as abandoned and no positively acknowledged for the outdated SDU in an RLC status report is sent from Rx side. Therefore this outdated SDU in the Tx side could not be discarded due to no ACK was received from ST and the transmission window is stalling accordingly.  The configuration of stopReTxObsoleteSDU in Tx side should be associated with the configuration t-RxDiscard in Rx side to avoid the transmission window stalling, which should be specified explicitly.

Proposal 1: The configuration of stopReTxObsoleteSDU in Tx side should be associated with the configuration t-RxDiscard in Rx side to avoid the transmission window stalling, which should be specified explicitly.

Issue RLC-11: FFS how to handle the case “if there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, there is no SDU to retransmit the poll with”

RAN2 agreement in #130 meeting: Working assumption: (RLC-11) No need to address window stalling issue with polling retransmission (TBC next meeting) - Assumption from companies is that this should not happen due to SR triggering from Rx side, but it needs to be checked whether this will always work

Since the only buffered SDU which is indicated to be obsolete by the PDCP, the polling retransmission could not be included in this discarded AMD PDU. If the transmission window is not stalling, the polling may not be needed since the Status Report is sent from receiving side when the t-RxDiscard expires. If the transmission window is stalling, the polling may be included as the case of expiry of t-PollRetransmit or still could wait for the the Status Report sent from receiving side when the t-RxDiscard expires. Therefore, it is not necessary to address window stalling issue with polling retransmission.

Proposal 2: It is not necessary to address window stalling issue with polling retransmission.

Conclusion

In this contribution, we discussed the RLC enhancements related issues. We propose:

Proposal 1: The configuration of stopReTxObsoleteSDU in Tx side should be associated with the configuration t-RxDiscard in Rx side to avoid the transmission window stalling, which should be specified explicitly.

Proposal 2: It is not necessary to address window stalling issue with polling retransmission.

Reference

- R2-25xxxx_Summary of RLC open issue list for R19 XR, vivo

- 3GPP TS 38.323 V18.5.0 (2025-03), Packet Data Convergence Protocol (PDCP) specification(Release 18)

- 3GPP TS 38.322 V18.2.0 (2024-12), Radio Link Control (RLC) protocol specification (Release 18).