3GPP TSG RAN WG2 Meeting #131	R2-2505960

Bangalore, India, August 25th – 29th, 2025

Agenda Item:	8.17

Source:	CMCC

Title:	Remaining issues on support of IoT-NTN TDD mode

Document for:	Discussion, Decision

Introduction

In last RAN2 meeting, there were some discussions on support of IoT-NTN TDD mode and some agreements were achieved as follows [1]:

Agreements:

1.	The SI-message transmission can be postponed to the next valid D frame within the SI-Window

2.	It is up to NW implementation to avoid SI-window overlap

3.	SI repetitions will not overlap (in case of collision the subsequent SI repetition is postponed)

4.	In IoT-NTN TDD mode, the RA-RNTI should be calculated based on the SFN of the first radio frame in which the Random-Access Preamble is transmitted (i.e. no spec change)

5.	For the timer of ra-ResponseWindowSize and mac-ContentionResolutionTimer, the absolute value limitation for FDD (i.e., 10.24s) is used for IoT NTN TDD.

6.	In IoT-NTN TDD mode the same formula as for RA-RNTI calculation for FDD is reused

7.	No extension is needed on the value range of timer in unit of ms or s for IoT NTN TDD

8.	The remaining paging repetitions falling on the invalid DL SFNs are postponed to the next valid DL SFNs.

9.	It is up to network to configure the gap between two POs (i.e., parameter NB) to be sufficiently long such that it includes enough number of valid DL subframes for NumRepetitionPaging-r13 (no spec impact)

10.	Introduce the following definition for IoT-NTN TDD mode in the impacted RAN2 specifications:
IoT-NTN TDD mode: allows use of NB-IoT channels with TDD mode for NTN with fixed values of D non-overlapping usable contiguous DL subframes and set of U usable contiguous UL subframes separated by fixed guard period (can revisit this based on the TP being prepared by RAN1)

11.	In Rel19, RAN2 will not work on any specific enhancements to ensure that the features being specified in IoT_NTN_Ph3-Core will also work for IoT NTN TDD mode

12. In IoT_NTN_Ph3-Core, RAN2 will not work on any specific enhancements to ensure that the features being specified in IoT_NTN_Ph3-Core will also work for IoT NTN TDD mode. RAN2 understands that, as part of the IoT_NTN_TDD WI, we can discuss on a case by case basis whether minor specific enhancements – not affecting other WGs - can be supported to ensure that (some of) the features being specified in IoT_NTN_Ph3-Core will also work for IoT NTN TDD mode

According to RAN1 discussion progress, the latest agreements are reached as following[2]:

Agreement NPRACH periodicities of 40ms and 80ms are not supported in NB-IoT NTN TDD mode - Instead, periodicities of 90ms and 180ms are supported Agreement For GNSS measurement gap: - GNSS measurement gap can be applied in NB-IoT NTN TDD - RAN1 does not specify any enhancement on the Rel-18 timeline for the start of GNSS gap. Conclusion Additional SIB1-NB transmissions are supported based on current specifications without the need of indicating DL bitmap. - NOTE: RAN1 assumes that all NTN-IoT TDD UEs can interpret additionalTransmissionSIB1-r15 in MIB Agreement Confirm the working assumption of RAN1#120b with the following modifications DL bitmap (downlinkBitmap / downlinkBitmapNonAnchor) does not apply to a NB-IoT NTN TDD cell. - NOTE1: All D NB-IoT subframes (except those carrying NPSS/NSSS/SIB1-NB/NPBCH) are NB-IoT DL subframes. - NOTE2: “D NB-IoT subframes” are subframes contained within the set of D=8 usable consecutive downlink subframes in the TDD structure. - Up to RAN2 to decide whether DL bitmap (downlinkBitmap / downlinkBitmapNonAnchor) is constrained to not being present in a NB-IoT NTN TDD cell, or it can be configured with values all set to ‘1’. Agreement Confirm the following working assumption with modifications: For precompensation, from RAN1 perspective: - The UE may adjust its time/frequency pre-compensation before the beginning of each set of consecutive 8 uplink subframes. No pre-compensation gap is needed before the beginning of each set of consecutive 8 uplink subframes. - The UE may adjust its time/frequency pre-compensation at the beginning of an NPUSCH/NPRACH transmission (same behavior as Rel-18) - Segmented precompensation is not supported. - It is not supported to perform precompensation within the set of 8 consecutive uplink subframes other than at the beginning of an NPUSCH/NPRACH transmission - FFS: whether spec impact is in RAN1, RAN4 or both. NOTE: RAN1 may revisit this agreement if RAN4 reply LS shows concerns, including concerns on meeting the requirements without segmented precompensation Agreement For the issue of handling DL gaps in NB-IoT NTN TDD mode: - DL gaps apply based on current specifications, with the following modification: - dl-GapPeriodicity with a value of “sf1024” is supported instead of the value of “sf64”

In this contribution, we would like to provide some further considerations on the remaining issues for IoT-NTN TDD mode.

Discussion

Open issue CAP-1: IoT-NTN TDD mode capability type

As in the RAN1 LS [3], it indicates that IoT-NTN TDD mode capability type is optional without capability signalling. And there is also a note in the LS that UE supporting band 249 must support this FG. Considering that IoT-NTN TDD is only supported in band n249, which means the UE supports band 249 and IoT-NTN TDD is a binding relationship. Therefore it is proposed to support the IoT-NTN TDD mode with a conditionally mandatory capability type as in the latest 36.306 running CR.

Proposal 1: (CAP-1) It is proposed to support the IoT-NTN TDD mode with a conditionally mandatory capability type as in the latest 36.306 running CR.

k-Mac extension

It is supported to extend k-Mac up to 1023ms for IoT-NTN TDD, and FFS extending this to IoT NTN and NR NTN. And there were some discussions in last meeting without convergence.

Proposal 1 For NR NTN, support k-Mac with a value range up to 1023 ms for Rel-19 - Xiaomi thinks we should think about backward compatibility issues - Samsung would support this if combined with the introduction of a new barring bit. ZTE tends to agree - QC would also like to understand which deployment would really need this Can come back to this in the next meeting

From our perspective, we need to limit the discussion to the IoT-NTN TDD, and at least for now, there is no need to extend the k-Mac for IoT NTN and NR NTN considering that we have considered both transparent and regenerative architecture related scenarios in R17 and R18 to define the value ranges of k-Mac.

Proposal 2: No need to extend the k-Mac for IoT NTN and NR NTN.

Start time of SI-window

As in the current TS 36.331, for NB-IoT UE, the related SI window determination is as following:

- 5.2.3a Acquisition of an SI message by BL UE or UE in CE or a NB-IoT UE When acquiring an SI message, the BL UE or UE in CE or NB-IoT UE shall: - determine the start of the SI-window for the concerned SI message as follows: 2> if the concerned SI message is configured in the schedulingInfoList, schedulingInfoListExt (if present) or if the concerned SI message is configured in the posSchedulingInfoList and si-posOffset is not configured; 3> for the concerned SI message, determine the number n which corresponds to the order of entry in the concatenated list of SI messages configured by schedulingInfoList, schedulingInfoListExt (if present) in SystemInformationBlockType1-BR (or SystemInformationBlockType1-NB in NB-IoT) and posSchedulingInfoList in SystemInformationBlockType1-BR; 3> determine the integer value x = (n – 1)*w, where w is the si-WindowLength-BR (or si-WindowLength in NB-IoT); 3> if the UE is a NB-IoT UE: 4> the SI-window starts at the subframe #0 in the radio frame for which (H-SFN * 1024 + SFN) mod T = FLOOR(x/10) + Offset, where T is the si-Periodicity of the concerned SI message and, Offset is the offset of the start of the SI-Window (si-RadioFrameOffset); - <<omitted>>

Regarding the starting point of the SI-window for the new TDD pattern with only 8 consecutive DL subframes in every 9 radio frames. Per the agreement of RAN1 #120, the downlink subframes within D=8 are fixed to [3 4 5 6 7 8 9 0] (across two consecutive radio frames), which would lead to that the actual calculated SI-window starting subframe #0 may not be used. And to minimize the specification impact, we could consider to postpone the start point of the SI-window to the next valid subframe.

Proposal 3: If the calculated subframe is invalid, the start time of the SI-window could be postponed to the next valid DL subframe.

Times with unit of pp (PDCCH periods)

In RAN2 129bis meeting, we agreed to reuse the existing value ranges of the timers in unit of pp. FFS on the possible clarification to take into account the impact of invalid subframes. Some companies indicate online that the invalid subframes is a general issue for NB-IoT, which is not a IoT-NTN TDD specific issue. However, compared to the new IoT-NTN TDD pattern, there might be much shorter/fewer invalid subframes for legacy NB-IoT. And maybe we could discuss to add some clarification to reduce the impact of invalid subframes. For example, add clarification that it is up to UE implementation to suspend the timers in invalid DL subframes duration and resume when valid subframes occur.

Observation 1: Compared to the new IoT-NTN TDD pattern, there might be much shorter/fewer invalid subframes for legacy NB-IoT.

Proposal 4: Kindly suggest RAN2 to add clarification that it is up to UE implementation to suspend the timers in invalid DL subframes duration and resume when valid subframes occur.

DL bitmap (downlinkBitmap / downlinkBitmapNonAnchor)

RAN1 has agreed that DL bitmap (downlinkBitmap / downlinkBitmapNonAnchor) does not apply to a NB-IoT NTN TDD cell. And up to RAN2 to decide whether DL bitmap (downlinkBitmap / downlinkBitmapNonAnchor) is constrained to not being present in a NB-IoT NTN TDD cell, or it can be configured with values all set to ‘1’. Some description for IoT-NTN TDD under the downlinkBitmapNonAnchor field has been added in the latest running CR [4], it seems acceptable and we could just do for this.

Proposal 5: Regarding the DL bitmap (downlinkBitmap / downlinkBitmapNonAnchor), the related modification in the latest running CR [4] could be supported.

HARQ

For HARQ of NB-IoT NTN FDD, in R18, we supported DL HARQ feedback disabled to support long propagation delay based on network configuration. For NB-IoT NTN TDD, considering the characteristic of the new TDD structure, for one HARQ process, the interval between a new transmission and a retransmission or two retransmission would be too long. That is a feedback used to determine whether a retransmission is needed may be not important. And we could consider an solution (e.g. add related description in TS 36.300) that if the NB-IoT NTN system operates in TDD mode, the HARQ feedback is disabled as default.

Proposal 6: It is proposed that if the NB-IoT NTN system operates in TDD mode, the HARQ feedback is disabled by default, and to be simple, we could just add related description in TS 36.300.

Measurement Gap

Based on the current TS 36.133, in E-UTRA-NR dual connectivity mode, NR - E-UTRA dual connectivity mode and E-UTRA standalone mode, all gap patterns #0~11 in Table 8.1.2.1-1 can be configured  for measurements of NR carriers only, and gap pattern#0, 1, 2, 3, 4, 6, 7, 8, 10 can be configured for measurements of E-UTRA carriers with the applicability as specified in Table 8.1.2.1-1.

Table 8.1.2.1-1: Gap Pattern Configurations supported by the UE

| Gap Pattern Id | MeasurementGap Length (MGL, ms) | Measurement Gap Repetition Period (MGRP, ms) | Minimum available time for inter-frequency and inter-RAT measurements during 480ms period (Tinter1, ms) | Measurement Purpose |
| --- | --- | --- | --- | --- |
| 0 | 6 | 40 | 60 | Inter-Frequency E-UTRAN FDD and TDD, UTRAN FDD, GERAN, LCR TDD, HRPD, CDMA2000 1x, inter-RAT NR |
| 1 | 6 | 80 | 30 | Inter-Frequency E-UTRAN FDD and TDD, UTRAN FDD, GERAN, LCR TDD, HRPD, CDMA2000 1x, inter-RAT NR |
| 2 | 3 | 40 | 24NOTE 1,2 | Inter-Frequency E-UTRAN FDD and TDD for cells with time difference as specified below. inter-RAT NR |
| 3 | 3 | 80 | 12NOTE 1,2 | Inter-Frequency E-UTRAN FDD and TDD for cells with time difference according as specified below. inter-RAT NR |
| 4 | 6 | 20 | 120 Note 1 | inter-RAT NR, Inter-Frequency E-UTRAN FDD and TDD Note 6 |
| 5 | 6 | 160 | Note 3 | inter-RAT NR |
| 6 | 4 | 20 | 72 Note 1, 5, 7 | inter-RAT NR, Inter-Frequency E-UTRAN FDD and TDD Note 6 |
| 7 | 4 | 40 | 36 Note 1, 5, 8 | inter-RAT NR, Inter-Frequency E-UTRAN FDD and TDD Note 6 |
| 8 | 4 | 80 | 18Note 1, 5, 9 | inter-RAT NR, Inter-Frequency E-UTRAN FDD and TDD Note 6 |
| 9 | 4 | 160 | Note 3 | inter-RAT NR |
| 10 | 3 | 20 | 48 Note 1, 5 | inter-RAT NR, Inter-Frequency E-UTRAN FDD and TDD Note 6 |
| 11 | 3 | 160 | Note 3 | inter-RAT NR |
| 24 | 10 | 80 | 54Note 1 | Inter-Frequency E-UTRAN FDD and TDD Note 11 |
| 25 | 20 | 160 | Note 3 | Note 11 |
| NOTE 1: When determing UE requirements using Tinter1 for GP2, 3, 4, 6, 7, 8, 10, Tinter1 = 60 for GP2, GP4, GP6, GP7, GP10 and Tinter1 = 30 for GP3, GP8 and GP24 shall be used. NOTE 2: Void. NOTE 3: This gap pattern can be used only for measurement of NR carriers, and Tinter is not applicable. NOTE 4: Void NOTE 5: Void. NOTE 6: This gap pattern is supported by UEs which are configured to perform both E-UTRA inter-frequency measurement and inter-RAT NR measurement or supported by UEs configured to perform inter-RAT NR measurement only. NOTE 7: When this gap pattern is used, the Tinter for E-UTRA interfrequency measurements is 48ms corresponding to the first 3ms of the 4ms gap NOTE 8: When this gap pattern is used, the Tinter for E-UTRA interfrequency measurements is 24ms corresponding to the first 3ms of the 4ms gap NOTE 9: When this gap pattern is used, the Tinter for E-UTRA interfrequency measurements is 12ms corresponding to the first 3ms of the 4ms gap NOTE 10: For UE only supporting measGapPatterns-NRonly-r16 for the corresponding gap patterns among GP2-11, the corresponding gap patterns are only applicable to measurement of inter-RAT NR. NOTE 11: Measurement gap patterns #24 and #25 can be requested by NR PCell [38] only when the UE is configured at least with any of RSTD, UE Rx-Tx, or PRS-RSRP measurements requiring such gaps and can only be used during the corresponding positioning measurement period |  |  |  |  |

From our perspective, the parameters MeasurementGap Length and measurement gap repetition period would be affected by the available DL subframes. And based on the current measurement gap configuration, the value range of MeasurementGap Length may be suitable to the new TDD pattern based on network configuration. While for the measurement gap repetition period, some new values could be considered based on the available DL subframes in one new TDD pattern period, for example, 90ms (i.e. corresponding to the period of the new TDD pattern) and multiples of 90ms. Therefore, measurement gap configuration enhancements could be discussed to match the new TDD pattern.

Proposal 7: It is proposed that measurement gap configuration enhancements (e.g introduction of new measurement gap repetition period values) could be discussed to match the new TDD pattern.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on IoT-NTN TDD mode and have the following observation and proposals:

Proposal 1: (CAP-1) It is proposed to support the IoT-NTN TDD mode with a conditionally mandatory capability type as in the latest 36.306 running CR.

Proposal 2: No need to extend the k-Mac for IoT NTN and NR NTN.

Proposal 3: If the calculated subframe is invalid, the start time of the SI-window could be postponed to the next valid DL subframe.

Observation 1: Compared to the new IoT-NTN TDD pattern, there might be much shorter/fewer invalid subframes for legacy NB-IoT.

Proposal 4: Kindly suggest RAN2 to add clarification that it is up to UE implementation to suspend the timers in invalid DL subframes duration and resume when valid subframes occur.

Proposal 5: Regarding the DL bitmap (downlinkBitmap / downlinkBitmapNonAnchor), the related modification in the latest running CR [4] could be supported.

Proposal 6: It is proposed that if the NB-IoT NTN system operates in TDD mode, the HARQ feedback is disabled by default, and to be simple, we could just add related description in TS 36.300.

Proposal 7: It is proposed that measurement gap configuration enhancements (e.g introduction of new measurement gap repetition period values) could be discussed to match the new TDD pattern.

Reference

- R2-130 NR-NTN-IoT-NTN (Sergio)_ EOM

- Chair notes RAN1#121 eom

- R1-2504676 Updated RAN1 UE features list for Rel-19 LTE after RAN1 #121

- R2-250xxxx RRC Runing CR for IoT-NTN TDD mode_v14_Rapp - clean