3GPP TSG-RAN WG2 Meeting #131	R2-2505963

# Bangalore, India, Aug 25th – 29th, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.314 | CR | 0035 | rev | 2 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introducing PDCCH CCE Usage for L2M [PDCCH_Usage] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC, Huawei, Ericsson, ZTE, CATT, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-08-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - PDCCH CCE Usage was specified in RAN2 to reflect the CCE usage with taking spatial multiplexing into consideration. But in the real network, the wireless environment is complicated and changes continuously. Configuring the same constant value Alpha at all times cannot reflect the CCE usage dynamically. - Since there are large number of NR base stations in the network and more base stations will be built in the future, configuring Alpha for each cell manually is difficult and time-consuming. Therefore, from the deployment and maintenance perspective, it is necessary to introduce a new CCE usage measurement with Alpha autonomously adjusted, e.g., based on statistical multi-UE spatial multiplexing. Corresponding to the constant value Alpha, the variable value can be named, e.g., β. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following indicators are introduced to evaluate PDCCH CCE Usage dynamically for statistical multi-UE spatial multiplexing: - Average value of multi-UE spatial multiplexing per CCE. - PDCCH CCE Usage based on statistical multi-UE spatial multiplexing per cell. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PDCCH CCE usage based on statistical multi-UE spatial multiplexing for L2M is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.1.10.x(New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

# 4	Layer 2 measurements

### 4.2.1	Measurements valid for all gNB deployment scenarios

<< Partially omitted >>

#### 4.2.1.10	PDCCH CCE Usage

<< Partially omitted >>

##### 4.2.1.10.x	PDCCH CCE Usage based on statistical multi-UE spatial multiplexing per cell

This measurement provides the total usage (in percentage) of PDCCH control-channel elements (CCEs) used for statistical multi-UE spatial multiplexing per cell. The objective of the measurement is to measure usage of time, frequency and space resources.

Protocol Layer: MAC, PHY

Table 4.2.1.10.x-1: Definition for PDCCH CCE Usage based on statistical multi-UE spatial multiplexing per cell

| Definition | PDCCH CCE Usage based on statistical multi-UE spatial multiplexing per cell is calculated in the time-frequency-space domain. Detailed Definition: MT1=∀i∀j{M1ij(T1)}∀j{Pj(T1)}*β*100 β=maxT2Lave(T), where explanations can be found in the table 4.2.1.10.x-2 below. |
| --- | --- |

Table 4.2.1.10.x-2: Parameter description for PDCCH CCE Usage based on statistical multi-UE spatial multiplexing per cell

| M(T1) | Total PDCCH CCE usage per cell which is percentage of CCEs, averaged during time period T1 with integer value. |
| --- | --- |
| M1ij(T1) | A count of PDCCH CCEs used for control information transmission for UE i per cell at sampling occasion j. Counting unit for CCE is 6 Resource Block x 1 symbol. (1 Resource Block = 12 sub-carrier) |
| i | A UE i that is scheduled during time period 𝑇1. |
| j | Sampling occasion during time period 𝑇1. A sampling occasion is 1 symbol. |
| Pj(T1) | Total number of PDCCH CCEs available for sampling occasion j per cell. |
| T1 | Time period during which the measurement is performed to calculate MT1, e.g., 15min, 1 hour, etc. |
| β | A variable factor for spatial multiplexing assigned with the maximum Lave(T) during time period 𝑇2 with float value 1.00-100.00. |
| Lave(T) | Average value of multi-UE spatial multiplexing per CCE on the DL during time period T with float value 1.00-100.00. The concrete calculation procedure can be found in the table 4.2.1.10.x-3 below. |
| T | Time period during which the measurement is performed to calculate Lave(T). |
| T2 | Time period during which the measurement is performed to calculate β, e.g., 1 week, etc. |

NOTE:	For this measurement, same β value is used for the entire duration of 𝑇1.

Table 4.2.1.10.x-3: Definition for average value of multi-UE spatial multiplexing per CCE

| Definition | Average value of multi-UE spatial multiplexing per CCE per cell is calculated in the time-frequency-space domain. Detailed Definition: Lave(T)=∀j∀kM1kjT*LkjT∀j∀kM1kjT, where explanations can be found in the table 4.2.1.10.x-4 below. |
| --- | --- |

Table 4.2.1.10.x-4: Parameter description for average value of multi-UE spatial multiplexing per CCE

| Lave(T) | The average value of multi-UE spatial multiplexing per CCE per cell with float value range: 1.00-100.00. |
| --- | --- |
| T | Time period during which the measurement is performed to calculate Lave(T), e.g., 15min, 1 hour, etc. |
| j | Sampling occasion during time period T. A sampling occasion is 1 symbol. |
| k | The number of kinds of spatial multiplexing. |
| Lkj(T) | The number of spatial multiplexing for PDCCH CCEs at sampling occasion j. |
| M1kj(T) | The number of PDCCH CCEs used for control information transmission corresponding to Lkj(T) at sampling occasion j. Counting unit for CCE is 6 Resource Block x 1 symbol. (1 Resource Block = 12 sub-carrier) |

## << End of change >>