3GPP TSG-RAN WG2 Meeting #131	R2-2505975

Bangalore, India  Aug 25th – 29th , 2025

Source:				ETRI

Title:					Discussion on UAI and Measurement Gaps

Agenda Item:		8.7.3 RRM measurement gaps/restrictions related enhancements

Document for:		Discussion and Decision

1. Introduction

During the discussion of measurement gap enhancements at the RAN2#130 meeting, agreements on UAI and prohibit timer have been made as the followings:

Agreements on measurement gap enhancements

- (RRC-05) UE reports the assistance information of recommended gap cancellation ratio when the configuration is received (and the UE has preference) or when the assistance information changes since the last report.

- (RRC-05) It is left to UE implementation to decide whether the recommended gap cancellation ratio changes.

- (RRC-06) A prohibit timer is used to limit frequent transmission of the UAI with recommended gap cancellation ratio. FFS the granularity of prohibit timer

- Consider RAN4 progress on the granularity for the recommended gap cancellation ratio when drafting the RRC CR

During the email discussion regarding the RRC running CR, there has been a divergence of views on how to configure and set the durations for prohibit timers for UAI transmissions. This document provides a discussion on the functionality and operation of these timers.

2. Discussion

Prohibit timer for UAI

UE may report its preferences for measurement gap occasion cancellation ratios via UAI. The frequency of these reports is controlled by a prohibit timer, which prevents the UE from sending a new UAI report for a specific duration after a previous one. A discussion has arisen regarding whether a single prohibit timer should be maintained for all measurement gap configurations or if multiple independent timers are needed.

Each measurement gap configuration may be controlled by its own prohibit timer. This approach is more flexible than relying on a single timer. For instance, after reporting a preferred ratio for one measurement gap configuration, a running prohibit timer should not prevent the UE from reporting a preferred ratio for another.  A single timer can also lead to inefficiencies, as it may force the UE to either report preferences for all configurations at once (causing unnecessary signaling overhead) or wait for the timer to expire, which introduces undesirable delays. By using independent timers, the UE can report only what is necessary, when it is necessary. This approach also aligns with existing UAI mechanisms for other purposes, such as overheating or power saving, where each preferred configuration is typically controlled by a separate prohibit timer. Following this established pattern provides a consistent and logical framework.

While using multiple timers might seem more complex, we do not expect UEs to be configured with a large number of measurement gap configurations. Therefore, allowing different timer values would not considerably increase the complexity of UE implementation. Furthermore, the network still has control and can configure a common value for all prohibit timers if a simplified approach is desired, offering flexibility without forcing a single-timer architecture.

Observation 1) A multiple prohibit timer approach, where one timer is assigned per gap configuration, enables more flexible and independent UAI transmission

Meanwhile, when a single prohibit timer is used across all measurement gap configurations, from the UE's perspective, a single timer is significantly easier to implement and maintain. This simplicity reduces the complexity in managing multiple timers simultaneously.

While it's true that a single timer could, in some cases, prevent a UE from reporting a change in preference for a different gap configuration while the timer is running, we consider this a manageable limitation. A properly implemented UE can be designed to determine if a change in preference is significant enough to warrant a new report, thereby avoiding this issue. Furthermore, a single timer provides the network with a more reliable way to control the frequency of UAI reports. With a single timer, the network can guarantee that the UE will comply with the maximum reporting frequency set by the gNB. If multiple independent timers were used, the UE might send several UAIs, contradicting the network's intent. To counter this, the gNB would be forced to set a longer timer value, which could be detrimental to the UE's ability to report a timely change in its preferences.

Observation 2) A single prohibit timer for all gap configurations offers a simple solution and allows the network to effectively control the frequency of UAI reports.

Proposal 1) When multiple gap configurations are provided, a single timer is maintained for all the gap configurations.

Prohibit timer associated with UAI reporting for gap occasion cancellation ratio preference may start when the UAI is transmitted. While the timer is running, the device is prohibited from sending new UAI messages to change its preference. This prevents excessive signalling and reduces network congestion. Once the timer expires, UE is then free to transmit a new UAI message if a change in preference is needed.

The prohibit timer is valid only if the measurement gap configuration is active. The measurement gap configuration should be confirmed following an RRC connection state change. Measurement gap skipping is performed to reduce the delay of delay-critical data for services like XR, ensuring faster transmission. The prohibit timer, which controls the transmission of UAI messages related to the operation of measurement gap skipping, is initialized during the RRC re-establishment procedure. It is appropriate for the prohibit timer to be active after the RRC re-establishment, in accordance with the gap configuration. These typical initialization procedures, where prohibit timers are reset at the start of RRC re-establishment and RRC resume procedures, are also applied to the prohibit timer that controls measurement gap skipping.

Proposal 2) The prohibit timer is cancelled when GapOccasionCancelRatioReportConfig is released.

Proposal 3) The prohibit timer is cancelled and the GapOccasionCancelRatioReportConfig is released when RRC re-establishment and RRC resume procedures are initiated.

3. Conclusion

In the discussion we made the following observations:

Observation 1) A multiple prohibit timer approach, where one timer is assigned per gap configuration, enables more flexible and independent UAI transmission

Observation 2) A single prohibit timer for all gap configurations offers a simple solution and allows the network to effectively control the frequency of UAI reports.

We propose the followings:

Proposal 1) When multiple gap configurations are provided, a single timer is maintained for all the gap configurations.

Proposal 2) The prohibit timer is cancelled when GapOccasionCancelRatioReportConfig is released.

Proposal 3) The prohibit timer is cancelled and the GapOccasionCancelRatioReportConfig is released when RRC re-establishment and RRC resume procedures are initiated.

4. References