3GPP TSG-RAN WG2 Meeting #131			         R2-2505976

Bangalore, India  Aug 25th – 29th , 2025

Agenda Item:	8.4.2

Source: 	LG Electronics Inc.

Title:         	Remaining issues on LP-WUS in RRC IDLE or INACTIVE

Document for: 	Discussion and Decision

Introduction

In this contribution, we will discuss following open issues on the LP-WUS operation in RRC_IDLE/INACTIVE:

- Open issue RRC-6 (essential): the value range of ThresholdPLP and ThresholdQLP for LR measurement based threshold

- Open issue RRC-12 whether/how to enable/disable LP-WUS, e.g. by RRC/NAS

- Open issue RRC-13 (essential): how to determine the cell quality for LR based measurement

- 38304-8: Whether LP-WUS is only used in the last used cell or in any cell.

- 38304-9: FFS the SubgroupID for LP-WUS used outside CN PTW in RRC_INACTIVE state with CN configured PTW.

- 38304-10: FFS whether/how LP-WUS with SDT is supported.

Discussion

Open issue RRC-6 (essential): the value range of ThresholdPLP and ThresholdQLP for LR measurement based threshold

Through email discussions, it has been confirmed that all companies agree, at least from the RAN2 perspective, that the same value range can be applied to both MR measurement thresholds and LR measurement thresholds. The only open issue is whether to proactively send an LS to RAN4 for confirmation, or to receive input from RAN4 later without sending an LS and reflect it during the maintenance stage.

Proposal 1	[OI RRC-6] Keep the value range of ThresholdPLP and ThresholdQLP in the current running CR, assuming that it will be revised later upon receiving input from RAN4.

Open issue RRC-12 whether/how to enable/disable LP-WUS, e.g. by RRC/NAS

It is also still FFS whether to support per UE de-activation of LP-WUS monitoring and we discussed two solutions for this issue:

- Solution 1. LP-WUS monitoring de-activation per UE via RRC release message

- Solution 2. LP-WUS monitoring de-activation per UE via NAS signalling

If the network can disable LP-WUS monitoring for a UE with a very high paging probability, it would help reduce false alarms. Alternatively, by preventing LP-WUS monitoring for a UE with an emergency PDU, it would be possible to avoid severe paging latency caused by LP-WUS. However, such NAS information is not available to the gNB.

If there is a need to support LP-WUS monitoring deactivation per UE for such reasons, it should be discussed and resolved in SA, not in RAN2. At the very least, since there is no need to deactivate LP-WUS monitoring per UE based on AS information, we propose not supporting LP-WUS monitoring deactivation per UE via RRC signaling.

Proposal 2	[OI RRC-12] Do not support LP-WUS monitoring de-activation per UE via RRC signalling.

Proposal 3	[OI RRC-12] Do not discuss whether to support LP-WUS monitoring de-activation per UE via NAS signalling. (out of RAN2 scope)

Open issue RRC-13 (essential): how to determine the cell quality for LR based measurement

UEs in RRC_IDLE/INACTIVE derive the cell quality in different ways depending on the procedure. For cell reselection in multi-beam operations, Qmeas,s/Rs is used as the metric representing serving cell quality and it is derived as the linear average of the power values of up to nrofSS-BlocksToAverage of highest beam measurement quantity values above absThreshSS-BlocksConsolidation, while for cell selection in multi-beam operations, Srxlev and Squal are used and which beam/how many beams are used to derive Srxlev and Squal is up to UE implementation.

In RAN2#129bis meeting, it was agreed to use Srxlev/Squal for all MR measurement based entry/exit condition evaluation whereas to use measured value for all LR measurement based entry/exit condition evaluation. However, it is still FFS how to derive the LR measurement quality, i.e. Qrxlevmeas _lr and Qqualmeas _lr, in multi-beam operations.

Cell selection aims to quickly find a cell on which the UE can camp. Immediately after cell selection, the UE starts evaluating for cell reselection to find and move to a cell with better quality. For this reason, cell selection is designed so that the UE can rapidly derive an approximate cell quality without complex configurations. However, it is questionable whether this approach is suitable for LP-WUS monitoring.

When the entry condition is met, the UE will not perform MR-based paging occasion monitoring until it detects the LP-WUS. In other words, the UE must not miss the LP-WUS once the entry condition is satisfied. Also, to maximize UE power saving, the exit condition should only be met when the UE truly fails to receive the LP-WUS correctly.

For the network to configure optimized entry and exit conditions, it must know how the UE derives MR/LR cell quality. The simplest approach would be for the UE to derive MR/LR-based cell quality using only the measurement results of the best beam. Alternatively, to support multi-beam-based cell quality, parameters such as absThreshSS-BlocksConsolidation and nrofSS-BlocksToAverage could be used.

Proposal 4	[OI RRC-13] Adopt one of the following two options

- Alt1. UE evaluates the MR/LR measurement-based entry/exit condition using only the best beam.

- Alt2. UE evaluates the MR/LR measurement-based entry/exit condition based on RRC configurations, i.e. absThreshSS-BlocksConsolidation and nrofSS-BlocksToAverage.

Open issue 38304-8: FFS Whether LP-WUS is only used in the last used cell or in any cell

The “only used in the last used cell” concept was introduced to prevent an increase in false alarms in a new cell after a UE monitoring PEI has moved cells. While LP-WUS can significantly reduce false alarms compared to PEI by supporting 32 subgroups, the same issue can still occur with LP-WUS. Since “only used in the last used cell” is already well specified for PEI, it can be easily applied to LP-WUS as well. Moreover, there is no technical reason why LP-WUS should not support this feature. Therefore, we propose that LP-WUS should also support the “only used in the last used cell”.

Proposal 5	[OI 38304-8] Support LP-WUS monitoring only in the last used cell.

Open issue 38304-9: FFS the SubgroupID for LP-WUS used outside CN PTW in RRC_INACTIVE state with CN configured PTW

In RRC_INACTIVE state with CN configured PTW, the SubgroupID (for PEI) used outside CN PTW is the same as the SubgroupID used inside CN PTW to ensure consistent identification of paging messages by the UE. This prevents confusion or misinterpretation of paging occasions whether they occur inside or outside the CN PTW, supporting reliable paging detection and handling. For the same reason, the SubgroupID for LP-WUS used outside CN PTW should be the same as the SubgroupID used inside CN PTW.

Proposal 6	[OI 38304-9] Capture in 7.y.2: In RRC_INACTIVE state with CN configured PTW, the SubgroupID for LP-WUS used outside CN PTW is the same as the SubgroupID used inside CN PTW.

Open issue 38304-10: FFS whether/how LP-WUS with SDT is supported

There is no technical blockage that would prevent performing MO/MT-SDT while monitoring LP-WUS. The currently running CR does not prohibit this either. Therefore, no CR change is required in this regard.

Proposal 7	[OI 38304-10] Confirm that UE can initiate MO/MT-SDT while monitoring LP-WUS. No CR change is needed.

Conclusion

Proposal 1	[OI RRC-6] Keep the value range of ThresholdPLP and ThresholdQLP in the current running CR, assuming that it will be revised later upon receiving input from RAN4.

Proposal 2	[OI RRC-12] Do not support LP-WUS monitoring de-activation per UE via RRC signalling.

Proposal 3	[OI RRC-12] Do not discuss whether to support LP-WUS monitoring de-activation per UE via NAS signalling. (out of RAN2 scope)

Proposal 4	[OI RRC-13] Adopt one of the following two options

- Alt1. UE evaluates the MR/LR measurement-based entry/exit condition using only the best beam.

- Alt2. UE evaluates the MR/LR measurement-based entry/exit condition based on RRC configurations, i.e. absThreshSS-BlocksConsolidation and nrofSS-BlocksToAverage.

Proposal 5	[OI 38304-8] Support LP-WUS monitoring only in the last used cell.

Proposal 6	[OI 38304-9] Capture in 7.y.2: In RRC_INACTIVE state with CN configured PTW, the SubgroupID for LP-WUS used outside CN PTW is the same as the SubgroupID used inside CN PTW.

Proposal 7	[OI 38304-10] Confirm that UE can initiate MO/MT-SDT while monitoring LP-WUS. No CR change is needed.