3GPP TSG-RAN WG2 Meeting #131                                    R2-2505983

Bengaluru, India, Aug. 25th– 29th, 2025

Agenda Item:	8.11.3

Source:	vivo

Title:	SBFD other aspects

Document for:	Discussion and Decision

Introduction

In this contribution, we will discuss other aspects of SBFD.

Discussion

Fallback from CFRA to CBRA

At last meeting, RO type determination when fallback from CFRA to CBRA was discussed [1] and the following agreement was achieved.

SBFD-aware UE uses the CBRA resource with same RO type as indicated in CFRA resource when fallback from CFRA to CBRA is performed, when the RACH resources for the same RO type is provided for CBRA.

However, if there is RO type change during the multiple PRACH attempts with CFRA RACH resources as shown in Figure 1, there may be different understanding of the meaning of ‘same RO type as indicated in CFRA resource’, one is the original RO type indication provided by network, and the other is the latest RO type for PRACH attempt with CFRA RACH resources.

Figure 1 RO type determination when fallback from CFRA to CBRA

To avoid frequently RO type modification, the latter understanding is more reasonable, and is also in line with the previous agreements achieved in RAN2#129.

RO-Type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols is supported when the number of PRACH transmission attempts exceed a threshold (we assume it is the same threshold with the fallback from additional RO to legacy RO). If fallback from legacy RO to additional RO occurs, no further fallback to legacy RO is supported.

Therefore, we propose that RAN2 confirm that SBFD-aware UE uses the CBRA resource with same RO type as the latest RO type used in CFRA resource when fallback from CFRA to CBRA is performed.

Proposal 1: RAN2 confirm that SBFD-aware UE uses the CBRA resource with same RO type as the latest RO type used in CFRA resource when fallback from CFRA to CBRA is performed, when the RACH resources for the same RO type is provided for CBRA.

Conclusion

Based on the discussion, we have the following proposals,

Proposal 1: RAN2 confirm that SBFD-aware UE uses the CBRA resource with same RO type as the latest RO type used in CFRA resource when fallback from CFRA to CBRA is performed, when the RACH resources for the same RO type is provided for CBRA.

References

- R2-2504397	Discussion on random access in SBFD	CMCC