3GPP RAN WG2 Meeting #131	R2-2505985

Bengaluru, India, Aug 25th – 29th 2025

Agenda Item:	8.8.2

Source:	InterDigital

Title:	Downlink coverage enhancement for NTN

Document for:	Discussion, Decision

# Introduction

In last RAN2 meeting (RAN2#130) [1], the fowlloing agreements and working assumption were made for the SMTC enhancements and the working assumption is to be confirmed in this meeting.

Agreements:

1.	the maximum configured SMTCs per frequency for idle/inactive UEs is 6 (can come back if we find and issue)

2.	To support SMTC enhancements for DL CE UE, introduce a new SMTC list instead of extending legacy smtc4list (can consider signalling optimizations for the new list to refer to the content of smtc4list to avoid signalling duplications)

3.	We introduce a location-based SMTC selection procedure where each SMTC can be associated with a reference location of the intended neighbor cells that need to be measured by the UE. FFS if also an SSB-index based SMTC selection is supported

4.	No enhancement for cell selection/reselection for DL CE is pursued (can re-discuss this if we will finally introduce a new barring scheme)

Working Assumption:

1.	We introduce a mechanism to assist the NW to configure the SMTCs in connected mode, according to one of the following 2 options:

Alt 1: UE provide the closest N reference locations/neighbor cells to network.

Alt 2: UE reports the selected SMTCs from configured SMTC set to network.

We continue in the next meeting on whether to confirm the WA (and go for either Alt1 or Alt2) or whether we don’t introduce any enhancement (i.e. not confirm the WA)

This contribution discusses about the working assumption.

# Discussion

The following offline discussion was held in the last RAN2 meeting [1].

[AT130][304][R19 NR NTN] DL CE (Xiaomi)

Then the offline discussion made the following proposal [2] but it was postponed to the next meeting.

Proposal 1: Under the WA to introduce a mechanism to assist the NW to configure the SMTCs in connected mode, RAN2 further down selects from the following 2 options:

- Alt 1: UE provide the closest N reference locations/neighbor cells to network.

- Alt 2: UE reports the selected SMTCs from configured SMTC set to network.

We continue in the next meeting on whether to confirm the WA (and go for either Alt1 or Alt2) or whether we don’t introduce any enhancement (i.e. not confirm the WA)

For idle mode, RAN2 made the following agreement.

- We introduce a location-based SMTC selection procedure where each SMTC can be associated with a reference location of the intended neighbor cells that need to be measured by the UE. FFS if also an SSB-index based SMTC selection is supported

So DL-CE capable UE would be capable of the location-based SMTC selection procedure. Thus, the location-based SMTC selection comes for free and so it makes sense to apply the same to connected mode operation unless there is any strong reason to have a different solution for connected mode. We don’t see any strong reason to justify any divergence from the idle mode for connected mode.

Observation 1: the same SMTC selection scheme can be applied for idle mode and connected mode.

For the connected mode operation, the difference from the idle mode operation is that the network needs to be aware of what SMTC is selected by the connected mode UEs as the network conducts mobility for the connected mode UEs.

Observation 2: For connected mode, the network needs to be aware of the selected SMTC by the UEs.

By seeing the observations, we think alt-2 makes more sense over alt-1.

Proposal 1: Confirm the WA of the SMTC selection in connected mode and conclude that UE reports the selected SMTCs from configured SMTC set to network.

# Conclusion

In this contribution the following observations and proposals are made regarding DL coverage enhancements:

Observation 1: the same SMTC selection scheme can be applied for idle mode and connected mode.

Observation 2: For connected mode, the network needs to be aware of the selected SMTC by the UEs.

Proposal 1: Confirm the WA of the SMTC selection in connected mode and conclude that UE reports the selected SMTCs from configured SMTC set to network.

# References

R2-2504673	Report from session on NR NTN and IoT NTN	Session chair (ZTE)

R2-2504775	Report of [AT130][304][R19 NR NTN] DL CE	Xiaomi	discussion