3GPP TSG-RAN WG2 Meeting #131											R2-2505989

Bengaluru, India, 25th– 29th August 2025

Agenda item:		8.5.2

Title: 	Remaining issues of on demand SSB SCell operation

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

- Introduction

In this paper, we share some understanding on the remaining open issues of on demand SSB SCell.

- Discussion

  - Maximum number of OD-SSB configuration per Cell

Since we have agreed that the OD-SSB MAC CE include a configuration index for each SCell activating OD-SSB, the maximum value of OD-SSB configuration for each SCell would impact the design of the MAC CE as it will impact the size of the configuration index.

Also considering that RAN1 would not discuss the detailed configuration and leave it to RAN2, we can make a decision on the maximum number of OD-SSB configuration for each SCell, e.g. 2 or 4.

Proposal 1: RAN2 make decision on the maximum number of OD-SSB configuration for each SCell, e.g. 2 or 4, which will decide the size of the configuration index in the on demand SSB MAC CE.

  - OD-SSB Activation/Deactivation MAC CE

The following MAC CE has been defined for on demand SSB Activation and Deactivation [1]:

-----------------------------38.321 CR---------------------------

The on-demand SSB Activation/Deactivation MAC CE of one Activation/Deactivation octet is identified by a MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a variable size; it includes an Activation/Deactivation bitmap and, in ascending order based on the SCellIndex, an on-demand SSB configuration index field for each SCell on which on-demand SSB is activated in the bitmap. A single octet bitmap is used for Activation/Deactivation of the on-demand SSB transmission and indicating the presence of an on-demand SSB configuration index per Serving Cell, containing seven C-fields and one R-field. The on-demand SSB Activation/Deactivation MAC CE of one Activation/Deactivation octet is defined as follows (Figure 6.1.3.x-1).

The on-demand SSB Activation/Deactivation MAC CE of four Activation/Deactivation octets is identified by a MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a variable size; it includes an Activation/Deactivation bitmap and, in ascending order based on the SCellIndex, an on-demand SSB configuration index field for each SCell on which on-demand SSB is activated in the bitmap. A four-octet bitmap is used for Activation/Deactivation of the pn-demand SSB transmission and indicating the presence of an on-demand SSB configuration index per Serving Cell, containing 31 C-fields and one R-field. The on-demand SSB Activation/Deactivation MAC CE of four Activation/Deactivation octets is defined as follows (Figure 6.1.3.x-2).

-	Ci: If there is an SCell configured for the MAC entity with SCellIndex i as specified in TS 38.331 [5], this field indicates the on-demand SSB activation/deactivation status for the SCell with SCellIndex i, else the MAC entity shall ignore the Ci field. The Ci field is set to 1 to indicate that on-demand SSB transmission on the SCell with SCellIndex i shall be activated with the corresponding on-demand SSB configuration index to be applied. The Ci field is set to 0 to indicate that on-demand SSB transmission on the SCell with SCellIndex i shall be deactivated;

-	ConfigIndexi: The on-demand SSB configuration index to apply for SCellIndex i. The octet containing this field is present for SCell with SCellIndex i if Ci is 1, and not present if Ci is 0.

-	R: Reserved bit, set to 0.

Figure 6.1.3.x-1: On-demand SSB Activation/Deactivation MAC CE of one Activation/Deactivation octet

Figure 6.1.3.x-2: On-demand SSB Activation/Deactivation MAC CE of four Activation/Deactivation octets

-----------------------------38.321 CR---------------------------

Based on the above structure, for example, if network want to active the following on demand SSB for each SCell:

| SCell Index | OD-SSB configuration index |
| --- | --- |
| 3 | 2 |
| 4 | 1 |

The activation MAC CE would look like the following:

| 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| OD-SSB configuration index: 2 |  |  |  | 0 | 0 | 0 | 0 |
| OD-SSB configuration index: 1 |  |  |  | 0 | 0 | 0 | 0 |

While for the other cases, e.g. NW would like to activate SSB configuration index 1 for multiple SCells:

| SCell Index | OD-SSB configuration index |
| --- | --- |
| 3 | 1 |
| 4 | 1 |
| 5 | 1 |

If we follow the existing structure, the activation MAC CE would like the following:

| 0 | 0 | 1 | 1 | 1 | 0 | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| OD-SSB configuration index: 1 |  |  |  | 0 | 0 | 0 | 0 |
| OD-SSB configuration index: 1 |  |  |  | 0 | 0 | 0 | 0 |
| OD-SSB configuration index: 1 |  |  |  | 0 | 0 | 0 | 0 |

With the above understanding, we are considering some signaling optimization for MAC CE. For example, when the following MAC CE is sent to UE, it means that SSB configuration index 1 is activated for SCell 3/4/5 individually:

| 0 | 0 | 1 | 1 | 1 | 0 | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| OD-SSB configuration index: 1 |  |  |  | 0 | 0 | 0 | 0 |
| OD-SSB configuration index: 1 |  |  |  | 0 | 0 | 0 | 0 |
| OD-SSB configuration index: 1 |  |  |  | 0 | 0 | 0 | 0 |

| 0 | 0 | 1 | 1 | 1 | 0 | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| OD-SSB configuration index: 1 |  |  |  | 0 | 0 | 0 | 0 |

Proposal 2: If all the SCells for which on demand SSB to be activated share the same OD-SSB configuration index, one single ConfigIndexi  can be included in the On demand SSB activation/deactivation MAC CE and applied to all the SCells with the Ci field set to 1

- Conclusion and proposals

Based on the analysis in previous sections, the following observations and proposals are given:

Proposal 1: RAN2 make decision on the maximum number of OD-SSB configuration for each SCell, e.g. 2 or 4, which will decide the size of the configuration index in the on demand SSB MAC CE.

Proposal 2: If all the SCells for which on demand SSB to be activated share the same OD-SSB configuration index, one single ConfigIndexi  can be included in the On demand SSB activation/deactivation MAC CE and applied to all the SCells with the Ci field set to 1

- Reference

[1] R2-250xxxx_Draft_TS38321_NES_RunningCR__v18.6_per meeting revision marks_v4_Rapp