3GPP TSG-RAN WG2 Meeting #131	R2-2505998

Bengaluru, India, August 25th – 29rd, 2025

Source:        ZTE Corporation

Title:           Consideration on the Remaining MAC Issues of UEIBM

Agenda item:   8.12.3

Document for:  Discussion and Decision

- Introduction

According to the post email discussion [Post130][218][MIMO_Ph5] Running CR for 38.331 (Ericsson) and [Post130][219][MIMO_Ph5] Running CR for 38.321(Samsung), there are still some remaining MAC issues on the UEIBM, In this paper we would like to share our views on these remaining issues.

- Discussion

Based the post email discussion, there are 3 main topics:

- Topic 1: How to capture multiplexing UCI of mode-A in PUSCH for R15 UL skipping

- Topic 2: Whether the Trigger RACH when there is no No PUCCH/PUSCH Resource (TAT expiry)

- Topic 3: Clarification for the UEIBM for the dormant BWP

In the next chapter we’d like to discuss these 3 topics one by one.

2.1 R15 UL Skipping Function

In the last meeting, an agreements has been made for the R15 UL skipping function as follow:

Agreements: For Rel-15 UL skipping (skipUplinkTxDynamic is configured), same principle as legacy aperiodic CSI applies for multiplexing UCI of mode-A DG-based UE-initiated report in PUSCH. Exact MAC spec can be discussed further.

Based on this agreements, an if condition was added together with an ED:

1> else if the MAC entity is configured with skipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or the grant indicated to the HARQ entity is a configured uplink grant: 2> if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and 2> if there is no mode-A UE-initiated CSI reporting triggered for this PUSCH transmission as specified in TS 38.214 [7]; and 2> if the MAC PDU includes zero MAC SDUs; and 2> if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR: 3> not generate a MAC PDU for the HARQ entity. Editor’s Note: FFS if the change is needed for the agreement: For Rel-15 UL skipping (skipUplinkTxDynamic is configured), same principle as legacy aperiodic CSI applies for multiplexing UCI of mode-A DG-based UE-initiated report in PUSCH.

As commented in the post email discussion, the Rel15 UCI multiplexing function is broken (and fixed from the R16), the background is that for the aperiodic CSI reporting, with the R15 UL skipping mechanism the network needs to decode the below 2 cases blindly：

- Case 1: MAC PDU + UCI (with aperiodic CSI)

- Case 2: UCI only（no aperiodic CSI）

To fix this issue, enhancedSkipUplinkTxDynamic was introduced in the R16. The similar view was also proposed in [1]. Thus the key issue is whether there is a need to further specify this broken function.

Proposal 1: RAN2 to confirm whether to further specify the broken R15 UL skipping function for the UEIBM.

Considering that the whole function is broken, and we have made the agreements in the chairman notes, there is no need to further specify it.

Proposal 1a: No need to further specify the broken R15 UL skipping function for the UEIBM.

2.2 Initiate RACH or not for TAT Expiry

On this topic, based on the companies’ inputs in the post email discussion, 2 options are selected.

Proposal 2: Regarding whether UE initiate RACH when UEI report is triggered but TAT is expired, select one of the following options: - Option 1: UE does not initiate RACH when UEI report is triggered but there is no PUCCH or type-1 CG due to the associated TAT is expired. No MAC impacts. - Option 3: UE initiates RACH when UEI report is triggered but there is no PUCCH or type-1 CG due to the associated TAT is expired, and UE indicates the cause of RACH. Discuss how to indicate the cause of RACH (e.g., MAC CE in Msg3).

The intention of the option 3 is to indicate the cause of RACH by the MAC CE. However, it will introduce much change on the RACH procedure, e.g. besides the new Msg 3, it also needs to introduce a new Msg4 for the reject case. The reason is that when the network receives a MAC CE with a cause in msg3 for this case, if the network doesn’t need the UE to report the UEIBR (e.g. the inactivity timer at the network side almost expiry and the network would release the connection in short time), the network needs to send an reject response to the UE. Otherwise, the UE would trigger the RACH again after the contention resolution timer expiry, and after the number of the preamble achieves the maximum, the UE MAC may indicate failure to the RRC layer, at last a re-establish procedure maybe triggered, which is quite unnecessary.

Based on the above discussion, we can see that this procedure is quite different from the other MAC initiated RACH procedures:

- 1: The network can reject this request, a reject MAC CE would be needed, and the UE can still keep at the current state upon receiving the reject, e.g. no need to inform upper layer for the reestablishment

- 2: If no rejection MAC CE designed, it means the rejection would be based on the contention resolution timer expiry, when the timer expiry, the UE may not need to trigger the preamble transmission again, otherwise a reestablishment procedure would be triggered unnecessarily.

Observation 1: For the UEIBM, the RACH procedure triggered by no PUCCH/PUSCH resource would differ significantly from the other MAC initiated RACH procedures, for example, it would require not only a new Msg3 but also a new Msg4 for rejection cases.

Furthermore, even the UE can indicate the cause, it still depends on the NW to determine whether to response this Msg3 or not. Thus it’s better to go with the option 1. With the option 1, the UE can initiate RACH when the UL data arrived or DL date arrived. For the TAT expiry indication to the PHY layer as some companies mentioned, it can be left to the UE implementation.

Proposal 2: UE does not initiate RACH when UEI report is triggered but there is no PUCCH or type-1 CG due to the associated TAT is expired. No MAC impacts.

2.3 UEIBR for the Dormant BWP

In the post email discussion [Post130][219][MIMO_Ph5] Running CR for 38.321(Samsung), many consensus on the MAC spec impact have been achieved for the MAC spec Impact. In this sub-chapter, we’d like to share our views on the dormant BWP.

For the dormant BWP, in the previous meetings, it has been agreed that

If the BWP in an SCell is a dormant BWP, the UE should not report mode-A beam measurement results. The UE cannot perform mode-B beam reporting on this BWP.

However it’s not clear on whether it’s allowed the UE to perform mode-B beam reporting for this BWP. There are 2 different understandings:

- Understanding 1: The UE can perform mode-B beam reporting for this BWP.

- Understanding 2: The UE can’t perform mode-B beam reporting for this BWP.

## 5.15.1 Downlink and Uplink 1> if a BWP is activated and the active DL BWP for the Serving Cell is dormant BWP: 2> stop the bwp-InactivityTimer of this Serving Cell, if running. 2> not monitor the PDCCH on the BWP; 2> not monitor the PDCCH for the BWP; 2> not receive DL-SCH on the BWP; 2> not report CSI on the BWP, report CSI except aperiodic CSI for the BWP;

If we take the legacy semi-persistent CSI report behaviour, it means the UE can report it for this dormant BWP, e.g. report it on the SpCell. However, for the UEI mode B CSI report for the dormant BWP, the question is how to determine the current beam. Even in the last RAN1 meeting a new parameter was introduced for some special cases, it’s still hard to determine the current beam for the dormant BWP when this new parameter is absent.

| CCofIndicatedTCI-r19 | New | This parameter is used to indicate a CC index of the serving cell on which the indicated TCI state used to determine the current beam RS is applied | ServCellIndex |
| --- | --- | --- | --- |

Based on the above analysis, the understanding 2 is preferred for the UEI mode B CSI report.

Proposal 3: When the BWP in a SCell is a dormant BWP, the UE should not perform UEI mode B CSI report for that BWP.

- Conclusion and proposals

With the above analysis, we have the following proposals:

Proposal 1: RAN2 to confirm whether to further specify the broken R15 UL skipping function for the UEIBM.

Proposal 1a: No need to further specify the broken R15 UL skipping function for the UEIBM.

Observation 1: For the UEIBM, the RACH procedure triggered by no PUCCH/PUSCH resource would differ significantly from the other MAC initiated RACH procedures, for example, it would require not only a new Msg3 but also a new Msg4 for rejection cases.

Proposal 2: UE does not initiate RACH when UEI report is triggered but there is no PUCCH or type-1 CG due to the associated TAT is expired. No MAC impacts.

Proposal 3: When the BWP in a SCell is a dormant BWP, the UE should not perform UEI mode B CSI report for that BWP.

- Reference

[1] R2-2503645  Clarification on the UL skipping for event-triggered beam reporting  OPPO