3GPP TSG-RAN WG2 Meeting #131	R2-2506015

Bengaluru, India, August, 25th-29rd, 2025

Agenda Item:	8.19.1

Source: 	CSCN, ZTE Corporation, Sanechips, Huawei, HiSilicon, CATT

Title: 		Discussion on measurement events for inter-frequency scenarios with overlapping coverage in NR NTN

Document for: 	Discussion

Introduction

In this TEI19 contribution, we propose a solution to a potential measurement issue identified in NR NTN inter-frequency scenarios with overlapping coverage.

Discussion

Location-based measurement events for NR NTN were introduced in Release 17 and Release 18. Due to the long distance and atmospheric attenuation between satellites and UEs, the near-far effect is not significant in NTN. Legacy measurement events based on RSRP are not suitable in such scenarios, so location-based measurement events were introduced and the location is defined by the distance between UE and a reference location. In Release 17, event/condEventD1 was introduced for the quasi-earth fixed cell scenario. In Release 18, event/condEventD2 was introduced for the earth-moving cell scenario. The definition of event/condEventD1 and event/condEventD2 are shown in the Appendix. The Figure 1 below shows the typical triggered conditions for eventD1 and eventD2: the green crescent-shaped section represents the area satisfying the entering condition, while the blue section represents the area satisfying the leaving condition.

Figure 1: the typical entering condition and leaving conditions for event D1/D2

So far, RAN2 has not yet discussed the inter-frequency overlapping coverage scenario in NR NTN (Figure 2). We should evaluate whether the existing D1/D2 solutions are applicable in this particular scenario. As shown in Figure 2, the network may need to trigger load balancing and consider switching UE C to Ncell2, even though UE C is located closer to the center of its serving cell compared to neighboring cells. To meet the entering condition in Figure 1, one possible solution is to configure the reference location to be far away to to prevent the UE from measuring the service cell; another possible solution is to set the distanceThreshFromReference1 to the minimum value (i.e., 50 meters) to prevent the UE from measuring the service cell.

Figure 2 Inter-frequency overlapping coverage

For the first possible solution, referenceLocation1 is associated with the serving cell and referenceLocation2 is associated with the neighbour cell for eventD1 [1], which means that referenceLocation1 can only be selected from points related to the serving cell. For eventD2, the reference location is determined based on the movingReferenceLocation and its corresponding satellite ephemeris and epoch time broadcast in SIB19. This means that the reference location point in D2 must be configured within the serving cell. Therefore, configuring the reference location of the serving cell far away to prevent the UE from measuring will not work well for inter-frequency scenarios with overlapping coverage.

For the second solution, the distanceThreshFromReference1 can be set to the minimum value (i.e., 50 meters) to prevent the UE from measuring the serving cell. For NTN, 50 meters is an extremely small range and can be almost ignored. However, to mitigate ping-pong effects in NTN deployment scenarios, the HysteresisLocation for measurement events is configured with a suitable value, such as in kilometers. This may result in some UEs within the area defined by referenceLocation1, distanceThreshFromReference1, and HysteresisLocation not triggering a measurement report, which could prevent them from switching to the neighboring cell and potentially affect the performance of load balancing.

In summary, the legacy event D1/D2 mechanisms are not suitable for inter-frequency overlapping coverage scenarios, particularly in the case of load balancing.

Observation 1: For the inter-frequency overlapping coverage scenario, legacy events D1/D2 are not suitable because serving cell-related configuration may hinder the UE from switching to a neighbor cell, especially for load balancing.

In order to reduce the impact of the serving cell parameters on neighbour cell measurements, new events/condEvents D3 and D4, aligned with event A4 in TN, can be defined:

Event D3: Distance between UE and referenceLocation becomes shorter than threshold;
Event D4: Distance between UE and moving reference Location becomes shorter than threshold.

These events utilize a single reference location and distanceThreshFromReference to define a circular area. The benefit of the new definition is that it is more straightforward and clearer, and the design also aligns with the legacy event A4. Regarding user consent, a UE can report its coarse UE location information with an accuracy of 2 km to the NG-RAN, so the minimum value of distanceThreshFromReference can be defined as 2 km. Here is an example of the event D3/D4 information element:

Event D3/D4 information element

eventD3-r19                                 SEQUENCE {

distanceThreshFromReference-r19             INTEGER(40.. 65535),

referenceLocation-r19                       ReferenceLocation-r17,

reportOnLeave-r19                           BOOLEAN,

hysteresisLocation-r19                      HysteresisLocation-r17,

timeToTrigger-r19                           TimeToTrigger

}

eventD4-r19                                 SEQUENCE {

distanceThreshFromReference-r19             INTEGER(40.. 65535),

reportOnLeave-r19                           BOOLEAN,

hysteresisLocation-r19                      HysteresisLocation-r17,

timeToTrigger-r19                           TimeToTrigger

}

Observation 2: The new event D3/D4 mechanism is more straightforward and clearly defined, and it aligns with the legacy event A4 in TN.

Another solution is to enhance the existing event D1/D2 with an implicit indication to ignore the reference location of the serving cell. According to the current specification, the minimum configuration value for the distance threshold in event D1/D2 is 1 (i.e., 50 meters), while the minimum value for condEvent D1/D2 is 0. These minimum value configurations are not commonly used in NTN. If the minimum value is configured, the reference location of the serving cell can be ignored. Therefore, the enhancement is to inform UEs with an implicit indication by modifying the field description of the event/condEvent D1 and D2 with a clarification. For example, add a descriptions for distanceThreshFromReference1 stating “In NTN scenarios, when distanceThreshFromReference1 for eventD1/D2 is configured as 1, or when distanceThreshFromReference1 for condEventD1/D2 is configured as 0, the UE invalidate serving cell related configuration information and triggers MR reporting based on the neighbour cell’s configuration only.” This serves as an implicit indication to inform the UE to ignore the reference location of the serving cell.

Proposal 1: RAN2 should consider defining new event/condEvent D3 and D4 or enhancing the existing event/condEvent D1 and D2 for inter-frequency overlapping coverage in NR NTN in R19:

- event/condEvent D3 and D4: utilize a single referenceLocation and distanceThreshFromReference, and align with event A4 in TN.

- event/condEvent D1 and D2 enhancement: inform UE with an implicit indication: “In NTN scenarios, when distanceThreshFromReference1 for eventD1/D2 is configured as 1, or when distanceThreshFromReference1 for condEventD1/D2 is configured as 0, the UE shall invalidate serving cell related configuration information and trigger MR reporting based on the neighbour cell’s configuration only.”

If RAN2 decides to introduce the enhancement of measurement events for inter-frequency overlapping coverage in R19, RAN2 should discuss whether it can also be applied in the early releases (i.e., R17 and R18).

Proposal 2: RAN2 should discuss whether it can also be applied in the early releases (i.e. R17 and R18).

Conclusions

Based on above analyses, observations and proposals are summarized as follows.

Observation 1: For the inter-frequency overlapping coverage scenario, legacy events D1/D2 are not suitable because serving cell-related configuration may hinder the UE from switching to a neighbor cell, especially for load balancing.

Observation 2: The new event D3/D4 mechanism is more straightforward and clearly defined, and it aligns with the legacy event A4 in TN.

Proposal 1: RAN2 should consider defining new event/condEvent D3 and D4 or enhancing the existing event/condEvent D1 and D2 for inter-frequency overlapping coverage in NR NTN in R19:

- event/condEvent D3 and D4: utilize a single referenceLocation and distanceThreshFromReference, and align with event A4 in TN.

- event/condEvent D1 and D2 enhancement: inform UE with an implicit indication: “In NTN scenarios, when distanceThreshFromReference1 for eventD1/D2 is configured as 1, or when distanceThreshFromReference1 for condEventD1/D2 is configured as 0, the UE shall invalidate serving cell related configuration information and trigger MR reporting based on the neighbour cell’s configuration only.”

Proposal 2: RAN2 should discuss whether it can also be applied in the early releases (i.e. R17 and R18).

Reference

[1] 38.331 “Radio Resource Control (RRC) protocol specification”, V18.6.0

Appendix

------------------------------- Example based on 38.331 V18.6.0 -------------------------------

5.1.1.1 ReportConfigNR

The IE ReportConfigNR specifies criteria for triggering of an NR measurement reporting event or of a CHO, CPA or CPC event or of an L2 U2N relay measurement reporting event. For events labelled AN with N equal to 1, 2 and so on, measurement reporting events and CHO, CPA or CPC events are based on cell measurement results, which can either be derived based on SS/PBCH block or CSI-RS.

Event A4:	Neighbour becomes better than absolute threshold;

Event A5:	PCell/PSCell becomes worse than absolute threshold1 AND Neighbour/SCell becomes better than another absolute threshold2;

Event D1:	Distance between UE and a reference location referenceLocation1 becomes larger than configured threshold distanceThreshFromReference1 and distance between UE and a reference location referenceLocation2 becomes shorter than configured threshold distanceThreshFromReference2;

Event D2:	Distance between UE and the serving cell moving reference location determined based on movingReferenceLocation and its corresponding satellite ephemeris and epoch time broadcast in SIB19 becomes larger than configured threshold distanceThreshFromReference1 and distance between UE and a moving reference location determined based on referenceLocation and its corresponding satellite ephemeris and epoch time for the neighbor cell provided in the associated MeasObjectNR becomes shorter than configured threshold distanceThreshFromReference2;

CondEvent D1: Distance between UE and a reference location referenceLocation1 becomes larger than configured threshold distanceThreshFromReference1 and distance between UE and a reference location referenceLocation2 of conditional reconfiguration candidate becomes shorter than configured threshold distanceThreshFromReference2;

CondEvent D2: Distance between UE and the serving cell moving reference location determined based on movingReferenceLocation and its corresponding satellite ephemeris and epoch time broadcast in SIB19 becomes larger than configured threshold distanceThreshFromReference1 and distance between UE and a moving reference location determined based on referenceLocation and its corresponding satellite ephemeris and epoch time for the conditional reconfiguration candidate provided in the associated MeasObjectNR becomes shorter than configured threshold distanceThreshFromReference2;

ReportConfigNR information element

-- ASN1START

-- TAG-REPORTCONFIGNR-START

ReportConfigNR ::=                          SEQUENCE {

reportType                                  CHOICE {

periodical                                  PeriodicalReportConfig,

eventTriggered                              EventTriggerConfig,

...,

reportCGI                                   ReportCGI,

reportSFTD                                  ReportSFTD-NR,

condTriggerConfig-r16                       CondTriggerConfig-r16,

cli-Periodical-r16                          CLI-PeriodicalReportConfig-r16,

cli-EventTriggered-r16                      CLI-EventTriggerConfig-r16,

rxTxPeriodical-r17                          RxTxPeriodical-r17,

reportOnScellActivation-r18                 ReportOnScellActivation-r18

}

}

CondTriggerConfig-r16 ::=        SEQUENCE {

condEventId                      CHOICE {

condEventD1-r17                  SEQUENCE {

distanceThreshFromReference1-r17 INTEGER(0.. 65525),

distanceThreshFromReference2-r17 INTEGER(0.. 65525),

referenceLocation1-r17           ReferenceLocation-r17,

referenceLocation2-r17           ReferenceLocation-r17,

hysteresisLocation-r17           HysteresisLocation-r17,

timeToTrigger-r17                TimeToTrigger

},

condEventD2-r18                  SEQUENCE {

distanceThreshFromReference1-r18 INTEGER(0.. 65535),

distanceThreshFromReference2-r18 INTEGER(0.. 65535),

hysteresisLocation-r18           HysteresisLocation-r17,

timeToTrigger-r18                TimeToTrigger

}

},

rsType-r16                       NR-RS-Type,

...,

[[

nesEvent-r18                     ENUMERATED {true}                                OPTIONAL    -- Need R

]]

}

EventTriggerConfig ::=                      SEQUENCE {

eventId                                     CHOICE {

eventA4                                     SEQUENCE {

a4-Threshold                                MeasTriggerQuantity,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger,

useAllowedCellList                          BOOLEAN

},

eventA5                                     SEQUENCE {

a5-Threshold1                               MeasTriggerQuantity,

a5-Threshold2                               MeasTriggerQuantity,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger,

useAllowedCellList                          BOOLEAN

},

...,

[[

eventD1-r17                                 SEQUENCE {

distanceThreshFromReference1-r17            INTEGER(1.. 65525),

distanceThreshFromReference2-r17            INTEGER(1.. 65525),

referenceLocation1-r17                      ReferenceLocation-r17,

referenceLocation2-r17                      ReferenceLocation-r17,

reportOnLeave-r17                           BOOLEAN,

hysteresisLocation-r17                      HysteresisLocation-r17,

timeToTrigger-r17                           TimeToTrigger

}

]],

[[

eventD2-r18                                 SEQUENCE {

distanceThreshFromReference1-r18            INTEGER(1.. 65535),

distanceThreshFromReference2-r18            INTEGER(1.. 65535),

reportOnLeave-r18                           BOOLEAN,

hysteresisLocation-r18                      HysteresisLocation-r17,

timeToTrigger-r18                           TimeToTrigger

}

]]

},

rsType                                      NR-RS-Type,

reportInterval                              ReportInterval,

reportAmount                                ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

reportQuantityCell                          MeasReportQuantity,

maxReportCells                              INTEGER (1..maxCellReport),

reportQuantityRS-Indexes                     MeasReportQuantity                                            OPTIONAL,   -- Need R

maxNrofRS-IndexesToReport                   INTEGER (1..maxNrofIndexesToReport)                            OPTIONAL,   -- Need R

includeBeamMeasurements                     BOOLEAN,

reportAddNeighMeas                          ENUMERATED {setup}                                             OPTIONAL,   -- Need R

...,

[[

measRSSI-ReportConfig-r16                   MeasRSSI-ReportConfig-r16                                      OPTIONAL,   -- Need R

useT312-r16                                 BOOLEAN                                                        OPTIONAL,   -- Need M

includeCommonLocationInfo-r16               ENUMERATED {true}                                              OPTIONAL,   -- Need R

includeBT-Meas-r16                          SetupRelease {BT-NameList-r16}                                 OPTIONAL,   -- Need M

includeWLAN-Meas-r16                        SetupRelease {WLAN-NameList-r16}                               OPTIONAL,   -- Need M

includeSensor-Meas-r16                      SetupRelease {Sensor-NameList-r16}                             OPTIONAL    -- Need M

]],

[[

coarseLocationRequest-r17                   ENUMERATED {true}                                              OPTIONAL,   -- Need R

reportQuantityRelay-r17                     SL-MeasReportQuantity-r16                                      OPTIONAL    -- Need R

]],

[[

numberOfTriggeringCells-r18                 INTEGER (2..maxCellReport)                                     OPTIONAL,   -- Need R

cellIndividualOffsetList-r18                SEQUENCE (SIZE (1..maxNrofCellMeas)) OF CellIndividualOffsetList-r18 OPTIONAL, -- Need R

eventX1-SD-Threshold1-r18                   SL-MeasTriggerQuantity-r16                                     OPTIONAL,   -- Need S

eventX2-SD-Threshold-r18                    SL-MeasTriggerQuantity-r16                                     OPTIONAL,   -- Need S

reportOnBestCellChange-r18                  ENUMERATED {n1, n2}                                            OPTIONAL,    -- Need R

enteringLeavingReport-r18                   ENUMERATED {true}                                              OPTIONAL     -- Need R

]]

}

-- TAG-REPORTCONFIGNR-STOP

-- ASN1STOP

CondTriggerConfig field descriptions

a4-Threshold Threshold value associated to the selected trigger quantity (e.g. RSRP, RSRQ, SINR) per RS Type (e.g. SS/PBCH block, CSI-RS) to be used in NR conditional reconfiguration triggering condition for cond event a4.

a5-Threshold1/ a5-Threshold2 Threshold value associated to the selected trigger quantity (e.g. RSRP, RSRQ, SINR) per RS Type (e.g. SS/PBCH block, CSI-RS) to be used in NR conditional reconfiguration triggering condition for cond event a5. In the same condeventA5, the network configures the same quantity for the MeasTriggerQuantity of the a5-Threshold1 and for the MeasTriggerQuantity of the a5-Threshold2.

condEventId Choice of NR conditional reconfiguration event triggered criteria.

distanceThreshFromReference1, distanceThreshFromReference2 Distance from a fixed reference location configured with referenceLocation1 or referenceLocation2 for condEventD1. Distance from a moving reference location determined by the UE based on the serving cell movingReferenceLocation broadcast in SIB19 or referenceLocation and the corresponding epoch time and satellite ephemeris configured within the MeasObjectNR associated to the event for condEventD2. Each step represents 50m.

duration This field is used for defining the leaving condition T1-2 for conditional HO event condEventT1. Each step represents 100ms.

referenceLocation1, referenceLocation2 The referenceLocation1 is associated to serving cell and referenceLocation2 is associated to candidate target cell.

t1-Threshold The field counts the number of UTC seconds in 10 ms units since 00:00:00 on Gregorian calendar date 1 January, 1900 (midnight between Sunday, December 31, 1899 and Monday, January 1, 1900).

timeToTrigger Time during which specific criteria for the event needs to be met in order to execute the conditional reconfiguration evaluation.

EventTriggerConfig field descriptions

coarseLocationRequest This field is used to request UE to report coarse location information.

distanceThreshFromReference1, distanceThreshFromReference2 Distance from a fixed reference location configured with referenceLocation1 or referenceLocation2 for eventD1. Distance from a moving reference location determined by the UE based on the serving cell movingReferenceLocation broadcast in SIB19 or referenceLocation and the corresponding epoch time and satellite ephemeris configured within the MeasObjectNR associated to the event for eventD2. Each step represents 50m.

eventId Choice of NR event triggered reporting criteria.

maxNrofRS-IndexesToReport Max number of RS indexes to include in the measurement report for A1-A6 events.

maxReportCells Max number of non-serving cells to include in the measurement report.

numberOfTriggeringCells Indicates the number of cells detected that are required to fulfill an event for a measurement report to be triggered. This field is applicable only for the events concerning neighbor cells, i.e. eventA3, eventA4, eventA5, eventA3H1, eventA3H2, eventA4H1, eventA4H2, eventA5H1, eventA5H2.

referenceLocation1, referenceLocation2 The referenceLocation1 is associated to serving cell and referenceLocation2 is associated to neighbour cell.

reportAddNeighMeas Indicates that the UE shall include the best neighbour cells per serving frequency.

reportAmount Number of measurement reports applicable for eventTriggered as well as for periodical report types.

reportOnBestCellChange Indicates whether the UE shall only send measurement report if the measured best cell (when configured to n1) or two best cells (when configured to n2) have changed. In this release of the specification, this field is applicable only for the events concerning neighbor cells. This field can only be configured when the value of the field reportAmount is set to any other value than r1.

reportOnLeave Indicates whether or not the UE shall initiate the measurement reporting procedure when the leaving condition is met for a cell in cellsTriggeredList or for a L2 U2N Relay UE in relaysTriggeredList, as specified in 5.5.4.1. Indicates whether or not the UE shall initiate the measurement reporting procedure when the leaving condition is met if configured in eventD1, eventD2, eventH1, eventH2 as specified in 5.5.4.1.

reportQuantityCell The cell measurement quantities to be included in the measurement report.

reportQuantityRS-Indexes Indicates which measurement information per RS index the UE shall include in the measurement report.

timeToTrigger Time during which specific criteria for the event needs to be met in order to trigger a measurement report.

useAllowedCellList Indicates whether only the cells included in the allow-list of the associated measObject are applicable as specified in 5.5.4.1.

useT312 If value TRUE is configured, the UE shall use the timer T312 with the value t312 as specified in the corresponding measObjectNR. If value FALSE is configured, the timer T312 is considered as disabled. Network configures value TRUE only if reportType is set to eventTriggered.

SIB19 field descriptions

movingReferenceLocation Reference location of the serving cell of an NTN Earth-moving cell at a time reference. It is used in the evaluation of eventD2 and condEventD2 criteria for the serving cell in RRC_CONNECTED, and location-based measurement initiation in RRC_IDLE and RRC_INACTIVE when distanceThresh is also configured, as defined in TS 38.304 [20]. The time reference of this field is indicated by epochTime in ntn-Config of the serving cell. This field is excluded when determining changes in system information, i.e., changes to movingReferenceLocation should neither result in system information change notifications nor in a modification of valueTag in SIB1. This field is only present in an NTN cell.