3GPP TSG-RAN-WG2 Meeting #131	R2-2506024

Bengaluru, India, Aug 25th – 29th, 2025

Agenda item	:		8.6.4

Source	: 	Sharp

Title	: 	Discussion on issues for supporting conditional LTM

Document for	:	Discussion and Decision

Introduction

Revised WID [1] for mobility enhancement was approved and RAN2 started discussion on the WI. For conditional LTM, RAN2 starts discussion from the RAN2#127bis meeting;

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

In the last meeting, RAN2 discussed conditional LTM and made agreements as below[2]:

- Support the co-existence of CLTM and (e)RedCap.

- Support the co-existence of CLTM and CovEnh.

- For RRC-3, keep current text in the running RRC CR.

- (RRC-9) Use the term “LTM” for parts which are common to LTM and CLTM. Use the term “CLTM” only for parts which apply only to CLTM (but not LTM). This will be used in all RAN2 specifications.

- (MAC-9) If there is at least one beam associated with the configured uplink grant with RSRP above configured RSRP threshold, the UE selects a beam among the beams and CG-based RACH-less CLTM is performed. If the UE finds out multiple beams above threshold, it is up to UE implementation to select which one. If there is no beam associated with the configured uplink grant with RSRP above configured RSRP threshold, RACH-based CLTM is performed.

- For L3 based CLTM, the UE performs RACH-based CLTM if there is no configured grant.

- Upon CLTM execution, if TA is valid (i.e. TAT timer is running) at the first available CG occasion, and there is CG resource configured, RACH-less CLTM will be performed.

- During CLTM is ongoing, after the first transmission, if TAT timer expires while RACH-less LTM is ongoing, fallback to RACH-based CLTM.

- In case CG resource is only associated with SSB, if the beam meeting the execution condition is CSI-RS, the UE determines whether a CG resource is valid based on the SSB associated with the CSI-RS.

- RAN2 will not do anything for the separate TCI mode optimization.

In this paper, we discuss some issued related to conditional LTM.

Discussion

(MAC-14) TAT handling for the case that coexistence with LTM

There is the open issue related to TAT handling for the case that the UE is configured with LTM and CLTM as follow:

As described above, it is necessary for RAN2 to clarify whether the UE should perform a RACH-based or RACH-less CLTM in the following situation:

- The UE has a valid TA value associated with the candidate cell’s running TAT; and

- The UE receives a Cell Switch Command (CSC) without a valid TA value for this candidate cell (i.e., Timing Advanced Command field is set to ‘FFF’).

If the UE is allowed to use the stored TA value for a network-triggered LTM, the procedure would deviate from the legacy LTM behaviour and introduce specification impacts. Furthermore, if the network intends to perform a network-triggered RACH-less LTM, the network should be able to provide a valid TA value within the CSC.

Observation 1. If the network intends to perform a network-triggered RACH less-LTM, the network should be able to provide a valid TA value within the CSC.

Proposal 1. (MAC-14) In the case where the UE has a valid TA value and the associated TAT is running, the UE performs RACH-based LTM upon receiving a CSC that does not include a valid TA value (i.e., the Timing Advance Command field is set to 'FFF').

Rel-19 LTM and Rel-19 CLTM

In RAN2#129bis meeting, RAN2 agreed that network can send an LTM Cell Switch Command MAC CE indicating a CLTM candidate configuration, and no specification change is expected. It means that CLTM candidate configuration can be used for intra-CU LTM configuration. In TS 38.331 running CR [3], the LTM-CandidateId-r18 included in the CLTM candidate configuration (i.e., CLTM-ExecutionConditions-r19) can be associated with the LTM-CandidateId-r18 included in the intra-CU LTM candidate configuration (i.e., LTM-Candidate-r18). Thus, Rel-19 CLTM and Rel-18 LTM can co-exist.

Proposal 2. RAN2 confirms that Rel-19 CLTM can co-exist with Rel-18 LTM.

However, RAN2 hasn’t discussed whether Rel-19 LTM and Rel-19 CLTM can coexist. If the coexistence between Rel-19 LTM and Rel-19 CLTM is not realized, the robustness of intra-CU LTM will be ensured, but mobility scenarios will be significantly restricted. Thus, Rel-19 LTM should coexist with Rel-19 CLTM. However, according to the WID, Rel-19 CLTM is limited to the intra-CU and non-DC case. Therefore, non-DC CLTM configurations can coexist with inter-CU LTM configurations. According to the current CR, UE stops CLTM evaluation upon LTM/HO execution and restarts the evaluation if evaluation configurations are included in the target configuration. So, if the UE executes LTM cell switch to a cell which has Rel-19 Set ID different from serving cell, UE suspends CLTM evaluation until the UE returns to the cell of the CU with which the CLTM configuration is associated. Therefore, the coexistence has no spec impact.

Proposal 3. RAN2 confirms that CLTM configuration can coexist with inter-CU MN LTM configuration in case of non-DC (no spec impact is expected).

Conclusion

Observation 1. If the network intends to perform a network-triggered RACH less-LTM, the network should be able to provide a valid TA value within the CSC.

Proposal 1. (MAC-14) In the case where the UE has a valid TA value and the associated TAT is running, the UE performs RACH-based LTM upon receiving a CSC that does not include a valid TA value (i.e., the Timing Advance Command field is set to 'FFF').

Proposal 2. RAN2 confirms that Rel-19 CLTM can co-exist with Rel-18 LTM.

Proposal 3. RAN2 confirms that CLTM configuration can coexist with inter-CU MN LTM configuration in case of non-DC (no spec impact is expected).

Reference

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4

- R2-2504671, Report from session on R18 SL, R18/19 MOB and R19 NES