3GPP TSG-RAN WG2 Meeting #131	R2-2506035

Bangalore, India, Aug 25th – 29th, 2025

Agenda Item:	8.12.3

Source: 	ASUSTeK

Title:  	Discussion on MAC open issues for MIMO phase 5

Document for:	Discussion and Decision

- Introduction

In the previous meeting RAN2#130, there were discussions regarding UEI report configuration resource release and whether to initiate a random access procedure when a UEI report is triggered but PUCCH resource has been released due to TAT expiry:

When the TAT of the pTAG expires, UE releases PUCCH resource for mode-A/B UEI report and clears type-1 CG for mode-B UEI report. FFS for the case when the TAT expires on the sTAG. R2-2504732 Report of offline discussion [AT130][202][MIMO_Ph5] Samsung discussion Rel-19 NR_MIMO_Ph5-Core Noted Proposal 2: (10/14) UE initiates RA procedure at least for the following cases: Case 1: If a UEI report is triggered and TAT is expired before mode-A/B PUCCH transmission for this report Case 2: If a UEI report is triggered and TAT is expired between PUCCH and PUSCH transmission for this report. Proposal 3: If PUCCH/PUSCH resource for UEI report is released/cleared, and RA is triggered, UE waits for NW to reconfigure resource.

In post email discussion [POST130][219], the discussion continued and has reached following proposal for further discussion:

Proposal 1 (12/12): If the PUCCH of a UEI report configuration is pointed to a SCell whose TAT of the single sTAG is expired, this PUCCH for the SCell is released by RRC. If the type-1 CG of a UEI report configuration is pointed to a SCell whose TAT of the single sTAG is expired, this type-1 CG for the SCell is cleared as a configured UL grant. There is no MAC specification impact. … Proposal 2: Regarding whether UE initiate RACH when UEI report is triggered but TAT is expired, select one of the following options: - Option 1: UE does not initiate RACH when UEI report is triggered but there is no PUCCH or type-1 CG due to the associated TAT is expired. No MAC impacts. - Option 3: UE initiates RACH when UEI report is triggered but there is no PUCCH or type-1 CG due to the associated TAT is expired, and UE indicates the cause of RACH. Discuss how to indicate the cause of RACH (e.g., MAC CE in Msg3).

In this contribution, we discuss further details for open issues for MIMO phase 5.

- Discussion

UEI report configuration resource release for different TAGs

In the previous meeting, it was agreed that when the TAT of the pTAG expires, UE releases PUCCH resource for mode-A/B UEI report and clears type-1 CG for mode-B UEI report. In email discussion	[Post130][219], UE behaviour for SCell TAT expiry was discussed and a common understanding has been reached that if the PUCCH of a UEI report configuration is pointed to a SCell whose TAT of the single sTAG is expired, this PUCCH for the SCell is released by RRC. If the type-1 CG of a UEI report configuration is pointed to a SCell whose TAT of the single sTAG is expired, this type-1 CG for the SCell is cleared as a configured UL grant. There is no MAC specification impact.

One issue raised by some companies is that “the PUCCH of a UEI report configuration” pointed to an SCell can be configured on the same TAG or on another TAG (e.g., PCell or PUCCH SCell) with the SCell, and with the current wording of the proposal, there could be some ambiguity whether the same UE behaviour should be applied to both scenarios.

Observation 1:	For a UEI report configuration pointed to a SCell, the PUCCH resource can be configured on a PCell, which can be on a different TAG than the SCell.

In our view, both scenarios should have a same behaviour which is already supported by current MAC spec, since the RRC should release the resources per-UEI report configuration associated with the SCell and should not distinguish these two scenarios. That is, the PUCCH and PUSCH resources of the UEI report configuration are released and cleared upon TAT expiry of the SCell, regardless of which Cells/TAGs they are configured on:

(quoted from 38.321) 1> when a timeAlignmentTimer expires: … 3> if the timeAlignmentTimer is associated with a TAG for an SCell configured with only this TAG; or 3> if the timeAlignmentTimer is associated with a TAG for an SCell, and if the SCell is configured with two TAGs and the timeAlignmentTimer associated with the other TAG is not running: 4> flush all HARQ buffers for all such SCells; 4> notify RRC to release PUCCH, if configured for all such SCells; 4> notify RRC to release SRS, if configured for all such SCells; 4> clear any configured downlink assignments and configured uplink grants for all such SCells; 4> clear any PUSCH resource for semi-persistent CSI reporting for all such SCells; 4> maintain NTA (defined in TS 38.211 [8]) of this TAG.

Observation 2:	Upon TAT expiry of the SCell, the UE should release PUCCH and UL configured grant resources of UEI report configuration of the SCell, regardless of which TAGs these resources are actually associated with.

Therefore, we proposes to have a common understanding in RAN2 about the current MAC spec behaviour is that the UE will release PUCCH and clear type-1 CG PUSCH resources in a UEI report configuration of a SCell upon TAT expiry of TAG associated with the SCell, regardless of the exact Cell on which the resources are configured.

Proposal 1:	RAN2 to confirm that based on the current MAC spec, the UE releases PUCCH resources and clears type-1 CG PUSCH resources configured in a UEI report configuration of a SCell if the TAT of the sTAG of the SCell is expired, regardless of the Cell on which the PUCCH and PUSCH resources are configured.

Random access procedure for UEI report when PUCCH

In the previous meeting, it was discussed whether the UE shall initiate a random access procedure in response to a triggered UEI report when there’s no PUCCH resource for such UEI report due to TAT expiry and was not concluded.

In our view, a random access procedure would be beneficial if the network can acknowledge the cause of the RACH in order to reconfigure/provide UL grant for corresponding UEI beam report as early as possible. While it may not be suitable to transmit the UEI report via UCI with Msg3, a notification or a report carried in Msg3, (e.g., a MAC CE) or a dedicated preamble for such random access procedure (similar to BFR-initiated RACH) would be helpful for the NW to quickly perform related actions.

Proposal 2:	The UE initiates RACH when a UEI report is triggered but there is no PUCCH or type-1 CG for the triggered UEI report due to the associated TAT being expired, and UE indicates the cause of RACH via at least one of the following options:

- Option 1: explicit indication via a MAC CE in Msg3

- Option 2: implicit indication via a dedicated preamble configured for UEI report-initiated RACH.

Clearing PUSCH resource regarding sDCI mTRP with 2TA

In the current MAC spec, the UE clears configured uplink grant if TCI state(s) of the configured uplink grant are associated with a TAG of an expired TAT:

1> when a timeAlignmentTimer expires: 2> if the timeAlignmentTimer is associated with a PTAG and the SpCell is not configured with two PTAGs; or 2> if the timeAlignmentTimer is associated with a PTAG, the SpCell is configured with two PTAGs, and the … 3> else if the timeAlignmentTimer is associated with a TAG for a Serving Cell configured with two TAGs, and if the timeAlignmentTimer associated with the other TAG is running, for all such Serving Cells: 4> clear any configured downlink assignment, if the activated TCI state(s) for all PUCCH resources configured for the configured downlink assignment is associated with the TAG of the expired timeAlignmentTimer; 4> clear any configured uplink grant, if the activated TCI state(s) for the configured uplink grant is associated with the TAG of the expired timeAlignmentTimer; 4> clear any PUSCH resource for semi-persistent CSI reporting, if the activated TCI state(s) for the PUSCH resource is associated with the TAG of the expired timeAlignmentTimer; 4> maintain NTA (defined in TS 38.211 [8]) of this TAG.

There was an open issue raised by other company regarding MAC behaviour on clearing configured uplink grant when TAT expires for sDCI mTRP with 2TA. Due to the fact that 2TA in Rel-18 aims for the scenario of mDCI mTRP, each configured uplink grant is configured to apply either 1-st TCI state or 2-nd TCI state (i.e., not apply both). Besides, since mDCI mTRP targets independent TRP behaviour, MAC behaviour of TRP-specific resource clearing specifies for single TAT expiry. However, for sDCI mTRP, since configured uplink grant could be configured to apply two TCI states that belong to different TAGs, we echo that MAC entity behaviour for sDCI mTRP (especially STxMP) needs to be discussed. That is, RAN2 needs to clarify whether MAC entity performs TRP-specific PUSCH resource clearing when only one TAT expires. In our view, we provide two alternatives below:

For sDCI mTRP and a configured uplink grant is configured to apply two TCI states associated with different TAGs,

Alt1: the MAC entity clears a configured uplink grant when one associated TAT expires and another associated TAT is running

(Possible text proposal for Alt 1):

3> else if the timeAlignmentTimer is associated with a TAG for a Serving Cell configured with two TAGs, and if the timeAlignmentTimer associated with the other TAG is running, for all such Serving Cells: 4> clear any configured downlink assignment, if the activated TCI state(s) for all PUCCH resources configured for the configured downlink assignment is associated with the TAG of the expired timeAlignmentTimer; 4> clear any configured uplink grant, if theany activated TCI state(s) for the configured uplink grant is associated with the TAG of the expired timeAlignmentTimer; 4> clear any PUSCH resource for semi-persistent CSI reporting, if theany activated TCI state(s) for the PUSCH resource is associated with the TAG of the expired timeAlignmentTimer;

Alt2: the MAC entity does not clear a configured uplink grant when one associated TAT expires and another associated TAT is running

(Possible text proposal for Alt 2):

3> else if the timeAlignmentTimer is associated with a TAG for a Serving Cell configured with two TAGs, and if the timeAlignmentTimer associated with the other TAG is running, for all such Serving Cells: 4> clear any configured downlink assignment, if the activated TCI state(s) for all PUCCH resources configured for the configured downlink assignment is associated with the TAG of the expired timeAlignmentTimer; 4> clear any configured uplink grant, if theall activated TCI state(s) for the configured uplink grant is associated with the TAG of the expired timeAlignmentTimer; 4> clear any PUSCH resource for semi-persistent CSI reporting, if theall activated TCI state(s) for the PUSCH resource is associated with the TAG of the expired timeAlignmentTimer;

Since the purpose of sDCI mTRP scheme includes enhancing reliability (e.g., TDM or STxMP with SDM scheme) and enhancing spectrum efficiency (E.G., STxMP with SFN scheme), it’s proper to guarantee sDCI mTRP scheme works when both TATs of applied TCI states are running. Thus, we prefer Alt1, and we propose RAN2 to down-select from one of the alternatives.

Proposal 3: For sDCI mTRP and a configured uplink grant is configured to apply two TCI states associated with different TAGs, RAN2 down-selects one alternative

- Alt1: the MAC entity clears a configured uplink grant when one associated TAT expires and another associated TAT is running

- Alt2: the MAC entity does not clear a configured uplink grant when one associated TAT expires and another associated TAT is running

- Conclusion

We have the following observations and proposals for UEI report:

Observation 1:	For a UEI report configuration pointed to a SCell, the PUCCH resource can be configured on a PCell, which can be on a different TAG than the SCell.

Observation 2:	Upon TAT expiry of the SCell, the UE should release PUCCH and UL configured grant resources of UEI report configuration of the SCell, regardless of which TAGs these resources are actually associated with.

Proposal 1:	RAN2 to confirm that based on the current MAC spec, the UE releases PUCCH resources and clears type-1 CG PUSCH resources configured in a UEI report configuration of a SCell if the TAT of the sTAG of the SCell is expired, regardless of the Cell on which the PUCCH and PUSCH resources are configured.

Proposal 2:	The UE initiates RACH when a UEI report is triggered but there is no PUCCH or type-1 CG for the triggered UEI report due to the associated TAT being expired, and UE indicates the cause of RACH via at least one of the following options:

- Option 1: explicit indication via a MAC CE in Msg3

- Option 2: implicit indication via a dedicated preamble configured for UEI report-initiated RACH.

Proposal 3: For sDCI mTRP and a configured uplink grant is configured to apply two TCI states associated with different TAGs, RAN2 down-selects one alternative

- Alt1: the MAC entity clears a configured uplink grant when one associated TAT expires and another associated TAT is running

- Alt2: the MAC entity does not clear a configured uplink grant when one associated TAT expires and another associated TAT is running

- Reference