3GPP TSG-RAN WG2 Meeting #131	R2-2506041

Bangalore, India Aug 25th – 29th, 2025

Source:                Qualcomm Incorporated

Title:	Open issues on MRO enhancements

Agenda Item:	8.10.2

Document for:	Discussion and Decision

# Introduction & Background

- MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]:

- Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3]

- Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

# Discussion

Open issue RRC-1: Avoiding duplicated measurement reports in SHR and SPR

In last RAN2 meeting, there is FFS issue on whether to add some correlation indication for SHR and SPR report.

FFS if we add some correlation indication.

We will not add any optimizations to avoid duplicated info in correlated reports

During post-meeting email discussion, there are four options are proposed:

- Correlation indication in RRC_Complete message (a flag beside availability indication)

- Correlation indication inside the SHR and SPR

- Logging C-RNTI of PCell and timeSinceSPR in SPR

- The duplicated information in correlated SHR and SPR is sufficient. no further information is needed for correlation.

It is understand that it should be corner case that both MCG and SCG encounter failure during CHO, then the SHR and SPR are generated and reported as the same time should not be frequent. In SHR, UE will log and report measurements for the PCell and PSCell. In SPR, UE will log and report measurements for PSCell and PCell. In case that SHR and SPR are reported as the same time, the PCell and PSCell measurement could be correlated with each other, and network can identify the correlation between SHR and SPR. Then there is no need to introduce additional correlation between SHR and SPR.

- There is no need to introduce additional correlation between SHR and SPR.

### RACH information in RLF Report

This issue was discussed in last RAN2 meeting but postponed.

When the UE performs RACH-less RACH but fails, UE could perform fast recovery. There could be two cases that the RACH-less LTM is not successful:

Case 1: The UE doesn’t receive a valid TA from LTM CSC or if the UE-based TA measurement is not successful, UE doesn’t even start the RACH-less LTM cell switch procedure and directly attempts a RACH-based LTM procedure.

Case 2: The RACH-less LTM is initiated (because of UE having a valid TA either received from LTM CSC or via UE-based measurement) but the RACH-less LTM cell switch execution is not successful e.g., because UE couldn’t determine that the network has successfully received its first UL data. In such a case, T304 would expire.

Upon T304 expiry, UE performs cell selection and if the selected cell is an LTM candidate cell and if network configured the UE to try LTM recovery after LTM execution failure, then the UE attempts RACH-based LTM recovery once, otherwise re-establishment is performed. Irrespective of whether the LTM recovery was configured or not and if the LTM recovery was successful or not, it is beneficial if the UE indicates to gNB if RACH-less LTM was attempted or not before RACH-based LTM failure so that gNB will know whether TA configuration was not proper or UL transmission was not good.

- UE indicates to gNB in SHR if RACH-less LTM is attempted or not in near failure case.

- UE indicates to gNB in RLF Report if RACH-less LTM is attempted or in failure case.

# Conclusion

- There is no need to introduce additional correlation between SHR and SPR.

- UE indicates to gNB in SHR if RACH-less LTM is attempted or not before LTM recovery success.

- UE indicates to gNB in RLF Report if RACH-less LTM is attempted or not when RACH-less LTM is not successful.

# Reference