3GPP TSG-RAN WG2 Meeting #131	R2-2506042

Bangalore, India Aug 25th – 29th, 2025

Source:                Qualcomm Incorporated

Title:	Open issues for NTN SONMDT

Agenda Item:	8.10.2

Document for:	Discussion and Decision

# Introduction & Background

This contribution addresses the open issues for NTN SON and MDT.

# Discussion

In last RAN2 meeting, there is FFS issue if the UE logs the MDT data or not when it cannot obtain its location.

If configured with additional geographical information, FFS if the UE logs the MDT data or not when it cannot obtain its location.

During post-meeting email discussion [1], open issue RRC-4: Logging MDT in NTN when location is unavailable was discussed. For this issue, Rapp’s proposal is: If the UE is unable to obtain its location, the UE follows legacy procedure as in logged MDT in terrestrial network.

However, it is unclear what the legacy procedure is, and UE behaviors are different pending on whether legacy area scope is configured.

If legacy area scope is configured, e.g. both areaConfiguration-r16 and areaConfigurationNTN-List-r19 are configured as shown in following. It is unclear for the UE which area scope should be follow, e.g. UE should only check the overlapped part of the legacy area scope and Rel-9 area scope, or UE should follow each area scope?  For the former one, UE will collect MDT data when the UE is inside of overlapped part; for the latter one, UE will collect MDT data when the UE is inside of either area scope.

Observation 1: If both legacy area scope and Rel-19 area scope are configured, it is unclear which area scope should be applied when collecting MDT data, 1) UE should collect data inside of the overlapped part, or 2) inside of either area scope.

Rel-19 area scope should be enough to control the area to collect MDT data, and it is unclear what is the usage to configure legacy area scope when Rel-19 area scope is configured.

In order to simplify UE implementation and specification efforts, it is proposed legacy area scope should not be configured when Rel-19 area scope is configured.

- The legacy area scope should not be configured when Rel-19 area scope is configured.

When the Rel-19 area scope is configured, if the UE has no location, UE cannot check whether to move outside of the area scope. So, in order not to collect MDT data network does not want, UE should suspend MDT data collection.

- UE should suspend data collection if there is no location.

# Conclusion

Observation 1: If both legacy area scope and Rel-19 area scope are configured, it is unclear which area scope should be applied when collecting MDT data, 1) UE should collect data inside of the overlapped part, or 2) inside of either area scope.

- The legacy area scope should not be configured when Rel-19 area scope is configured.

- UE should suspend data collection if there is no location.

# Reference

[1]	 [Post130][605][SONMDT] Running NR RRC CR (Ericsson)