3GPP TSG-RAN2 Meeting #131	R2-2506055

# Bangalore, India, 25th - 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 5448 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on reporting noSuitableCellFound |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | If the UE is not able to find a suitable cell during RRCReestablishment procedure, T311 will expire. Accordinf to the current specification, upon T311 expiry, UE logs noSuitableCellFound if the Reestablishment procedure was initiated due to radio link failure or handover failure. The reason for adding the condition “if the procedure was initiated due to radio link failure or handover failure” is to exclude cases like Reestablishment due to reconfiguration failure, integrity failure etc. With fast MCG link recovery, RRC Reestablishment procedure may be initiated when T316 expires. One of the most frequent reasons for the T316 expiry is that the network is not able to find out a suitable cell where the UE can be moved during fast MCG link recovery. Fast MCG link recovery occurs after radio link failure in MCG. However the section 5.3.7.2 about the initiation of RRC Reestablishment considers T316 expiry and Radio Link Failure in MCG seperately as below. The UE initiates the procedure when one of the following conditions is met: 1> upon detecting radio link failure of the MCG and t316 is not configured, in accordance with 5.3.10; or - upon T316 expiry, in accordance with clause 5.7.3b.5; or This will lead to the UE not logging noSuitableCellFound even when the root cause of the generation of RLF report is the Radio link failure in MCG. Such failure may also occur due to some actions in SCG while T316 is running, but this may be relatively less critical to fix after the spec is frozen. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added that the UE logs noSuitableCellFound in RLF report when the RRC Reestablishment procedure is initiated due to T316 expiry Impact analysis Impacted functionality: RLF Report Impacted 5G architecture options: NR-DC Inter-operability: - If the network is implemented according to the CR and the UE is not, Upon T311 expiry, the UE may not log that there was no suitable cell found when RRC Reestablishment procedure is intiated due to the expiry of T316. - If the UE is implemented according to the CR and the network is not, then there are no interoperability issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Network will not be able to identify that there was no suitable cell when the Reestablishment procedure was initiated due to T316 expiry. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.7.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

#### 5.3.7.6	T311 expiry

Upon T311 expiry, the UE shall:

1>	if the procedure was initiated due to radio link failure or handover failure or (if the UE supports RLF-Report for fast MCG recovery procedure) T316 expiry:

2>	set the noSuitableCellFound in the VarRLF-Report to true;

1>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure'.

END OF CHANGE