3GPP TSG-RAN WG2 Meeting #131	                                                                    R2-2506056

Bangalore, India, Aug 25th – 29th, 2025

Title		Discussion on remaining open issues for AIML positioning

Source	LG Electronics

Document for	Discussion and Decision

Agenda Item	8.1.2.3 LCM for Positioning use case

WID/SID	NR_AIML_air-Core

# 1	Introduction

Following the post-email discussion after RAN2#130 [1], companies are invited to provide contributions for RAN2#131 addressing the remaining open issues for AI/ML positioning.

This paper presents our views on these open issues and related proposals.

# 2	Discussion

## 2.1	LPP-6a (new): Number of PRUs in NR-PRU-DL-Info

Issue description: Currently, the IE NR-PRU-DL-Info provides PRU measurements for a single PRU. However, the issue was raised at RAN2#130 whether more PRUs are needed to be included for training purposes. If more PRUs are needed, a LMF can always provide multiple Provide Assistance Data messages. However, the Request Assistance Data message may need to include an indication of e.g., number of PRUs requested (if needed). Proposed resolution: Companies to provide contributions to the following meeting on whether more PRUs are needed to be included for training purposes, and if so, the corresponding signalling details.

In the current LPP specification, the IE NR-PRU-DL-Info is defined to deliver carrier phase and associated measurements from a single PRU to a target UE via the LMF, primarily for DL-TDOA-based positioning support. This has been sufficient for conventional inference-oriented use cases.

For AI/ML positioning Case 1, however, especially during training data collection, a single PRU is insufficient. Training an AI/ML model for positioning requires a diverse and large set of labeled data samples, each comprising DL-PRS measurements from multiple PRUs together with the corresponding location information. To efficiently collect such data across various environments, frequency bands, and radio conditions, the LMF needs to provide measurements from multiple PRUs.

The existing NR-PRU-DL-Info IE supports only one PRU per instance. As a result, the LMF must send multiple ProvideAssistanceData messages to deliver measurements from multiple PRUs, introducing additional signaling overhead and reducing efficiency.

Observation 1.	The current NR-PRU-DL-Info structure supports only a single PRU per instance, which limits signaling efficiency and scalability for AI/ML positioning training that requires multiple PRU data points.

To address this limitation, supporting multiple PRU measurement–location pairs within a single ProvideAssistanceData message would significantly improve signaling efficiency and enable scalable training data collection. This can be achieved by extending the NR-PRU-DL-Info IE to include multiple PRU measurement–location entries in one instance.

Such a structure would allow the location server to deliver multiple PRU measurements in a single signaling exchange, ensuring efficient transmission of training datasets, consistent temporal alignment across PRUs, and flexibility for future extensions — for example, selective data delivery based on environment, frequency band, or location.

The following ASN.1 extension (red color) illustrates the proposed addition:

NR-PositionCalculationAssistance-r16 ::= SEQUENCE { nr-TRP-LocationInfo-r16 NR-TRP-LocationInfo-r16 OPTIONAL, -- Need ON nr-DL-PRS-BeamInfo-r16 NR-DL-PRS-BeamInfo-r16 OPTIONAL, -- Need ON nr-RTD-Info-r16 NR-RTD-Info-r16 OPTIONAL, -- Need ON ..., [[ nr-TRP-BeamAntennaInfo-r17 NR-TRP-BeamAntennaInfo-r17 OPTIONAL, -- Need ON nr-DL-PRS-Expected-LOS-NLOS-Assistance-r17 NR-DL-PRS-ExpectedLOS-NLOS-Assistance-r17 OPTIONAL, -- Need ON nr-DL-PRS-TRP-TEG-Info-r17 NR-DL-PRS-TRP-TEG-Info-r17 OPTIONAL -- Need ON ]], [[ nr-IntegrityServiceParameters-r18 NR-IntegrityServiceParameters-r18 OPTIONAL, -- Need OR nr-IntegrityServiceAlert-r18 NR-IntegrityServiceAlert-r18 OPTIONAL, -- Need OR nr-IntegrityRiskParameters-r18 NR-IntegrityRiskParameters-r18 OPTIONAL, -- Need OR nr-IntegrityParametersTRP-LocationInfo-r18 NR-IntegrityParametersTRP-LocationInfo-r18 OPTIONAL, -- Cond Integrity1 nr-IntegrityParametersDL-PRS-BeamInfo-r18 NR-IntegrityParametersDL-PRS-BeamInfo-r18 OPTIONAL, -- Cond Integrity2 nr-IntegrityParametersRTD-Info-r18 NR-IntegrityParametersRTD-Info-r18 OPTIONAL, -- Cond Integrity3 nr-IntegrityParametersTRP-BeamAntennaInfo-r18 NR-IntegrityParametersTRP-BeamAntennaInfo-r18 OPTIONAL, -- Cond Integrity4 nr-PRU-DL-Info-r18 NR-PRU-DL-Info-r18 OPTIONAL -- Need ON nr-PRU-DL-Info-list-r19 NR-PRU-DL-Info-list-r19 OPTIONAL -- Need ON ]] } NR-PRU-DL-Info-list-r19 ::= SEQUENCE (SIZE(1..maxPRUs)) OF NR-PRU-DL-Info-r18

This proposed enhancement improves the scalability and practicality of AI/ML-based positioning deployments while maintaining compatibility with existing single-PRU behavior.

Proposal 1.	Extend NR-PRU-DL-Info to support multiple PRU measurement–location pairs within a single message, enabling efficient collection of diverse training data for AI/ML-based positioning without impacting existing single-PRU DL-TDoA operations.

In addition, the NR-DL-AIML-RequestAssistanceData IE may be optionally enhanced to allow the target UE to indicate the number of PRUs it intends to receive, for example through a parameter such as requestedPRUcount.

The following ASN.1 extension (red color) illustrates the proposed addition:

NR-DL-AIML-RequestAssistanceData-r19 ::= SEQUENCE { nr-PhysCellID-r19 NR-PhysCellID-r16 OPTIONAL, nr-DL-PRS-AssistanceDataReq-r19 ENUMERATED { requested } OPTIONAL, nr-DL-PRS-ExpectedAoD-or-AoA-Req-r19 ENUMERATED { eAoD, eAoA } OPTIONAL, nr-on-demand-DL-PRS-Req-r19 NR-On-Demand-DL-PRS-Request-r17 OPTIONAL, pre-configured-AssistanceDataReq-r19 ENUMERATED { true } OPTIONAL, nr-PositionCalculationAssistanceReq-r19 BIT STRING { trpLoc (0), beamInfo (1), rtdInfo (2), losNlosInfo (3), trpTEG-Info (4), nr-IntegrityBounds (5), pruInfo (6), trpImplicitLoc-Implicit (7) } (SIZE (1..8)) OPTIONAL, nr-IntegrityAssistanceReq-r19 BIT STRING { serviceParameters (0), serviceAlert (1), riskParameters (2), integrityParaTRP-Loc (3), integrityParaBeamInfo (4), integrityParaRTD-Info (5) } (SIZE (1..8)) OPTIONAL, requestedPRUcount-r19 INTEGER (1..maxPRUs) OPTIONAL, ... }

Proposal 2.	Enhance NR-DL-AIML-RequestAssistanceData (optionally) to include a parameter (e.g., requestedPRUcount) allowing the UE to indicate the desired number of PRUs.

## 2.2	LPP-10a: Applicability of "batch reporting" for AI/ML positioning.

Not discussed at RAN2#130 and no further RAN1 input is available.

Batch reporting is an enhancement rather than a fundamental function of a positioning method, but its application to AI/ML positioning Case 1 is both feasible and beneficial. Since Case 1 shares the same DL-PRS-based measurement architecture and reporting behavior with UE-based DL-TDoA, the existing batch reporting structure can be reused without architectural changes and with minimal complexity.

Given the above similarities, there is no technical reason to defer this enhancement to a future release. Supporting batch reporting in Case 1 in the current specification will ensure consistency across UE-based positioning methods and improve efficiency in scenarios such as periodic reporting, buffered delivery during DRX, and UAV trajectory collection.

In running CR [2], nr-DL-AIML-LocationInformationInstances-r19 is already included as follow:

NR-DL-AIML-ProvideLocationInformation-r19 ::= SEQUENCE { nr-DL-AIML-LocationInformation-r19 NR-DL-AIML-LocationInformation-r19 OPTIONAL, nr-DL-AIML-LocationInformationInstances-r19 SEQUENCE (SIZE (1..maxMeasInstances-r17)) OF NR-DL-AIML-LocationInformation-r19 OPTIONAL, -- Cond batch nr-DL-AIML-Positioning-Error-r19 NR-DL-AIML-Positioning-Error-r19 OPTIONAL, ... }

Proposal 3.	Maintain the batch reporting structure, including nr-DL-AIML-LocationInformationInstances-r19, in NR-DL-AIML-ProvideLocationInformation-r19 as already defined in the running CR [2].

## 2.3	LPP-11: Details of IE NR-AI-ML-PositioningRequestLocationInformation

Agreement from RAN2#130: The IE NR-AI-ML-PositioningRequestLocationInformation contains (at least) the AssistanceAvailability flag. Additional details/information can be discussed via company contributions.. The status is kept "open" for additional company contributions (if any).

The IE NR-DL-TDOA-RequestLocationInformation includes various fields for configuring measurement and reporting behavior for UE-assisted positioning, such as requested measurements, report configuration, and timing references. These are not applicable to AI/ML positioning Case 1, where the UE performs inference internally and reports only the final location estimate (UE-based positioning).

Therefore, NR-DL-AIML-RequestLocationInformation should be defined in a simplified form, excluding measurement-related parameters for UE-assisted positioning, and including only UE-based and common parameters such as the nr-AssistanceAvailability flag. No additional fields are needed in NR-DL-AIML-RequestLocationInformation.

In the current running CR [2], only nr-AssistanceAvailability is included, as shown below:

NR-DL-AIML-RequestLocationInformation-r19 ::= SEQUENCE { nr-DL-PRS-RstdMeasurementInfoRequest-r16 ENUMERATED { true } OPTIONAL,-- Need ON nr-RequestedMeasurements-r16 BIT STRING { prsrsrpReq (0), firstPathRsrpReq-r17 (1), dl-PRS-RSCPD-Request-r18 (2) } (SIZE(1..8)), nr-AssistanceAvailability-r16 BOOLEAN, nr-DL-TDOA-ReportConfig-r16 NR-DL-TDOA-ReportConfig-r16 OPTIONAL, -- Need ON additionalPaths-r16 ENUMERATED { requested } OPTIONAL, -- Need ON ..., [[ nr-UE-RxTEG-Request-r17 ENUMERATED { requested } OPTIONAL, -- Need ON nr-los-nlos-IndicatorRequest-r17 SEQUENCE { type-r17 LOS-NLOS-IndicatorType1-r17, granularity-r17 LOS-NLOS-IndicatorGranularity1-r17, ... } OPTIONAL, -- Need ON additionalPathsExt-r17 ENUMERATED { requested } OPTIONAL, -- Need ON additionalPathsDL-PRS-RSRP-Request-r17 ENUMERATED { requested } OPTIONAL, -- Need ON multiMeasInSameReport-r17 ENUMERATED { requested } OPTIONAL -- Need ON ]], [[ nr-DL-PRS-JointMeasurementRequest-r18 SEQUENCE { nr-DL-PRS-JointMeasurementRequestedPFL-List-r18 SEQUENCE (SIZE (2..3)) OF INTEGER (0..nrMaxFreqLayers-1-r16) OPTIONAL -- Need ON } OPTIONAL, -- Need ON nr-DL-PRS-RxHoppingRequest-r18 SEQUENCE { nr-DL-PRS-RxHoppingTotalBandwidth-r18 CHOICE { fr1 ENUMERATED {mhz40, mhz50, mhz80, mhz100}, fr2 ENUMERATED {mhz100, mhz200, mhz400} } OPTIONAL -- Need ON } OPTIONAL -- Need ON ]] }

Proposal 4.	Keep NR-DL-AIML-RequestLocationInformation, excluding UE-assisted measurement parameters, and retain only UE-based and common parameters (e.g., nr-AssistanceAvailability).

In this case, the LocationInformationType field in the CommonIEsRequestLocationInformation IE of the RequestLocationInformation message shall be set to locationEstimateRequired. Other values such as locationMeasurementsRequired, locationEstimatePreferred, locationMeasurementsPreferred, and locationEstimateAndMeasurementsRequired are not applicable and shall not be used in AI/ML positioning Case 1.

Proposal 5.	For AI/ML positioning Case 1, the LocationInformationType field in CommonIEsRequestLocationInformation shall be set to locationEstimateRequired. Other values, including locationMeasurementsRequired, locationEstimatePreferred, locationMeasurementsPreferred, and locationEstimateAndMeasurementsRequired, are not applicable and shall not be used.

## 2.4	LPP-14: Target device error causes

Agreement from RAN2#130: Introduce ‘DL AIML positioning not available’ as new target device error cause for AI/ML positioning case 1, to indicate UE cannot perform positioning method (e.g. model not available and performance monitoring outcome not available).. Above error cause is added to the running CR _v02 [3]. Company contributions (if any) can propose additional details.

According to the RAN2#130 agreement, a new target device error cause for AI/ML positioning Case 1, dl-aiml-positioning-not-available, has been added to NR-DL-AIML-TargetDeviceErrorCauses in the running CR [2].

NR-DL-AIML-TargetDeviceErrorCauses-r19 ::= SEQUENCE { cause-r19 ENUMERATED { undefined, assistance-data-missing, unableToMeasureAnyTRP, attemptedButUnableToMeasureSomeNeighbourTRPs, thereWereNotEnoughSignalsReceived, locationCalculationAssistanceDataMissing, dl-aiml-positioning-not-available, ... }, remoteUE-Indication-r19 ENUMERATED {true} OPTIONAL, -- Cond NR ... }

While this provides a general indicator for AI/ML unavailability, it does not differentiate between UE-internal issues and externally resolvable problems. In AI/ML positioning, a common scenario is when the assistance data from the LMF does not match the input features expected by the UE’s pre-trained model (e.g., TRP configurations, PRS parameters, environmental conditions, inference configuration). Such mismatches are not typical in deterministic positioning methods but can occur frequently in AI/ML approaches due to the decoupled nature of training and inference. Without a dedicated cause value, the LMF cannot distinguish between internal UE limitations (e.g., processing constraints) and issues it can resolve by reconfiguring assistance data.

Proposal 6.	Add a dedicated error cause, e.g., "assistance-data-incompatible-with-model", to indicate that the provided assistance data does not match the UE’s trained AI/ML model requirements. This will enable the LMF to identify externally resolvable issues and take corrective action, such as adjusting assistance data, rather than falling back unnecessarily to legacy positioning methods.

## 2.5	LPP-18 (new): Consistency between training and inference

Issue description: At RAN2#130, contribution R2-2503403 "Discussion on consistency between training and inference for AI POS", vivo et al. was discussed which resulted in the following agreement: To ensure the consistency between training and inference, the UE should be able to request assistance data associated with a specific group of TRPs. FFS the request associated information in on demand prs request.. Proposed resolution: Companies to provide contributions to the following meeting on details for "the request associated information in on demand prs request".

AI/ML-based positioning models are often trained using PRS measurements from a fixed subset of TRPs. If inference is performed with measurements from different TRPs, training–inference consistency is lost, which may degrade accuracy. Allowing the UE to indicate preferred or required TRP group(s) when requesting assistance data would enable the LMF to provide PRS configurations aligned with the model’s training assumptions.

However, this capability depends on the resolution of LPP-21, which addresses the definition and usage of an associated ID for TRP location coordinates.

Proposal 7.	RAN2 to postpone the implementation of LPP-18 until LPP-21 is resolved, to ensure consistent handling of TRP identification and grouping in AI/ML positioning.

## 2.6	LPP-19 (new): Applicability of BM related agreements

Issue description: As mentioned in [POST130][023][AI PHY] 38.305 CR (CATT), since the RAN2 intention is to have the same design for BM case and positioning case as much as possible, whether the agreements made for BM applicability reporting can also be applicable to positioning case 1. Whether the following agreements made for BM applicability reporting are also applicable to positioning case 1 and whether there are any associated LPP impacts: - UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. - Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. Proposed resolution: Companies to provide contributions to the following meeting.

Throughout several previous RAN2 meetings, the following agreements have been reached regarding the functionality for AI/ML positioning Case 1:

At RAN2#127bis The following procedures for LCM for UE sided model for AI positioning case 1 is the baseline: Step 1: LMF may request the UE to report the supported functionalities at the UE side by LPP request capabilities message. Step 2: UE sends LPP provide capabilities message to LMF with the supported functionalities at the UE side. Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition). Step 4: UE reports the applicable functionality to the LMF by the LPP provide capabilities message. Step 5: The LMF requests the inferred location information using the LPP request location information message. Step 6: UE reports the inferred location using LPP provide location information message. At RAN2#128 For POS Case 1, RAN2 confirm that the existing unsolicited UE capability report mechanism in LPP can support UE to report the applicable functionality in both “proactive” and “reactive” as a baseline. - Proactive case: When the applicability change, UE can send an unsolicited LPP ProvideCapabilities message to LMF. - Reactive case: If the applicability changes based on the configuration in LPP ProvideAssistanceData message in step 3, UE can send an unsolicited LPP ProvideCapabilities message to LMF. Configuration details are FFS As a baseline, If the AIML based positioning method becomes non-applicable when LMF requests UE location estimation, UE cannot perform the AIML based positioning, and reply with LPP Providelocationinformation message with error cause. FFS if other fallback options are considered At RAN2#129 UE reports the applicable functionality to the LMF by the LPP provide capabilities message if there is a change of applicable functionality. FFS if any additional LMF control is needed. At RAN2#129bis LMF is responsible for functionality management UE reports the applicable functionality to the LMF by the LPP provide capabilities message without any additional LMF control. An AIML positioning functionality is considered “activated” once UE receives an LPP

Based on agreements from RAN2#127bis to RAN2#129bis, applicability reporting for AI/ML positioning Case 1 follows the same principles as for the AI/ML Beam Management (BM) use case:

- Applicability (= applicable functionality) is determined solely by whether the UE supports AI/ML-based positioning.

- The UE reports applicability via ProvideCapabilities during the LPP Capability Exchange procedure, and may also send unsolicited updates when applicability changes (proactively or reactively).

- If applicability changes from applicable to non-applicable during an ongoing LPP Location Information procedure, the UE responds with a ProvideLocationInformation message including the error cause dl-aiml-positioning-not-available [LPP-14].

Proposal 8.	No LPP changes are needed for applicability reporting in AI/ML positioning Case 1, as the existing capability exchange and unsolicited reporting mechanisms are sufficient.

The current NR-DL-AIML-ProvideCapabilities does not include an explicit parameter to indicate applicability of AI/ML positioning Case 1. Although applicability could be inferred implicitly from the use of NR-DL-AIML-ProvideCapabilities, the absence of an explicit indicator means the LMF cannot reliably distinguish UEs that support only measurement features from those that support full AI/ML-based location estimation.

To address this issue, introduce an explicit field (nr-dl-aiml-positioning-supported-r19) in NR-DL-AIML-ProvideCapabilities to indicate the applicability of AI/ML positioning Case 1.

The following ASN.1 extension (red color) illustrates the proposed addition:

NR-DL-AIML-ProvideCapabilities-r19 ::= SEQUENCE { locationCoordinateTypes-r19 LocationCoordinateTypes OPTIONAL, periodicalReporting-r19 ENUMERATED { supported } OPTIONAL, periodicReportingIntervalMsSupport-r19 PeriodicReportingIntervalMsSupport-r18 OPTIONAL, ten-ms-unit-ResponseTime-r19 ENUMERATED { supported } OPTIONAL, scheduledLocationRequestSupported-r19 ScheduledLocationTimeSupport-r17 OPTIONAL, nr-PosCalcAssistanceSupport-r19 BIT STRING { trpLocSup (0), beamInfoSup (1), rtdInfoSup (2), trpTEG-InfoSup (3), nr-IntegritySup (4), pruInfoSup (5), trpImplicitLoc-ImplicitSup (6) } (SIZE (1..8)) OPTIONAL, nr-los-nlos-AssistanceDataSupport-r19 SEQUENCE { type LOS-NLOS-IndicatorType2-r17, granularity LOS-NLOS-IndicatorGranularity2-r17, ... } OPTIONAL, nr-DL-PRS-ExpectedAoD-or-AoA-Sup-r19 BIT STRING { eAoD (0), eAoA (1) } (SIZE (1..8)) OPTIONAL, nr-DL-AIML-On-Demand-DL-PRS-Support-r19 NR-On-Demand-DL-PRS-Support-r17 OPTIONAL, nr-DL-AIML-On-Demand-DL-PRS-ForBWA-Support-r19 ENUMERATED { supported } OPTIONAL, nr-dl-prs-AssistanceDataValidity-r19 SEQUENCE { area-validity INTEGER (1..maxNrOfAreas-r17) OPTIONAL, ... } OPTIONAL, multiLocationEstimateInSameMeasReport-r19 ENUMERATED { supported } OPTIONAL, nr-IntegrityAssistanceSupport-r19 BIT STRING { serviceParametersSup (0), serviceAlertSup (1), riskParametersSup (2), integrityParaTRP-LocSup (3), integrityParaBeamInfoSup (4), integrityParaRTD-InfoSup (5) } (SIZE (1..8)) OPTIONAL, nr-dl-aiml-positioning-supported-r19 ENUMERATED { supported }, ... }

This will enable the LMF to:

- Distinguish between measurement-only and full estimation capabilities

- Manage applicability reporting consistently with AI/ML BM principles

- Avoid ambiguity in interpreting UE capabilities

Proposal 9.	Add an explicit field, e.g., nr-dl-aiml-positioning-supported-r19, to NR-DL-AIML-ProvideCapabilities to indicate the applicability of AI/ML positioning Case 1.

## 2.7	LPP-20 (new): LPP Impacts related to Case 3a/3b

Issue description: Determine whether there are any LPP impacts related to Case 3a/3b. Proposed resolution: Companies to provide contributions to the following meeting.

Unlike Case 1, Case 3a (NG-RAN node assisted positioning with gNB-side model) and Case 3b (NG-RAN node assisted positioning with LMF-side model) require the UE to provide its location information for Part B (ground truth label) in training data collection and possibly for performance monitoring.

Existing LPP procedures can be reused for obtaining Part B information in Case 3a and Case 3b

- In Case 3a, the gNB obtains Part A (channel measurements) from TRPs and Part B from the LMF, potentially via (maybe new) NRPPa procedures. If the LMF does not have the UE’s Part B, it can initiate positioning using existing LPP procedures.

- In Case 3b, the LMF obtains Part A from gNB/TRP over (maybe existing) NRPPa procedures and Part B from the UE over existing LPP procedures.

Proposal 10.	No LPP impact is foreseen for AI/ML positioning Case 3a/3b.

Both Case 3a and Case 3b are NR UL-based methods in which the gNB/TRP measures UL-SRS transmissions from the UE and reports either AI/ML-assisted or legacy UL-SRS measurements. Synchronization between Part B (location information from UE) and UL-SRS measurements at the gNB can be challenging. We propose that further study on mechanisms to improve synchronization between Part B information from the UE and UL-SRS measurements from the gNB/TRP be carried out in coordination with RAN1/RAN3.

Proposal 11.	RAN2 to consider further study on mechanisms to improve synchronization between Part B information from the UE and UL-SRS measurements from the gNB/TRP in AI/ML positioning Case 3a/3b.

## 2.8	LPP-21 (new): "Associated ID" for TRP Location Coordinates (IE TRP-ImplicitLocationInfo-r19)

Issue description: The following issues have been discovered during LPP running CR review: - Must the "Associated ID" for the TRP coordinates be associated with a "cell" (e.g., NCGI)? If so, how should PRS-only TPs (which are not associated with a cell) being handled? And how to implement RAN1 agreement that UE does not expect to receive different values of associated ID for TRPs belonging to the same cell? - Is the "Associated ID" only for the TRP location coordinates, or also for the associated DL-PRS Resource Set/DL-PRS Resource ARPs? - Is the value range of 0..255 for the "Associated ID" sufficient? In particular if the "Associated ID" should be "configured per cell"? - Are the explicit TRP coordinates (NR-TRP-LocationInfo-r16) and implicit TRP coordinates (NR-TRP-ImplicitLocationInfo-r19) mutually exclusive, or can a deployment provide both? - If a UE supports explicit TRP location info (NR-TRP-LocationInfo-r16), must a UE support also implicit location info (nr-TRP-ImplicitLocationInfo-r19) (and vice versa)? Proposed resolution: Wait for more RAN1 input and/or companies to provide contributions to the following meeting.

The IE NR-TRP-LocationInfo-Implicit has been newly introduced to support consistency between training and inference phases for AI/ML positioning. It enables signaling of TRP-related location metadata in an implicit manner using nr-AIML-AssociatedID. However, the definition, scope, and operational rules for NR-TRP-LocationInfo-Implicit and its associated ID usage have not yet been fully agreed upon, and require clarification from RAN1.

Proposal 12.	RAN2 to wait for further RAN1 input before making any updates to LPP ASN.1 or procedure definitions.

# 3	Conclusion

Regarding LPP-6a,

Proposal 1.	Extend NR-PRU-DL-Info to support multiple PRU measurement–location pairs within a single message, enabling efficient collection of diverse training data for AI/ML-based positioning without impacting existing single-PRU DL-TDoA operations.

Proposal 2.	Enhance NR-DL-AIML-RequestAssistanceData (Optionally) to include a parameter (e.g., requestedPRUcount) allowing the UE to indicate the desired number of PRUs.

Regarding LPP-10a,

Proposal 3.	Maintain the batch reporting structure, including nr-DL-AIML-LocationInformationInstances-r19, in NR-DL-AIML-ProvideLocationInformation-r19 as already defined in the running CR [2].

Regarding LPP-11,

Proposal 4.	Keep NR-DL-AIML-RequestLocationInformation, excluding UE-assisted measurement parameters, and retain only UE-based and common parameters (e.g., nr-AssistanceAvailability).

Proposal 5.	For AI/ML positioning Case 1, the LocationInformationType field in CommonIEsRequestLocationInformation shall be set to locationEstimateRequired. Other values, including locationMeasurementsRequired, locationEstimatePreferred, locationMeasurementsPreferred, and locationEstimateAndMeasurementsRequired, are not applicable and shall not be used.

Regarding LPP-14,

Proposal 6.	Add a dedicated error cause, e.g., "assistance-data-incompatible-with-model", to indicate that the provided assistance data does not match the UE’s trained AI/ML model requirements. This will enable the LMF to identify externally resolvable issues and take corrective action, such as adjusting assistance data, rather than falling back unnecessarily to legacy positioning methods.

Regarding LPP-18,

Proposal 7.	RAN2 to postpone the implementation of LPP-18 until LPP-21 is resolved, to ensure consistent handling of TRP identification and grouping in AI/ML positioning.

Regarding LPP-19,

Proposal 8.	No LPP changes are needed for applicability reporting in AI/ML positioning Case 1, as the existing capability exchange and unsolicited reporting mechanisms are sufficient.

Proposal 9.	Add an explicit field, e.g., nr-dl-aiml-positioning-supported-r19, to NR-DL-AIML-ProvideCapabilities to indicate the applicability of AI/ML positioning Case 1.

Regarding LPP-20,

Proposal 10.	No LPP impact is foreseen for AI/ML positioning Case 3a/3b.

Proposal 11.	RAN2 to consider further study on mechanisms to improve synchronization between Part B information from the UE and UL-SRS measurements from the gNB/TRP in AI/ML positioning Case 3a/3b.

Regarding LPP-21,

Proposal 12.	RAN2 to wait for further RAN1 input before making any updates to LPP ASN.1 or procedure definitions.

# 4	References

- R2-25xxxxx_([POST130][025][AI PHY] LPP Open issues Discussion)_v05_Rap.docx

- R2-25xxxxx_(Running CR 37355-i50)_v02_update1.docx